Response to Reviewers for Manuscript NATCHEMENG-24061044

Code:	Comments in blue | Replies in black | Actions in bold

Note:	Indicated page, line, figure, or reference numbers refer to the revised manuscript and/or supplemental information with changes highlighted.

Reviewer #1

There is not much to say about this manuscript: it is a very well written, comprehensive review about acetylene production with some comments also on downstream processes, including comments on sustainability and cost of the different pathways. As a review, it doesn't provide new discoveries, but it provides an interesting angle on a somewhat neglected topic. It is timely, since the chemical industry is facing a conversion process, in which acetylene could play a more important role again, and there are no recent comparable papers available, as far as I am aware. It has a very different focus compared to the review by Trotus et al., which is already ten years old. Within the scope of the review, all relevant aspects are adequately covered, the references are also appropriate. Thus, I suggest acceptance of the manuscript, I would require no changes. Whether this review format fits the scope of Nature Chemical Engineering as a recently launched journal is a question the editors have to answer - for me it would be an interesting supplement to the other papers that Nature Chemical Engineering carries.

Thank you for your kind and encouraging feedback. We are delighted that you found our review comprehensive, unique, timely, and offering an interesting angle on a neglected topic. We appreciate your recommendation for acceptance.

Reviewer #2

This review aims to provide a holistic overview of acetylene production technologies, proposes sustainable production initiatives and quantifies their economic and environmental performance, and explores potential applications. From my point of view, the starting point is that the authors seek to rekindle interest and foster collaborative efforts in advancing sustainable acetylene production and broadening its use as a feedstock for bulk chemicals. However, as a review paper submitted to Nature Chem Eng, the current form is still far from being considered for publication. Following are my specific comments on the review.

We appreciate the Reviewer’s comments and have made significant changes accordingly. Specifically, we have reformatted the manuscript from a ‘Review’ to a ‘Perspective’ with a new title, ‘Towards Green Acetylene Synthesis and Application’. In Section 2, we have significantly condensed the discussion on conventional acetylene (C2H2) production technologies and added new emerging C2H2 application technologies to reduce redundancy with previous literatures and highlight our forward-looking focus. Furthermore, we have incorporated data on the predicted availability of renewable feedstocks, including biochar and biogas, by 2050, to assess the feasibility of proposed green C2H2 production technologies. The contribution of our work lies not in merely summarizing past technologies, as done in the recent review (ChemPlusChem 2024, e202400247), but in critically analyzing the existing challenges and, more importantly, outlining a pathway for future green acetylene production, as highlighted by Reviewer #1 and #3. We believe this perspective provides valuable insights by envisioning a sustainable trajectory for C2H2 production while encouraging innovation in its production methods and downstream applications. Detailed responses to the reviewer’s comments are provided below.

2.1. As a review article, this manuscript spends the major part to discuss the conventional technologies to produce acetylene, such as CaC2 route, partial oxidation and thermal plasma pyrolysis. To be noted, the acetylene production technologies nowadays all have no connections with catalysis. The reason would be attributed to the thermodynamic features of C2H2. The group of the authors are expertized in catalysis. I can’t clearly see the C2H2-related research background of the authors, as well as the potential solutions to produce C2H2 via thermal catalysis route. Regarding the downstream organic synthesis of C2H2, I also cannot justify the new chemistry or new big market given from the review context. From my personal opinion, the authors for a review paper should have had deep understanding on such a research area with sound publications (i.e., research experience).

We have significantly condensed Section 2 by merging Fig. 2 and 3, removing Fig. 2a, 2b, 3b, and 3c, along with the associated discussions on conventional C2H2 production technologies. Additionally, as suggested by Reviewer #3, we have incorporated recent advancements in C2H2 applications into Section 3 (page xx, line xx). These include: (1) the conversion of C2H2 to butadiene (Nat. Catal. 2024, doi.org/10.1038/s41929-024-01250-0) or ethylene (C2H4) (Nat. Sustain. 2023, 6, 827; Nat. Catal. 2021, 4, 565) via electrocatalysis; (2) C2H2 hydrochlorination to VCM via photocatalysis (J. Am. Chem. Soc. 2024, 146, 29441); and (3) the coupling of C2H2 with CO into ester or acid products via thermocatalysis (Molecules 2024, 29, 1987; Angew. Chem. Int. Ed. 2023, 52, e202307570). These revisions sharpen the focus of this review on the future of sustainable C2H2 production and its potential applications using emerging technologies such as electrocatalysis and photocatalysis, as well as novel C2H2 chemistries.

While it is true that current commercial C2H2 production technologies are not catalysis-driven due to thermodynamic limitations, catalytic C2H2 synthesis efforts are ongoing, particularly through non-thermal plasma and thermal pyrolysis approaches. As stated in the Perspective and Outlook section (page xx, line xx), we propose that the near future of green C2H2 production lies in the integration of green electricity and renewable carbon feedstocks, unless emerging technologies achieve significant breakthroughs. Our Perspective does not assert that catalytic thermal pyrolysis is the definitive path forward; rather, we aim to highlight the insufficient attention given to C2H2 research by the catalysis community and inspire greater interest in this field.

Lastly, we disagree with the Reviewer’s comments on our qualifications to write this perspective. Our group has extensive research experience in the C2H2 field, as demonstrated by several recent publications (ACS Catal. 2024, 14, 13652; Nat. Commun. 2023, 14, 5557; Nat. Nanotechnol. 2022, 17, 606; Nat. Commun. 2021, 12:4016; Angew. Chem. 2019, 131, 12425; Chem. Sci. 2019, 10, 359; ACS Catal. 2018, 8, 1114; Chem. Rev. 2017, 117, 4185; Angew. Chem. 2017, 129, 10895). We are confident that our expertise enables us to provide valuable insights and forward-looking perspectives in this field.

2.2. Some review papers on acetylene are well written in comparison with the current review manuscript, for example, Production of Acetylene and Acetylene-based Chemicals from Coal. Chem. Rev. 2014, 114, 3, 1743–1760; Catalytic Reactions of Acetylene: A Feedstock for the Chemical Industry Revisited. Chem. Rev. 2014, 114, 3, 1761–1782; Production of Acetylene from Viable Feedstock: Promising Recent Approaches. ChemPlusChem 2024, e202400247. https://doi.org/10.1002/cplu.202400247.

We agree with Reviewer #1 that our review significantly differs from the decade-old work by Trotus et al. (Chem. Rev. 2014, 114, 1761) and addresses a gap left by the absence of recent, comparable reviews. Regarding the review (Chem. Rev. 2014, 114, 1743), it is clear that its focus is restricted to coal-based C2H2 production, which is not the primary subject of our work.

As for the recent review (ChemPlusChem 2024, e202400247), we acknowledge some overlap with Section 2 of our work. However, that review lacks the extensive analysis of various conventional C2H2 production technologies and their applications, as stated by Reviewer #3. More importantly, our primary goal is to rekindle interest in future green C2H2 production. To reflect this, we have revised the manuscript from a ‘Review’ to a ‘Perspective’ and updated the title to ‘Towards Green Acetylene Synthesis and Applications.’ Additionally, we have significantly shortened Section 2 to reduce redundancy with the ChemPlusChem review. These revisions sharpen the focus of our work on the future, ensuring that it stands out distinctly from previous literatures.

2.3. The difference of this review from the existing reviews in literature would be the emphasis on the green footprint of acetylene production, e.g., changing coal to biochar. Considering the scale of acetylene production in market, this replacement (coal to biochar) is not convincing, and of course not realistic, especially in China. A possible way to promote the green footprint would be the utilization of renewable energy generated electricity to drive the arc furnace or arc plasma for the high-temperature reaction environment in CaC2 route or plasma pyrolysis of hydrocarbons.

Countries other than China have largely transitioned from C2H2-based downstream production to C2H4-based production, primarily due to the higher environmental impact of conventional calcium carbide (CaC2) route. This perspective aims to promote the production of C2H2 in countries outside China by showcasing how it can be produced in an economically and environmentally sustainable manner. Different markets, depending on the country, may respond differently to the adoption of such technologies.

We acknowledge the Reviewer’s concern regarding the availability of biochar and biomethane. The current availability of biochar (0.4 Mt per year) and biomethane (0.03 Gt per year) is insufficient to meet the global demand for C2H2. However, numerous research studies have highlighted the environmental benefits of biochar and biomethane, leading to optimistic projections for their future availability. Specifically, future projections indicate that by 2050, the annual availability of biochar will range from approximately 0.3 to 0.5 Gt (Environ. Chem. Lett. 2021, 19, 3023). Considering that the CaC2 process requires 2 kg of biochar per kg of C2H2 produced, rough estimates suggest that only 4 to 7% of the available biochar could meet the global C2H2 demand, estimated at about 11 Mt per year. Similarly, for biomethane-based C2H2 production pathways, specifically the partial oxidation route, approximately 2.8 kg of biomethane is required to produce 1 kg of C2H2. It is projected that the biomethane potential within the European Union (EU) will reach around 0.1 Gt by 2050 (Biomethane production potentials in the EU, European Biogas Association 2022). Consequently, only 26% of this biomethane potential would be needed to meet the current C2H2 demand of 11 Mt. We have emphasized these aspects in the main manuscript (page xx, line xx).

As highlighted by the Reviewer, the prospects of using green or decarbonized electricity represents one of the most promising technologies for enhancing the environmental sustainability of C2H2 production. In this context, we highlight the potential future climate change impacts of C2H2 production under a decarbonized electricity grid in Fig. xx of the main manuscript (purple scatter). It is important to note that these results reflect an optimistic scenario, assuming that the 1.5 °C temperature target is achieved by 2050. However, these results are based on a global average to provide a general overview; regional electricity grids will vary significantly depending on the specific scenario and location.

2.4. The authors mentioned the major applications of C2H2 to synthesize VCM or 1, 4-butanediol. First, C2H2-VCM-PVC is indeed an important value chain, for instance, in China. However, this product chain can be totally substituted by C2H4-PVC route. One of the reasons for China to have to produce C2H2 for PVC is the balance of the chlorine from the chlor-alkali industry during the production of a large amount of alkali. Another reason would be the limited petroleum resources, which further limit the application of C2H4. Second, the technique to synthesize 1, 4-butanediol has been mature so that the production of 1, 4-butanediol is already serious overcapacity. Therefore, this part of the reviewed contents is not stimulating in the current and future world.

We agree that the C2H2-VCM-PVC route has largely been replaced by the C2H4-VCM-PVC route outside of China, such as in Europe and the United States. However, it is important to recognize that this shift was primarily driven by the long-term availability of inexpensive petroleum, which facilitates the production of low-cost C2H4. As the Reviewer noted, in regions with limited petroleum resources, such as China, the C2H2 route remains economically more viable, regardless of environmental considerations. We respectfully disagree with the Reviewer’s suggestion that the presence of the C2H2 industry is primarily to balance chlorine from the chlor-alkali industry, as the C2H4-VCM-PVC route can also achieve this balance. Although the C2H2 route has been largely phased out outside of China, it cannot be ruled out as a potential option in the future, particularly with the advent of green C2H2 production technologies.

1,4-Butanediol (BDO) was initially commercialized using C2H2 as the feedstock through classical Reppe chemistry. Over time, several complex, multi-step alternative routes emerged, partially replacing the C2H2-based process in regions with limited C2H2 availability. These alternatives use intermediates such as maleic anhydride or 1,4-diacetoxy-2-butene. While it is true that global BDO production is currently in a state of overcapacity, this situation is primarily driven by the recent development of a more cost-effective C2H2-to-BDO process in China. This process has significantly reduced market prices and now accounts for over 55% of the world’s capacity and 40% of global production (ChemCatChem 2020, 12, 2859; J. Adv. Manuf. Process. 2020, 2, e10054). The success of C2H2-based BDO production in China, though contributing to overcapacity, indeed underscores the unique role of C2H2 and its irreplaceability in certain chemical processes. We have explained the reasons behind BDO overcapacity in Section 2.1 to highlight the significance of C2H2 (page xx, line xx).

2.5. Regarding the environmental and economic assessment of acetylene synthesis routes, fossil based and green, I am not convinced by the replacement of methane by biomethane, coal by biochar as mentioned above. The production of C2H2 must consider where feedstock comes and how much volume of feedstock can be supplied sufficiently. Also, as mentioned above, C2H2 industry is closely connected with chlor-alkali industry. Therefore, the environmental and economic assessment of acetylene synthesis routes must be carried out in a specific chemical industrial park, or in a province/state, or across a country, even beyond. From my point of view, to improve the green footprint with acceptable environmental and economic assessment, the key is the green electricity, instead of the “green” feedstock, because of the process features.

We acknowledge the concern regarding the availability of biochar and biomethane. As noted in comment 2.3, by 2050, their projected availability–4% and 26% of their potential, respectively–will be sufficient to meet current global C2H2 demands, despite present limitations. The discussion in the main manuscript has been expanded to highlight this point (page xx, line xx).

We would like to emphasize that the system boundary for this analysis has been defined exclusively as the C2H2 production process. It is assumed that the CaC2 production facilities are integrated with the C2H2 plant; therefore, CaC2 is treated as an input to the economic and environmental assessment, with its associated climate change impacts and costs accounted for accordingly. The downstream utilization of acetylene is not considered in this study. However, a representative case for VCM production is presented, where C2H2 and HCl are assumed to be inputs to a VCM production plant integrated with the C2H2 and HCl production facilities. Section Sxx of the Supplementary Information has been updated to reflect these details.

In the context of using renewable electricity for C2H2 production, as highlighted in comment 2.3, our analysis includes a climate change impact assessment based on the assumption of a decarbonized (green) electricity mix in 2050, as shown in Fig. xx. However, recognizing that no single solution fits all scenarios, we broaden our focus beyond green electricity for sustainable C2H2 production to consider renewable feedstocks.

2.6. In the context, the major technical terms, such as partial combustion, hot plasma, thermal plasma, non-thermal plasma, cold plasma, warm plasma, thermal pyrolysis and so on, should not be confused.

We have standardized the key technical terms into four categories: partial combustion, thermal plasma, non-thermal plasma (including cold and warm plasma), and thermal pyrolysis.

2.7. Figure 1 gives some historical information. However, why does not anticipate the near future, e.g., extend the timeline to 2050?

Predicting C2H2 production in 2050 is challenging due to numerous variables, including advancements in C2H2 production technologies, the development of competing renewable C2H4 production pathways, the availability of renewable carbon feedstocks, and differing environmental policies across countries. Therefore, it would not be appropriate to include a prediction for C2H2 production in 2050 in Fig. 1. Additionally, the Introduction and Fig. 1 primarily focus on historical data and outline the potential direction for green C2H2 production. Including predicted data in this section would therefore be inappropriate. Instead, we have incorporated a discussion on the anticipated availability of renewable carbon feedstocks for green C2H2 production by 2050, such as biochar and biomethane, in Section 4 (page xx, line xx).

2.8. Figure 2 provides the C2H2 production routes, involving past and current applications. However, trial routes and abandoned routes are not that justified. On feedstock, why not add the biomethane/biochar in? On products, why not predict possible market from the C2H2 chemistry for near future?

The abandoned C2H2 production and application routes provide valuable context for understanding the history of C2H2 development. Retaining these routes in Fig. 2 while emphasizing current technologies is therefore beneficial. In response to the Reviewer’s concern, we have shortened the discussion of these historical and discontinued routes in Section 2.1. Fig. 2 now serves as a comprehensive analysis of both past and present C2H2 production and applications, with the associated challenges for each technology discussed in Section 2. These challenges highlight the need for exploring green C2H2 production in the future, such as using biomethane or biochar as feedstocks, which we address thoroughly in Fig. 5 and the corresponding discussions.

As mentioned in our response to comment 2.7, predicting the future scale of C2H2 production and its market potential in C2H2 chemistry remains highly uncertain due to various influencing factors. As an alternative, we have included discussions on recent studies in Section 3, focusing on emerging C2H2 applications (page xx, line xx). Further details can be found in the response to comment 2.1.

2.9. Figure 3: in 3(a), some technical words are not appropriate, such as dryer, carbon, C2H2 furnace(?), CO→CO2(?), etc., 3(b) is not informative at all; in 3(c) the numbers are not representative, which should be specified with the corresponding C2H2 plant.

2.10. Figure 4: 4(a) has been in many existing references; 4(b) is a bit redundant with 4(a); 4(c) is not very informative at all.

Regarding comment 2.9 and 2.10, we have significantly shortened Sections 2.1–2.3 and consolidated Fig. 3c, 4a, and 4d into a new figure, removing all other components from the original Fig. 3 and 4. This adjustment reduces redundancy with existing literature and highlights the forward-looking vision for renewable C2H2 production. The data in Fig. 3c are directly associated with an C2H2 plant in China (J. Clean. Prod. 2021, 322, 129055). We chose to retain Fig. 4a, despite its presence in other references, because the thermodynamic curve it presents is essential for helping readers outside the field quickly grasp why C2H2 production from hydrocarbons at low temperatures is challenging.

2.11. Figure 5: 5(a) is somewhat like a textbook or a slide of PPTs; 5(b) has little valued information; 5(c), such a plot (conversion vs. selectivity) can be often seen for comparing catalyst performance, however, it does not fit for C2H2 production via partial oxidation or thermal pyrolysis; 5(d) is not an appropriate evaluation for C2H2 production

We respectfully disagree with the Reviewer’s assessment of Fig. 5. Each panel addresses critical aspects of the technologies discussed:

Fig. 5a showcases the reactor configurations of various non-thermal plasma technologies, emphasizing the complexity of reactor design and its pivotal role in advancing these systems.

Fig. 5b illustrates a representative membrane reactor used in thermal pyrolysis to shift the methane (CH4) conversion equilibrium towards the product side, demonstrating the importance of reactor design not only for non-thermal plasma but also for thermal pyrolysis.

Fig. 5c is intended to compare thermal plasma, non-thermal plasma, and thermal cracking processes, rather than partial combustion technologies, which are addressed in the previous section.

Fig. 5d uniquely presents CH4 conversion, C2H2 selectivity, and energy consumption in a single plot, underscoring that energy requirements—often overlooked in non-thermal plasma studies—are as critical as conversion and selectivity for industrial viability.

We believe these figures collectively provide essential insights into the challenges and opportunities of emerging C2H2 production technologies and are crucial to the overall narrative.

2.12. Figure 6: most chemical engineers would not have interest in those “Potential green C2H2 synthesis routes.”

In the context of the ambitious targets set by global frameworks such as the Paris Climate Agreement and the United Nations Sustainable Development Goals (SDGs), sustainability is a critical issue for all industries, and particularly for the chemical industry due to its heavy reliance on fossil fuels for feedstock and as an energy vector. This reliance makes the chemical industry one of the most difficult sectors to decarbonize, accounting for approximately 5.6 Gt CO2e, or 10% of global emissions (Curr. Opin. Chem. Eng. 2023, 39, 100881). As a result, the chemical industry is undergoing a major transition to achieve sustainable production by utilizing defossilized feedstocks and energy. Consequently, these potential green C2H2 synthesis routes align with the broader global imperatives mentioned above.

Other comments:

2.13.1 Sections 2.2 and 2.3 discuss indirect methods of acetylene production (the calcium carbide method) and direct methods of acetylene production, are there any new developments of the methods to the research considered in the relevant reviews Schobert H. Production of acetylene and acetylene-based chemicals from coal[J]. Chemical Reviews, 2013, 114(3):1743-1760., Trotuş I T, Zimmermann T, Schüth F. Catalytic reactions of acetylene: a feedstock for the chemical industry revisited[J]. Chemical Reviews, 2014, 114: 1761-1782. a decade ago?

The conventional commercial C2H2 production technologies discussed in Sections 2.2 and 2.3 have seen little to no development over the past decade. We have consolidated these into a new Section 2.2 in a concise manner, providing a brief introduction to these technologies. Our focus is on the future rather than the past, and this concise introduction serves as an important primer for readers, helping them familiarize themselves with the field and understand our motivation for pursuing green C2H2 in the future. Other details can be found in the response to comment 2.1.

2.13.2. The title of section 2.3 (Direct commercial acetylene production routes) does not seem to be very appropriate, since the title of 2.2 (Indirect acetylene production route via calcium carbide) does not emphasize the word commercial. Therefore, this title may mislead readers unfamiliar with the relevant field into thinking that the calcium carbide method is not a commercial acetylene preparation method.

We have significantly shortened Sections 2.1, 2.2, and 2.3, and combined Sections 2.2 and 2.3 into a new section titled ‘Commercial C2H2 production routes’.

2.13.3. Emerging routes for acetylene synthesis are discussed in Section 2.4, where the authors describe a route for the production of acetylene from CH4 using non-thermal plasma technology. Should this technique be viewed as a different way of generating plasma or as a revolutionary new technology for acetylene production? In the paper, the authors point out that the use of non-thermal plasma actually requires a greater energy input than the use of thermal plasma (Line 237), and that the CH4 conversion and acetylene selectivity are not as good as in the thermal plasma approach. Thus, despite the savings in the quenching, non-thermal plasma technology should be far less economical than thermal plasma technology at this time. So, if this is viewed as a promising technology, what are the drivers for its development? What are the current key technical barriers? What technological breakthroughs are needed to industrialize it? At present the authors seem to be merely talk about the results reported in the literature, but not point out and analyze these issues clearly.

Non-thermal plasma is fundamentally different from thermal plasma in its generation process. Unlike thermal plasma, where all species (electrons, ions, and neutrals) are in thermal equilibrium, non-thermal plasma generates high-energy electrons while maintaining a relatively low bulk gas temperature. This characteristic potentially opens up new reaction pathways that are not accessible in thermal equilibrium systems. As shown in Fig. 5d, non-thermal plasma requires a similar or even higher energy input compared to thermal plasma. However, CH4 conversion and C2H2 selectivity are comparable to those of the thermal plasma process. As an emerging technology capable of enabling C2H2 production at low temperatures, we believe its promise should not be dismissed solely due to its slightly higher energy input compared to the commercial thermal plasma process.

We have elaborated on our perspectives on non-thermal plasma in the ‘Perspective and Outlook’ section: Emerging technologies, such as non-thermal plasma, face commercialization challenges that require advancements in reactor design, catalyst development, and a deeper understanding of reaction mechanisms. Non-thermal plasma technologies show performance close to that of commercial thermal plasma but typically require higher energy input and more complex equipment. While catalysts can favor C2H2 production, the catalytic and plasma-assisted mechanisms are still not well understood. Future research should aim to improve C2H2 yield, energy efficiency, and equipment lifespan, with the goal of reducing energy requirements to the theoretical limit of 4.0 kWh kg–1 C2H2. Cold plasma offers advantages by preventing C2H2 decomposition and eliminating costly cooling processes, while warm plasma usually achieves higher CH4 conversion. The future dominance of either plasma type remains uncertain.

2.13.4. As far as I know, the production of acetylene from coal as a feedstock using plasma technology is also a promising technology and large-scale experimental installations have been established in China. However, only the acetylene production technologies based on natural gas as feedstock using plasma (thermal and non-thermal) are discussed in this paper. It is proposed to add a discussion in section 2.4 on the new developments and prospects of plasma technology for acetylene production from coal as a feedstock, in particular the new trends in recent years compared to what has been discussed in the previous reviews.

In Section 2.1, we briefly mentioned that plasma technologies for coal-to-C2H2 production are still being explored at the pilot scale in China. To better highlight the emerging coal-to-C2H2 processes utilizing plasma technology, we have removed the relevant discussions from Section 2.1 and included the following description in the new Section 2.3 (page xx, line xx): Coal-to-C2H2 production via plasma technologies has seen significant development in China during the 21st century. Between 2002 and 2016, Xinjiang Tianye Group Co. Ltd. developed 2 MW and 5 MW hydrogen plasma coal-to-C2H2 pilot plants.[xx] While partially successful, scaling issues led to the discontinuation of the 5 MW system. Since 2014, Zhejiang University and Xinjiang Yuehetai Chemical Technology Co. Ltd. have collaboratively advanced 2 MW and 10 MW hydrogen plasma systems.[xx] These efforts have achieved an annual production capacity of 5 kt, with reported costs over 20% lower than conventional CaC2 routes and a significantly reduced carbon footprint.

[xx] Xia, W., Shi, K., Wang, C., Wang, H., Chen, X. The plasma energy route to industrial carbon neutrality in China. Chin. J. Theor. Appl. Mech. 55, 2779–2795 (2023). [xx] Guo, X. et al. Analysis and prospect on plasma pyrolysis technology for coal-to-C2H2 production. Environ. Prot. Chem. Ind. 41, 229–234 (2021).

2.13.5. In section 3, the authors argue that the adoption of bio-based feedstocks for the production of acetylene, and hence PVC, could be an important driver for the revival of acetylene. However, the fact that more than 45% of the market for PVC is held by China, and that this PVC is mainly produced from calcium carbide acetylene, implies that the ATV pathway would have had an importance comparable to that of EBP. Therefore, in terms of VCM or PVC production, it needs further consideration to whether the topic about the revival of acetylene exist. In addition, it is suggested that the production and environmental costs of the four methods in Figure 6 (i.e., fossil and green ATV or EBP) be put together in order to visualize the advantages of green ATV when combining production and environmental costs.

As the Reviewer noted, China holds 45% of the global PVC market, where the ATV pathway dominates. The primary challenge associated with the ATV route is its high carbon footprint. Currently, nearly 20% of PVC in China is produced via the EBP route. If C2H2 production technologies shift from fossil-based to green alternatives, the EBP route may be partially replaced by the more cost-effective ATV route. More importantly, the ATV route could see a resurgence outside of China.

Additionally, we have incorporated discussion of recent studies into Section 3, highlighting emerging C2H2 applications (see details in the response to the comment 2.1). These advancements further reinforce the potential for C2H2 to return as a key chemical building block.

Fig. 6a and 6b present the production costs and climate change impacts of the four methods, respectively. By separating the production cost data and the climate change impact data into two distinct figures, we ensure clarity and make the information easier to follow. Therefore, we believe combining them into a single figure is unnecessary.

2.14.1. The second half of section 2.1 (Summary of acetylene production routes and its applications) puts too much emphasis on the substitution of acetylene applications, contrary to the main theme which calls attention to the reuse of acetylene. Suggests that the fact that acetylene applications have been replaced is coupled with the main advantages of using acetylene for synthesis and the problems that need to be overcome.

We have shortened Sections 2.1–2.3 and reduced the discussion on the substitution of C2H2 applications, as suggested. In Section 2.1, our primary goal is to provide an extensive analysis of C2H2 production and applications, both past and present. By comparing C2H2’s historical and current uses, we identify which applications have been replaced over time. This analysis also highlights why certain applications, such as the production of VCM, butanediol, acetylene black, and vinyl acetylene, remain irreplaceable, emphasizing C2H2’s uniqueness. Following the discussion of substitution in Section 2.1, we have included the main advantages of using C2H2 as a building block (page xx, line xx). These advantages stem from its unique C≡C triple bond, which typically enables more streamlined downstream synthesis. The primary challenge associated with C2H2-based routes is their high carbon footprint, a concern mentioned throughout the perspective.

2.14.2. Section 2.4 (Emerging direct acetylene synthesis routes and catalytic applications) is rather informative and the refined title is not accurate enough. Acetylene from thermally cracked CH4 is not part of the emerging direct acetylene synthesis route and the main focus of the presentation is on the design of new catalysts for the TP process. It is suggested that plasma acetylene production and the development of new catalyst design for the TP process be presented in two separate sections.

We have divided Section 2.4 into two subsections: (1) Emerging C2H2 synthesis routes via the plasma process, and (2) Advancements in catalyst design for the thermal pyrolysis process.

2.14.3. Section 5 (Environmental and economic assessment of acetylene synthesis routes) provides a more in-depth analysis of the environmental and economic evaluation of acetylene synthesis. However, processes related to acetylene inevitably need to consider safety as an equally important or even higher priority than economic and environmental aspects. It is proposed to supplement the evaluation of the safety performance related to the several synthesis methods mentioned with a summary of the risks and challenges faced by each process in terms of safety.

We agree that safety is a critical consideration in C2H2 synthesis due to its high flammability, explosive nature, and susceptibility to decomposition under specific conditions. Each C2H2 production process has unique safety challenges. For instance, in the CaC2 process, CaC2 must be stored in a dry, airtight container to prevent unintended contact with moisture. Similarly, in the partial combustion process, thermal runaway reactions can occur if temperature is not carefully monitored and controlled. Finally, plasma-based processes for producing C2H2 require high-voltage electrical systems, which pose risks such as shock and electrical arcing. Accordingly, a summary of the risks and challenges associated with each process is provided in Section Sxx of the Supplementary Information.

It is worth noting that the global annual acetylene production, which reaches nearly 11 Mt, underscores the feasibility of large-scale industrial operations when proper safety measures are in place. Furthermore, integrating acetylene production and application within a single facility effectively minimizes risks associated with its storage and transportation.

Reviewer #3

This review comprehensively introduced acetylene production technologies and provided extensive analyses regarding the possible ways to sustainably produce acetylene. Thus, it undoubtfully demonstrate the underlying and unexcavated value of acetylene. This review is expected to contribute to the clarification of the developing trend of acetylene in industry and advance relevant research with the basic guidelines and updated data. I recommend its publication in Nature Chemical Engineering after minor revisions.

Thank you for your positive and thoughtful feedback. We appreciate your recognition of our extensive analysis of acetylene (C2H2) production technologies, which effectively highlights the untapped potential of C2H2. We are also pleased that the review is considered valuable in clarifying C2H2’s evolving trends and advancing related research.

3.1. The authors planned a footprint of the revival of acetylene by only proposing the possibility of developing green acetylene, which seems not convincing enough. The authors may add extra part of discussion about the existing technologies or emerging technologies that may enlarge the application field of acetylene to make the plan more acceptable. For instance, recent work about the conversion of acetylene to ethylene using electrocatalysis way and to vinyl chloride using photocatalysis way provides the newest application field for acetylene.

We thank the Reviewer for the thoughtful suggestion. In Section 3, our environmental and economic analysis demonstrates that production of vinyl chloride monomer (VCM) could shift from ethylene (C2H4) route to the C2H2 route if the biochar-based C2H2 process is industrialized. This shift would increase C2H2 utilization in VCM production, underscoring its expanding role in the chemical industry. As suggested, emphasizing emerging technologies for C2H2 applications through electrocatalysis and photocatalysis further highlights C2H2’s potential as a key building block for the future chemical industry.

To address this, we have incorporated discussions in Section 3 (page xx, line xx) on recent studies that highlight emerging C2H2 applications: (1) the conversion of C2H2 to butadiene (Nat. Catal. 2024, doi.org/10.1038/s41929-024-01250-0) or C2H4 (Nat. Sustain. 2023, 6, 827; Nat. Catal. 2021, 4, 565) via electrocatalysis; (2) C2H2 hydrochlorination to VCM via photocatalysis (J. Am. Chem. Soc. 2024, 146, 29441); and (3) the coupling of C2H2 with CO into ester or acid products via thermocatalysis (Molecules 2024, 29, 1987; Angew. Chem. Int. Ed. 2023, 52, e202307570).

3.2. The authors briefly introduced several kinds of catalysts in part 2.4. What are the key issues for these catalysts during acetylene production, which hinders efficient acetylene production. It would be better to improve several perspectives about the current challenging problems in different catalysts and thus give some guidance for future research.

Although catalysts have been widely reported to enhance C2H2 production in both non-thermal plasma and thermal pyrolysis processes, the reasons these methods have never been industrialized remain underexplored. To guide future research on catalytic C2H2 production, we have summarized the key challenges associated with catalysis in these two technologies in Section 2.4 (page xx, line xx).

Specifically, in the catalytic non-thermal plasma process, while C2H2 selectivity has already reached a high level comparable to that of commercial thermal plasma processes, the addition of catalysts does not appear to reduce energy requirements. In fact, it often leads to energy consumption that is similar to or even higher than that of thermal plasma processes. Moreover, the design of catalytic plasma reactors capable of durable C2H2 production remains complex and insufficiently explored.

On the other hand, in catalytic thermal pyrolysis, C2H2 selectivity is limited to approximately 35% due to the competitive formation of other hydrocarbons and the thermodynamically favorable formation of coke. Consequently, catalytic thermal pyrolysis remains far from industrial viability, with overcoming coke formation being a critical challenge for future research.