Supplementary Information

Towards Green Acetylene Synthesis and Application

Zihao Zhang, Abhinandan Nabera, Gonzalo Guillén-Gosálbez, Javier Pérez-Ramírez*

Institute for Chemical and Bioengineering, Department of Chemistry and Applied Biosciences, ETH Zurich, Vladimir-Prelog-Weg 1, 8093 Zurich, Switzerland.

* Corresponding author. E-mail: jpr@chem.ethz.ch

Table of Contents

Supplementary Notes	2

Supplementary Tables	4

Supplementary References	15

# Supplementary Notes

Supplementary Note 1 | Life cycle assessment methodology

The global average climate change impacts are quantified using an attributional life cycle assessment (LCA) in accordance with ISO 14044/14040 standards.1,2 We consider 1 kg of vinyl chloride monomer (VCM) or acetylene (C2H2) produced as the functional unit, following a cradle-to-gate approach that encompasses all activities from raw material acquisition to production. This approach assumes that the end-use phase is identical regardless of the synthesis route. The mass and energy flows for the VCM and C2H2 production routes were obtained from literature, as shown in Supplementary Tables 3 and 6, respectively. This data allows us to model the foreground system (VCM or C2H2 production plant), while data for the background system (activities supplying inputs to the production facility) were primarily sourced from the Ecoinvent v3.10 database.3 We consider the 100-year average global warming potentials (GWPs) as implemented in the IPCC 2021 methodology.4 The inventories used to calculate the climate change impact for all VCM and C2H2 production technologies are presented in Supplementary Tables 4 and 7, respectively. Notably, we assume that the biochar and calcium carbide production facilities are in close proximity to the acetylene production plant; therefore, we do not consider the negligible impact from transportation.

Furthermore, to calculate the climate change impacts for the year 2050, we conducted a prospective LCA. In this context, we considered the most optimistic future scenario aligned with the 1.5 °C climate target. To update the background system, we utilized future projections derived from Integrated Assessment Models (IAMs). IAMs, such as IMAGE (Integrated Model to Assess the Global Environment), provide a comprehensive understanding of the interactions among the biosphere, society, and the climate.5 We used the premise v2.1.1.6 framework to construct future background data consistent with the results from the IMAGE model. Additionally, the IMAGE model assumes specific Shared Socioeconomic Pathways (SSPs) that describe the relationship between challenges for adaptation and mitigation. In our study, we assumed the SSP2 scenario (“middle-of-the-road”). For the mitigation scenario, we considered the Representative Concentration Pathway (RCP) aimed at limiting the global mean surface temperature to 1.5 °C (RCP19 from premise). All calculations for the environmental assessment were conducted using the Brightway2 framework.7

We also perform an uncertainty assessment for the current scenarios using Monte Carlo sampling with 500 iterations, generated from the ecoinvent pedigree matrix that provides uncertainty levels for the life cycle inventory parameters. However, we do not perform an uncertainty assessment for the prospective 2050 scenarios due to the lack of available uncertainty data (i.e., the corresponding pedigree matrix).

Supplementary Note 2 | Techno-economic assessment methodology

The mass and energy flows for the VCM and C2H2 production routes were obtained from literature, as shown in Supplementary Tables 3 and 6, respectively. The economic assessment considers only operational expenditures for the reference year 2022. Generally, capital costs are much lower than raw material and utilities consumption expenses;8 therefore, we neglect capital and transportation expenditures, providing a lower bound for the production costs in our analysis. Supplementary Tables 5 and 8 present the prices of feedstock and utilities used in the production cost calculations for all the VCM and C2H2 production technologies, respectively. We also conduct an uncertainty assessment by considering both high and low costs for the feedstock and utilities needed in our process (as detailed in Supplementary Tables 5 and 8). Unlike the prospective LCA performed in this work, we do not conduct an economic assessment for 2050 due to the significant challenges posed by the high uncertainty and volatility of future energy prices, which are influenced by geopolitical scenarios and technological advancements.

Supplementary Note 3 | Safety risks and challenges of acetylene production technologies

Acetylene is a highly unstable compound under normal conditions. It reacts with oxygen at room temperature, producing water and carbon dioxide while releasing significant heat (1255.6 kJ mol–1). Its tendency to detonate makes it unsuitable for storage as a gas. Consequently, various safety risks and challenges in acetylene production must be addressed, depending on the synthesis technology. In the calcium carbide-based acetylene production route, calcium carbide is a highly flammable solid. Moreover, contact with moisture can form flammable acetylene gas and lime where the reaction heat can ignite acetylene. Similarly, the partial combustion route involves significant safety risks and challenges, including high temperatures during preheating and preventing premature ignition due to the reactive oxygen-hydrocarbon mixture. Soot formation, which can clog the reactor and destabilize the system, is also a major concern. Thermal and non-thermal plasma processes for acetylene production pose distinct safety risks. Thermal plasma processes involve high temperatures, thereby increasing risks of explosions from flammable gases. On the other hand, non-thermal plasma operates at lower temperatures but require high voltages, leading to electricity hazards and the risk of uncontrolled reactions.

# Supplementary Tables

Supplementary Table 1 | Comparison of representative results of thermal and non-thermal (cold and warm) plasma-driven methane coupling processes. Power supply, applied voltage, catalyst type, feed type, CH4 conversion, selectivity to C2 products, energy requirement (ER), specific energy input (SEI), and plasma temperature (thermal, cold, warm) are provided for comparison.

| Plasma type | Power supply / W | Applied voltage / kV | Catalyst | Feed | Conv. / % | Sel. / % |  |  | ER / kWh kg–1 C2H2 (SEI / kJ mol–1 CH4) | Plasma temp. | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | C2H2 | C2H4 | C2H6 |  |  |  |
| Hüls arc | 8000k | 8 | / | CH4a | 71 | 73 | 5 | ~ | 12.1 (275) | thermal | 9,10 |
| DuPont arc | 9000k | ~ | / | CH4/H2 | 90 | 80 | ~ | ~ | 8.8 | thermal | 11 |
| LC arcb | 850 | <12 | / | CH4/H2 | 70 | 90 | 4 | <1 | 10.2 | thermal | 12 |
| corona | 8.2 | 10.9 | / | CH4 | 28 | 76 | ~ | ~ | 26.2 (3403) | cold | 13 |
| corona | 8.4 | 7 | NaY | CH4/H2/O2 | 38 | 32 | 2 | 7 | ~ | cold | 14 |
| corona | 8.4 | 7 | / | CH4/H2/O2 | 12 | 0 | 0 | 26 | ~ | cold | 14 |
| pulse corona | 30 | 20-32 | Al2O3 | CH4/CO2 | 43 | 23 | 5 | 3 | ~ | cold | 15 |
| pulse corona | 30 | 20-32 | La2O3/Al2O3 | CH4/CO2 | 25 | 55 | 9 | 9 | ~ | cold | 15 |
| pulse corona | ~ | 18-36 | / | CH4 | 45 | 68 | ~ | ~ | ~ (1788) | cold | 16 |
| DBDc | 2 | 20 | Fe-MFM-300 | CH4 | 3 | 38 | 58 | 0 | ~ (44.8) | cold | 17 |
| DBD | 2 | 20 | / | CH4 | 1 | 17 | 47 | 22 | ~ (44.8) | cold | 17 |
| DBD | 3.3 | 6 | MgO/Al2O3 | CH4 | 23 | 28 | 30 | 27 | ~ | cold | 18 |
| DBD | 3.4 | 6 | MgO/Al2O3 | CH4 | 16 | 22 | 26 | 36 | ~ | cold | 18 |
| DBD | 3.5 | 6 | MgO/Al2O3 | CH4 | 10 | 11 | 19 | 48 | ~ | cold | 18 |
| DBD | 7-8 | 7.6 | / | CH4 | 32 | 7 | 6 | 20 | ~ | cold | 19 |
| DBD | 7-8 | 10 | BaTiO3 | CH4 | 6 | 18 | 32 | 20 | ~ | cold | 19 |
| DBD | 7-8 | 9 | γ-Al2O3 | CH4 | 30 | 11 | 12 | 13 | ~ | cold | 19 |
| DBD | 7-8 | 8.9 | α-Al2O3 | CH4 | 33 | 12 | 10 | 12 | ~ | cold | 19 |
| DBD | 7-8 | 9.8 | SBA15 | CH4 | 20 | 10 | 18 | 15 | ~ | cold | 19 |
| DBD | ~ | 17 | / | CH4 | 12 | 3 | 3 | 31 | ~ | cold | 20 |
| DBD | ~ | 17 | 1g Pt/Al2O3 | CH4 | 12 | 2 | 2 | 49 | ~ | cold | 20 |
| DBD | ~ | 17 | 3g Pt/Al2O3 | CH4 | 19 | ~ | <1 | 31 | ~ | cold | 20 |
| DBD | 36 | <30 | Pt/CeO2 | CH4 | 36 | 6 | 7 | 47 | ~ | cold | 21 |
| DBD | 72 | <30 | Pt/CeO2 | CH4 | 43 | 2 | 3 | 45 | ~ | cold | 21 |
| DBD | >150 | ~ | TiO2/ZSM-5 | CH4/O2 | 18 | 7 | 16 | 9 | ~ | cold | 22 |
| DBD | >150 | ~ | TiO2/silicalite | CH4/O2 | 8 | 10 | 16 | 11 | ~ | cold | 22 |
| pulsed | 25 | ~ | / | CH4 | 44 | 95 | 3 | <1 | ~ | cold | 23 |
| pulsed | 25 | ~ | / | CH4/O2 | 59 | 59 | ~ | ~ | ~ | cold | 23 |
| pulsed | 25 | ~ | / | 1CH4/1H2 | 46 | 94 | 4 | 1 | ~ | cold | 23 |
| pulsed | 25 | ~ | / | 1CH4/9H2 | 12 | 64 | 15 | 18 | ~ | cold | 23 |
| pulsed | ~ | 21 | / | CH4 | 51 | 72 | 9f |  | ~ (250) | cold | 24 |
| pulsed spark | ~ | ~ | / | CH4 | 39 | 83 | 4 | ~ | 12.7 | warm | 25 |
| pulsed spark | ~ | 12.3 | / | CH4 | 24 | 85 | 5 | 2 | 9.2 (86) | warm | 26 |
| microwave | 120 | ~ | / | CH4/H2 | 51 | 61 | 12 | ~ | 73.8 (1076) | warm | 27 |
| microwave | 130 | ~ | Cu | CH4 | 38 | 86 | <1 | 2 | ~ | warm | 28 |
| microwave | 130 | ~ | Cu/W | CH4 | 22 | 82 | 1 | 3 | ~ | warm | 28 |
| microwave | 130 | ~ | Cu/Ni | CH4 | 17 | 76 | 2 | 4 | ~ | warm | 28 |
| microwave | 600 | ~d | / | CH4 | 34 | 59 | 2 | ~ | 24.0 (224) | warm | 29 |
| microwave | 600 | ~d | / | 3CH4/1H2 | 43 | 62 | 2 | ~ | 25.0 (224) | warm | 29 |
| microwave | 600 | ~d | / | 1CH4/1H2 | 49 | 70 | 2 | ~ | 28.3 (224) | warm | 29 |
| microwave | 600 | ~e | / | CH4 | 31 | 46 | 8 | ~ | 33.2 (224) | warm | 29 |
| microwave | 600 | ~e | / | 3CH4/1H2 | 36 | 47 | 10 | ~ | 38.5 (224) | warm | 29 |
| microwave | 600 | ~e | / | 1CH4/1H2 | 42 | 60 | 10 | ~ | 38.3 (224) | warm | 29 |
| microwave | off | ~ | Mo/ZSM-5 | CH4 | 10 | 0 | 15 | 3 | ~ | warm | 30 |
| microwave | 110 | ~ | Mo/ZSM-5 | CH4 | 11 | 35 | 10 | 2 | ~ | warm | 30 |
| microwave | 16 | ~ | / | CH4 | 24 | 41 | ~ | ~ | 143 (652) | warm | 31 |
| microwave | 34 | ~ | / | CH4 | 57 | 57 | ~ | ~ | 44.7 (674) | warm | 31 |
| microwave | 48 | ~ | / | CH4 | 80 | 61 | ~ | ~ | 28.1 (659) | warm | 31 |
| microwave | 70 | ~ | / | CH4 | 94 | 71 | ~ | ~ | 20.0 (627) | warm | 31 |
| microwave | 81 | ~ | / | CH4 | 93 | 71 | ~ | ~ | 20.8 (644) | warm | 31 |

aNatural gas is used as the feed;

bLow-current arc;

cDBD: dielectric barrier discharge;

d110 mbar;

e55 mbar;

fTotal selectivity to C2H4 and C2H6;

~ : not mentioned;

/ : not used.

Supplementary Table 2 | Comparison of representative results of thermal pyrolysis (TP) processes under various reaction conditions.

| T / °C | GHSV / L gcat−1 h−1 (h−1) | Catalyst | Feed | Conv. / % | Sel. / % |  |  |  | Total sel. to C2 / % | Ref |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | C2H2 |  | C2H4 | C2H6 |  |  |
| 1100 | 24 | Mo2C | CH4 | 23 | 5 |  | 13 | 3 | 21 | 32 |
| 1100 | 24 | WC | CH4 | 10 | 6 |  | 18 | 8 | 32 | 32 |
| 1000 | 12 | Li/MgO | CH4 | 5 | 1 |  | 3 | 6 | 10 | 33 |
| 865 | ~ | FeS2 | CH4/S2 | 7-10 | 1-2 |  | ~ | ~ | 20-37 | 34 |
| 700 | 302 | Mo/ZSM-5 | CH4 | 0.05 | 20 |  | 65 | 5 | 90 | 35 |
| 950 | 6 | CaO-Na-Cl | CH4/CO2 | 19 | 9 |  | 63 | 22 | 94 | 36 |
| 950 | 6 | CaO | CH4/CO2 | 9 | 5 |  | 64 | 31 | 100 | 36 |
| 200-1500 | (105) | Pt/Al2O3 | CH4/O2 | 63 | 7-15 |  | 4-5 | <1 | ~ | 37 |
| 925 | 12 | Ni–P/SiO2 | CH4 | 1 | 6 |  | 64 | 18 | 88 | 38 |
| 900 | ~ | Ni–P/SiO2 | C2H6 | 100 | 20 | 22 |  | ~ | ~ | 38 |
| 1020 | (9400) | / | CH4 | 5 | 18 | 28 |  | 6 | 52 | 39 |
| 1020 | (9400) | CRSa | CH4 | 8 | 19 | 26 |  | 4 | 49 | 39 |
| 1020 | (9400) | Fe©CRS | CH4 | 7 | 19 | 28 |  | 5 | 52 | 39 |
| 1020 | (9400) | Fe/CRS | CH4 | 8 | 19 | 25 |  | 3 | 47 | 39 |
| 1020 | (9400) | Fe-CRS | CH4 | 6 | 18 | 27 |  | 5 | 50 | 39 |
| 1020 | (9400) | Fe/SiO2 | CH4 | 7 | 18 | 27 |  | 5 | 50 | 39 |
| 1020 | (9400) | Fe©CRS | CH4/H2 | 6 | 29 | 53 |  | 4 | 86 | 39 |
| 1020 | 1 | / | CH4 | 4 | 11 | 28 |  | 5 | 44 | 40 |
| 1020 | 1 | SiO2 | CH4 | 11 | 13 | 25 |  | 2 | 40 | 40 |
| 1020 | 1 | Fe-SiO2 | CH4 | 9 | 15 | 29 |  | 3 | 47 | 40 |
| 1020 | 1 | CoFe-SiO2 | CH4 | 10 | 13 | 26 |  | 2 | 41 | 40 |
| 1020 | 1 | NiFe-SiO2 | CH4 | 10 | 14 | 27 |  | 2 | 43 | 40 |
| 1020 | 1 | PdFe-SiO2 | CH4 | 8 | 14 | 29 |  | 3 | 46 | 40 |
| 1020 | 1 | PtFe-SiO2 | CH4 | 10 | 14 | 25 |  | 2 | 41 | 40 |
| 950 | ~ | Fe reactor | CH4/THNb | 9 | 7 | 28 |  | 2 | 37 | 41 |
| 970 | ~ | Fe reactor | CH4/THN | 11 | 8 | 27 |  | 2 | 37 | 41 |
| 990 | ~ | Fe reactor | CH4/THN | 14 | 9 | 24 |  | 2 | 35 | 41 |
| 1010 | ~ | Fe reactor | CH4/THN | 17 | 10 | 22 |  | 1 | 33 | 41 |
| 1030 | ~ | Fe reactor | CH4/THN | 19 | 10 | 20 |  | 1 | 31 | 41 |
| 1050 | ~ | Fe reactor | CH4/THN | 26 | 10 | 16 |  | 1 | 27 | 41 |
| 975 | 6 | / | CH4 | 1 | 0 | 0 |  | 4 | 4 | 42 |
| 975 | 6 | CeO2 | CH4 | 7 | 2 | 7 |  | 2 | 11 | 42 |
| 975 | 6 | Ptn/CeO2 | CH4 | 10 | 6 | 8 |  | 2 | 16 | 42 |
| 975 | 6 | Pt1/CeO2 | CH4 | 14 | 35 | 33 |  | 7 | 75 | 42 |
| 950 | 3.2 | Fe/SiO2c | CH4 | 5 | 14 | 59 |  | 12 | 85 | 43 |
| 1000 | 3.2 | Fe/SiO2c | CH4 | 13 | 16 | 44 |  | 4 | 64 | 43 |
| 1050 | 3.2 | Fe/SiO2c | CH4 | 21 | 14 | 40 |  | 4 | 58 | 43 |
| 1070 | 3.2 | Fe/SiO2c | CH4 | 26 | 14 | 37 |  | 3 | 54 | 43 |
| 1050 | 3.2 | Fe/SiO2 | CH4 | 14 | 18 | 49 |  | 5 | 72 | 43 |
| 1050 | 3.2 | Fe/SiO2 | CH4/H2 | 4 | 25 | 62 |  | 6 | 93 | 43 |
| 800 | 6 | NiP/SiO2 | CH4 | <1 | 0 | 19 |  | 38 | 57 | 44 |
| 900 | 6 | NiP/SiO2 | CH4 | 3 | 4 | 35 |  | 5 | 44 | 44 |
| 1000 | ~ | Fe/SiO2 | CH4 | 11 | 7 | 22 |  | 2 | 31 | 45 |

aCRS: cristobalite;

bTHN: 1,2,3,4-tetrahydronaphthalen;

cHydrogen-permeable tubular membrane reactor;

~ : not mentioned;

/ : not used.

Supplementary Table 3 | Inputs and outputs of the acetylene to VCM (ATV) and ethylene-based balanced process (EBP) routes per kg of VCM produced.

| Stream | ATV46 | EBP47 |  |
| --- | --- | --- | --- |
| Feedstock |  |  |
| Acetylene / kg | 0.42 | - |  |
| Ethylene / kg | - | 0.45 |  |
| HCl / kg | 0.58 | - |  |
| Chlorine / kg | - | 0.57 |  |
| Utilities |  |  |
| Electricity / kWh | - | 0.02 |  |
| Cooling water / kg | 75.99 | 196.85 |  |
| Refrigeration to –25 °C / MJ | 0.39 | - |  |
| Refrigeration to –50 °C / MJ | - | 2.15 |  |
| Heating – natural gas / kg | 82⋅10–5 | 0.05 |  |
| Emissions to water |  |  |
| Water / kg | - | 44⋅10–4 |  |
| Emissions to air |  |  |  |
| Carbon dioxide / kg | - | 0.02 |  |

Supplementary Table 4 | Inventories used in the life cycle assessment (LCA) to calculate the climate change impact for all VCM production technologies.

| Flow | Activity | Source |
| --- | --- | --- |
| Ethylene | market for ethylene | Ecoinvent v3.10 |
| Bio ethylene | market for bio ethylene | Yang et al.48 |
| Acetylene | acetylene production, coal, calcium carbide (Supplementary Table 6) | This work |
| Bio acetylene | acetylene production, biochar, calcium carbide (Supplementary Table 6) | This work |
| Chlorine | market for chlorine, gaseous | Ecoinvent v3.10 |
| HCl | market for hydrochloric acid, without water, in 30% solution state | Ecoinvent v3.10 |
| Heating – natural gas | heat production, natural gas, at boiler condensing modulating >100kW | Ecoinvent v3.10 |
| Electricity | market group for electricity, high voltage | Ecoinvent v3.10 |
| Cooling water | market for water, deionized | Ecoinvent v3.10 |
| Refrigeration –25 °C | refrigeration –25 °C | Luyben et al.49 |
| Refrigeration –50 °C | refrigeration –50 °C | Luyben et al.49 |

Supplementary Table 5 | Cost parameters used in calculating the unitary production cost for VCM.

| Flow | Average | Low | High | Ref |
| --- | --- | --- | --- | --- |
| Ethylene / USD kg–1 | 1.770 | 1.293 | 1.895 | 47 |
| Bio ethylene / USD kg–1 | 4.386 | - | - | 48 |
| Acetylene / USD kg–1 | 1.179 | 0.862 | 1.865 | This work |
| Bio acetylene / USD kg–1 | 1.553 | 0.963 | 2.481 | This work |
| Chlorine / USD kg–1 | 0.389 | - | - | 47 |
| HCl / USD kg–1 | 0.153 | - | - | 50 |
| Heating – natural gas / USD kg–1 | 0.285 | 0.193 | 0.393 | 51 |
| Cooling water / USD kg–1 | 48⋅10–6 | - | - | 52 |
| Grid electricity / USD kWh–1 | 0.093 | 0.067 | 0.151 | 52 |
| Refrigeration to –25 °C / USD MJ–1 | 0.019 | - | - | 49 |
| Refrigeration to –50 °C / USD MJ–1 | 0.043 | - | - | 49 |

All costs are reported for the year 2022 as the reference year. The production costs for acetylene (only operational expenditures) are calculated in this work, referring to the material and energy flows and cost parameters provided in Supplementary Table 6 and Supplementary Table 7, respectively.

Supplementary Table 6 | Inputs and outputs of the calcium carbide (CaC2), partial combustion (PC), thermal plasma, and non-thermal plasma routes per kg of acetylene (C2H2) produced.

| Stream | CaC253 | PC53 | Thermal plasma53,54 | Non-thermal plasma25 |
| --- | --- | --- | --- | --- |
| Feedstock |  |  |  |  |
| Limestone / kg | 5.00 | - | - | - |
| Coke or biochar / kg | 2.00 | - | - | - |
| Natural gas or biomethane / kg | - | 4.46 | 1.55 | 1.48 |
| N-methyl-2-pyrrolidone / kg | - | 0.01 | 0.01 | - |
| Sodium hypochlorite / kg | 0.05 | - | - | - |
| Sodium hydroxide / kg | 16⋅10–4 | - | - | - |
| Utilities |  |  |  |  |
| Electricity / kWh | 10.81 | 3.30 | 12.10a | 12.65 |
| Cooling water / kg | 12.50 | 3.10 | 6.00 | - |
| Heating – steam / kg | - | 4.50 | 4.00 | - |
| Emissions to air |  |  |  |  |
| Particulate matter / kg | 51⋅10–5 | 25⋅10–5 | 23⋅10–5 | - |
| Nitrogen oxides / kg | 78⋅10–5 | 48⋅10–5 | 45⋅10–5 | - |
| Carbon dioxide / kg | 3.60 | 0.80 | - | - |
| Volatile organic compounds (VOC) / kg | 0.32 | - | - | - |
| Non-methane volatile organic compounds (NMVOC) / kg | - | 30⋅10–6 | 36⋅10–6 | - |

aThe electricity consumption was updated to 12.10 kWh kg–1 C2H2, as the original reference used for the thermal plasma inventory (from Zhang et al.53) reported a lower amount than the average range in the literature.10,25,54–56

Supplementary Table 7 | Inventories used in the life cycle assessment (LCA) to calculate the climate change impact for all acetylene production technologies. The life cycle inventories employed for biomethane were based on premise v2.1.16 sourced from Ecoinvent v3.10, which provides prospective inventories for a range of technologies based on the results derived from the IMAGE Integrated Assessment Model5 under the SSP2 pathway.

| Flow | Activity | Source |
| --- | --- | --- |
| Limestone | market for limestone, unprocessed | Ecoinvent v3.10 |
| Coke | market for coke | Ecoinvent v3.10 |
| Biochar | market for biochar (using poplar as the biomass source) | Peters et al.57 |
| Natural gas | market group for natural gas, high pressure | Ecoinvent v3.10 |
| Biomethane | biomethane production, from biogas upgrading, using amine scrubbing | IMAGE SSP2-RCP6-2020 Ecoinvent v3.10 |
| N-methyl-2-pyrrolidone | market for N-methyl-2-pyrrolidone | Ecoinvent v3.10 |
| Sodium hypochlorite | market for sodium hypochlorite, without water, in 15% solution state | Ecoinvent v3.10 |
| Sodium hydroxide | market for sodium hydroxide, without water, in 50% solution state | Ecoinvent v3.10 |
| Electricity | market group for electricity, high voltage | Ecoinvent v3.10 |
| Cooling water | market for water, deionised | Ecoinvent v3.10 |
| Heating – steam | market for steam, in chemical industry | Ecoinvent v3.10 |

Supplementary Table 8 | Cost parameters used in calculating the unitary production cost for acetylene.

| Flow | Average | Low | High | Ref |
| --- | --- | --- | --- | --- |
| Coke / USD kg–1 | 0.084 | 0.067 | 0.117 | 58 |
| Biochar / USD kg–1 | 0.271 | 0.117 | 0.425 | 59 |
| Methane / USD kg–1 | 0.285 | 0.193 | 0.393 | 51 |
| Biomethane / USD kg–1 | 1.488 | 1.058 | 1.901 | 60 |
| Heating – steam / USD kg–1 | 0.021 | - | - | 52 |
| Cooling water / USD kg–1 | 48⋅10–6 | - | - | 52 |
| Grid electricity / USD kWh–1 | 0.093 | 0.067 | 0.151 | 52 |

All costs are reported for the year 2022 as the reference year.

# Supplementary References

- International Standards Organization. ISO 14040:2006 Environmental management—Life cycle assessment—Principles and framework (2006).

- International Standards Organization. ISO 14044:2006 Environmental management—Life cycle assessment—Requirements and guidelines (2006).

- Wernet, G. et al. The ecoinvent database version 3 (part I): overview and methodology. Int. J. Life Cycle Assess. 21, 1218–1230 (2016).

- IPCC. Climate Change 2021: The Physical Science Basis. Contribution of working group I to the sixth assessment report of the intergovernmental panel on climate change (Cambridge University Press, 2021).

- Stehfest, E., van Vuuren, D., Bouwman, L. & Kram, T. Integrated assessment of global environmental change with IMAGE 3.0: Model description and policy applications (Netherlands Environmental Assessment Agency (PBL), 2014).

- Sacchi, R. et al. PRospective EnvironMental Impact asSEment (premise): A streamlined approach to producing databases for prospective life cycle assessment using integrated assessment models. Renew. Sustain. Energy Rev. 160, 112311 (2022).

- Mutel, C. Brightway: An open source framework for life cycle assessment. J. Open Source Softw. 2, 236 (2017).

- Galán-Martín, Á. et al. Sustainability footprints of a renewable carbon transition for the petrochemical sector within planetary boundaries. One Earth 4, 565–583 (2021).

- Boulos, M. I., Fauchais, P. & Pfender, E. High-power plasma torches and transferred arcs. in Handbook of Thermal Plasmas 905–960 (Springer International Publishing, 2023).

- Scapinello, M., Delikonstantis, E. & Stefanidis, G. D. The panorama of plasma-assisted non-oxidative methane reforming. Chem. Eng. Process. Process Intensif. 117, 120–140 (2017).

- Bilera, I. V. & Lebedev, Yu. A. Plasma-chemical production of acetylene from hydrocarbons: history and current status (a review). Pet. Chem. 62, 329–351 (2022).

- Khoe Dinh, D. et al. Efficient methane-to-acetylene conversion using low-current arcs. RSC Adv. 9, 32403–32413 (2019).

- Beloqui Redondo, A., Troussard, E. & van Bokhoven, J. A. Non-oxidative methane conversion assisted by corona discharge. Fuel Process. Technol. 104, 265–270 (2012).

- Liu, C., Mallinson, R. & Lobban, L. Nonoxidative methane conversion to acetylene over zeolite in a low temperature plasma. J. Catal. 179, 326–334 (1998).

- Zhang, X., Dai, B., Zhu, A., Gong, W. & Liu, C. The simultaneous activation of methane and carbon dioxide to C2 hydrocarbons under pulse corona plasma over La2O3/γ-Al2O3 catalyst. Catal. Today 72, 223–227 (2002).

- Zhu, A., Gong, W., Zhang, X. & Zhang, B. Coupling of methane under pulse corona plasma (I) in the absence of oxygen. Sci. China Ser. B-Chem. 43, 208–214 (2000).

- Ma, Y. et al. Direct conversion of methane to ethylene and acetylene over an iron-based metal–organic framework. J. Am. Chem. Soc. 145, 20792–20800 (2023).

- Kasinathan, P. et al. Plasma-enhanced methane direct conversion over particle-size adjusted MOx/Al2O3 (M = Ti and Mg) catalysts. Plasma Chem. Plasma Process. 34, 1317–1330 (2014).

- Taheraslani, M. & Gardeniers, H. Coupling of CH4 to C2 hydrocarbons in a packed bed DBD plasma reactor: the effect of dielectric constant and porosity of the packing. Energies 13, 468 (2020).

- Kim, S. S., Lee, H., Choi, J. W., Na, B. K. & Song, H. K. Methane conversion to higher hydrocarbons in a dielectric-barrier discharge reactor with Pt/γ-Al2O3 catalyst. Catal. Commun. 8, 1438–1442 (2007).

- Liu, L., Das, S., Zhang, Z. & Kawi, S. Nonoxidative coupling of methane over ceria-supported single-atom Pt catalysts in DBD plasma. ACS Appl. Mater. Interfaces 14, 5363–5375 (2022).

- Han, Q. et al. Conversion of methane to C2 and C3 hydrocarbons over TiO2/ZSM-5 core–shell particles in an electric field. RSC Adv. 9, 34793–34803 (2019).

- Kado, S., Sekine, Y. & Fujimoto, K. Direct synthesis of acetylene from methane by direct current pulse discharge. Chem. Commun. 24, 2485–2486 (1999).

- Lotfalipour, R., Ghorbanzadeh, A. M. & Mahdian, A. Methane conversion by repetitive nanosecond pulsed plasma. J. Phys. D: Appl. Phys. 47, 365201 (2014).

- Yao, S., Nakayama, A. & Suzuki, E. Acetylene and hydrogen from pulsed plasma conversion of methane. Catal. Today 71, 219–223 (2001).

- Yao, S. L., Suzuki, E., Meng, N. & Nakayama, A. A high-efficiency reactor for the pulsed plasma conversion of methane. Plasma Chem. Plasma Process. 22, 225–237 (2002).

- Zhang, J., Yang, Y., Zhang, J., Liu, Q. & Tan, K. Non-oxidative coupling of methane to C2 hydrocarbons under above-atmospheric pressure using pulsed microwave plasma. Energy Fuels 16, 687–693 (2002).

- Radoiu, M. T., Chen, Y. & Depew, M. C. Catalytic conversion of methane to acetylene induced by microwave irradiation. Appl. Catal. B: Environ. 43, 187–193 (2003).

- Wnukowski, M. et al. Influence of hydrogen addition on methane coupling in a moderate pressure microwave plasma. Fuel 288, 119674 (2021).

- Julian, I., Ramirez, H., Hueso, J. L., Mallada, R. & Santamaria, J. Non-oxidative methane conversion in microwave-assisted structured reactors. Chem. Eng. J. 377, 119764 (2019).

- Heintze, M. & Magureanu, M. Methane conversion into acetylene in a microwave plasma: optimization of the operating parameters. J. Appl. Phys. 92, 2276–2283 (2002).

- Zhang, S. B. X. Y. et al. Role and dynamics of transition metal carbides in methane coupling. Chem. Sci. 14, 5899–5905 (2023).

- Zhang, S. B. X. Y. et al. Lithium promotes acetylide formation on MgO during methane coupling under non-oxidative conditions. Angew. Chem. Int. Ed. 62, e202307814 (2023).

- Liu, S. et al. “Soft” oxidative coupling of methane to ethylene: Mechanistic insights from combined experiment and theory. Proc. Natl Acad. Sci. USA 118, e2012666118 (2021).

- Razdan, N. K., Kumar, A., Foley, B. L. & Bhan, A. Influence of ethylene and acetylene on the rate and reversibility of methane dehydroaromatization on Mo/H-ZSM-5 catalysts. J. Catal. 381, 261–270 (2020).

- Zhang, Y. et al. CO2 oxidative coupling of methane using an earth-abundant CaO-based catalyst. Sci. Rep. 9, 15454 (2019).

- Hohn, K. L., Witt, P. M., Davis, M. B. & Schmidt, L. D. Methane coupling to acetylene over Pt-coated monoliths at millisecond contact times. Catal. Lett. 54, 113–118 (1998).

- Dipu, A. L. et al. Direct nonoxidative conversion of methane to higher hydrocarbons over silica-supported nickel phosphide catalyst. ACS Catal. 10, 375–379 (2020).

- Han, S. J., Lee, S. W., Kim, H. W., Kim, S. K. & Kim, Y. T. Nonoxidative direct conversion of methane on silica-based iron catalysts: effect of catalytic surface. ACS Catal. 9, 7984–7997 (2019).

- Han, S. J. et al. Methane direct conversion to olefins, aromatics, and hydrogen over silica entrapped bimetallic MeFe-SiO2 (Me = Co, Ni, Pd, Pt) catalysts. Mol. Catal. 535, 112864 (2023).

- Hao, J. et al. Enhanced methane conversion to olefins and aromatics by H-donor molecules under nonoxidative condition. ACS Catal. 9, 9045–9050 (2019).

- Xie, P. et al. Nanoceria-supported single-atom platinum catalysts for direct methane conversion. ACS Catal. 8, 4044–4048 (2018).

- Sakbodin, M., Wu, Y., Oh, S. C., Wachsman, E. D. & Liu, D. Hydrogen-permeable tubular membrane reactor: promoting conversion and product selectivity for non-oxidative activation of methane over an Fe©SiO2 catalyst. Angew. Chem. Int. Ed. 55, 16149–16152 (2016).

- Dipu, A. L., Nishikawa, Y., Inami, Y., Iguchi, S. & Yamanaka, I. Development of highly active silica-supported nickel phosphide catalysts for direct dehydrogenative conversion of methane to higher hydrocarbons. Catal. Lett. 152, 199–212 (2022).

- Oh, S. C. et al. Direct non-oxidative methane conversion in a millisecond catalytic wall reactor. Angew. Chem. Int. Ed. 58, 7083–7086 (2019).

- Dattani, J., Devani, D. & Sahu, O. Cleaner production of vinyl chloride monomer (VCM). Int. J. Sci. Eng. Res. 4, 701–705 (2013).

- Medrano-García, J. D. et al. Economic and environmental competitiveness of ethane-based technologies for vinyl chloride synthesis. ACS Sustain. Chem. Eng. 11, 13062–13069 (2023).

- Yang, M., Tian, X. & You, F. Manufacturing ethylene from wet shale gas and biomass: comparative technoeconomic analysis and environmental life cycle assessment. Ind. Eng. Chem. Res. 57, 5980–5998 (2018).

- Luyben, W. L. Estimating refrigeration costs at cryogenic temperatures. Comput. Chem. Eng. 103, 144–150 (2017).

- Wang, Y. et al. A decarbonized vinyl chloride synthesis via selective oxidative coupling of methyl chloride. Preprint at https://doi.org/10.26434/chemrxiv-2024-vlq7m-v2 (2024).

- Commodity markets. World Bank. Available at: https://www.worldbank.org/en/research/commodity-markets (2024).

- Ioannou, I., D’Angelo, S. C., Martín, A. J., Pérez-Ramírez, J. & Guillén-Gosálbez, G. Hybridization of fossil- and CO2-based routes for ethylene production using renewable energy. ChemSusChem 13, 6370–6380 (2020).

- Zhang, S. et al. Life cycle assessment of acetylene production from calcium carbide and methane in China. J. Clean. Prod. 322, 129055 (2021).

- Jiang, C. & Hu, J. Conversion of Methane to Acetylene. 93–116 (Royal Society of Chemistry, 2023).

- Anderson, R. P., Fincke, J. R. & Taylor, C. E. Conversion of natural gas to liquids via acetylene as an intermediate. Fuel 81, 909–925 (2002).

- Shlyapin, D. A., Afonasenko, T. N., Glyzdova, D. V., Leont’eva, N. N. & Lavrenov, A. V. Acetylene production technologies in the 21st century: main trends of their development in the paradigm of low-carbon economy of the future. Catal. Ind. 14, 251–267 (2022).

- Peters, J. F., Iribarren, D. & Dufour, J. Biomass pyrolysis for biochar or energy applications? A life cycle assessment. Environ. Sci. Technol. 49, 5195–5202 (2015).

- Coal PRICE Today | Coal Spot Price Chart | Live Price of Coal per Ounce. Markets Insider. Available at: https://markets.businessinsider.com/commodities/coal-price (2024).

- Peters, J. Pyrolysis for biofuels or biochar? A thermodynamic, environmental and economic assessment. PhD thesis (2015).

- Market state and trends in renewable and low-carbon gases in Europe. European Biogas Association. Available at: https://www.europeanbiogas.eu/market-state-and-trends-in-renewable-and-low-carbon-gases-in-europe/.