Landscape of Acetylene Production

Zihao Zhang, Abhinandan Nabera, Gonzalo Guillén-Gosálbez, Javier Pérez-Ramírez*

Institute for Chemical and Bioengineering, Department of Chemistry and Applied Biosciences, ETH Zurich, Vladimir-Prelog-Weg 1, 8093 Zurich, Switzerland.

*Corresponding author. E-mail: jpr@chem.ethz.ch

Abstract (ca. 214 words)

Acetylene (C2H2), produced from coal or natural gas, was a cornerstone of the organic chemical industry until the 1960s due to its rich chemistry afforded by the triple bond. However, the global availability of inexpensive petroleum in the 1950s accelerated the production of olefins (e.g., C2H4), diminishing C2H2’s market prominence due to their similar range of downstream chemical production. Both fossil-based C2H4 and C2H2 production processes are highly energy-intensive and pose significant environmental concerns. While the focus has been on alternative C2H4 production due to its larger market share (225 vs. 11 million tons), the potential of new C2H2 production methods warrants equal attention for producing C2H4-derived chemicals and its unique applications, such as oxyacetylene welding, butanediol, and acetylenic chemical synthesis. This review offers a holistic overview of C2H2 production technologies, including the indirect calcium carbide route and direct routes via partial combustion, thermal plasma, non-thermal plasma, and thermal pyrolysis. In addition to traditional fossil-based routes, we explore sustainable resources like biomass-derived biochar and biomethane as carbon feedstocks. We aim to elucidate the status and challenges of each production technology and encourage the development of sustainable C2H2 production by evaluating their potential through life cycle assessments. By highlighting novel synthesis approaches, we seek to rekindle interest in C2H2 chemistry and foster collaborative efforts for its technological advancement.

Keywords: Acetylene production; Calcium carbide; Plasma; Methane coupling; Sustainable carbon resources; Life cycle assessment.

- Introduction (ca. 753 words)

Acetylene (C2H2) was accidentally discovered in 1836 by Edmund Davy in Ireland through the reaction of water with potassium carbide (K2C2), obtained by heating a mixture of tartar (potassium hydrogen tartrate, KC4H5O6) and charcoal powder (Fig. 1).1 Significant study only began after French chemist Marcellin Berthelot coined the name “acetylene” in 1860 and reported its synthesis via electric arc in 1862.2 The first commercial calcium carbide (CaC2) plant for C2H2 production was established in 1894 due to the production of CaC2 by the electrothermal reduction of lime (CaO) with carbon sources and its simple reaction with water to form C2H2, discovered by Thomas Willson.3 Early applications of C2H2 were mainly in lighting and oxyacetylene welding and cutting due to the brightness and high temperature of its flame.2,4 The importance of C2H2 was reflected by its triple bond, which makes the molecule reactive and offers rich chemical possibilities. Industrial acetaldehyde production via C2H2 hydration in 1916, along with studies on C2H2 chemistry by Walter Reppe to produce various commodity chemicals, positioned C2H2 as a pillar of the organic chemical industry.5,6 Coal-based CaC2 was the sole source for C2H2 production until the development of commercial light hydrocarbon cracking methods, including the electric arc (Hüls arc) process in 1940, and the partial combustion (PC) and Wulff thermal pyrolysis (TP) processes in 1950.7,8

The petrochemical industry’s growth in the USA during the 1940s and Europe during the 1950s, coupled with the safer handling of olefins compared to C2H2, led to the development of more cost-effective steam cracking technologies for the production of ethylene (C2H4) and propylene (C3H6), gradually displacing the C2H2 market primarily with C2H4.9,10 C2H2 was the sole pillar in the chemical industry before 1940, but by 1960, global production of C2H2 and C2H4 reached 3 and 3.5 million metric tons per year (Mt y–1), respectively,11–13 and these figures shifted to 11 Mt y–1 for C2H2 and 225 Mt y–1 for C2H4 by 2021.14–16 Between the 1970s and 1980s, PC became the dominant technology for C2H2 production, as other methods were mostly abandoned. In recent years, China saw a resurgence in the CaC2 route, with CaC2 production rising from over 2 Mt y–1 in 1990 to 25 Mt y–1 in 2014,17 where nearly 3 tons of CaC2 are needed to produce 1 ton of C2H2 depending on the CaC2 purity. China is currently the sole leader in C2H2 production, with an output exceeding 8 Mt y–1, while other countries produce less than 3 Mt y−1.15,16,18 The historical transition from C2H2 to C2H4 resulted from cheaper C2H4 production technologies due to the long-term availability of cheap oil before 1973.19,20 Afterwards, C2H2 production became more economical in some regions when oil prices were high and electricity/natural gas costs were low.21 Both C2H4 and C2H2 production are energy-intensive and have significant environmental impacts, producing 1 to -2 tons of CO2 per ton of C2H4 depending on the feedstocks and 0.8 and 3.6 tons of CO2 per ton of C2H2 via PC and CaC2 routes, respectively.15,22–24 This highlights the need for alternative production routes for both C2H2 and C2H4. While alternative C2H4 production has been extensively studied,25,26 research on C2H2 alternatives has lagged over the last several decades, possibly due to its smaller market size.

Although C2H2 chemistry has largely been replaced since the 1960s, it is important to remember that C2H2 is more reactive, and its derived processes often show streamlined synthesis pathways compared to C2H4-derived processes.5 Additionally, some applications of C2H2, such as oxyacetylene welding and the production of acetylene black, polyacetylene (2000 Nobel Prize in Chemistry), and Reppe chemicals like 1,4-butanediol, are difficult to replace.27–30 As we strive for a more sustainable chemical industry based on non-fossil carbon and renewable energy, research on C2H2 and C2H4 should be revisited, with C2H2 potentially playing a crucial role in this grand transition.31 For example, methane (CH4), possibly sourced from biogas, has been widely explored for C2H4 production but remains unindustrialized.26 In contrast, CH4 to C2H2 via PC or electric arc was commercialized nearly 70 years ago, despite the high natural gas input or high energy consumption.32 Therefore, the shift towards sustainability presents an opportunity for revitalizing C2H2 production, warranting increased attention. In this review, we provide a holistic summary of C2H2 production methods and sustainable initiatives, in addition to the challenges and opportunities presented with this shift. Our aim is to guide sustainable technology development and advocate for greater focus on alternative C2H2 routes.

Fig. 1. History of C2H2 manufacture. The timeline illustrates the discovery of C2H2, the evolution of various industrial C2H2 production technologies, the advancements in C2H2 chemistry, global production changes at key historical points, and the gradual market transition from C2H2 to C2H4.

- Production of acetylene (ca. 2770 words)

2.1.	Summary of acetylene production routes and its applications (ca. 802 words)

Acetylene (C2H2) can be produced from fossil-derived feedstocks such as coal, natural gas, and oil using four primary technologies: CaC2, PC, plasma, and TP routes (Fig. 2). Among these, oil-based routes have been explored but have never reached large commercial viability. Plasma technology includes thermal plasma, where all generated species reach thermal equilibrium, and non-thermal plasma, where energy is preferentially channeled to electrons, leading to a significantly higher electron temperature.33 Thermal plasma, particularly the electric arc method, was first commercialized for C2H2 production from natural gas by Hüls in Germany in 1940.7,34 However, the electric arc process was eventually replaced by PC due to its extremely high energy consumption (12.1 kWh kg–1 C2H2).7 Plasma technology for converting coal and natural gas to C2H2 continues at the pilot scale in China. The natural gas plasma method, which is expected to be commercialized, uses one-third of the methane required for PC, though specific details of the plasma technology and energy consumption have not yet been reported.15,35 TP of hydrocarbons was commercialized by the Wulff process in 1950, which involved preheating bricks to 1300 °C followed by feeding hydrocarbons for cracking to produce C2H2.8 However, due to low heating rate and significant soot formation compared to PC, Wulff plants have been closed.11 Currently, coal-derived CaC2 and natural gas-derived PC technologies dominate industrial C2H2 production. Notably, C2H2 production is region-dependent, with China producing over 8 Mt y–1 primarily from the CaC2 route (~90%) due to its large coal reserves, while other countries produce less than 3 Mt y–1, mainly via PC.11,15

The first commercial application of C2H2 after 1894 was for lighting, followed by its use in oxyacetylene welding and cutting developed in 1903.2 In the late 19th and early 20th centuries, acetylene lighting was widely used for illumination in mining, vehicles, street lighting, lighthouses, and aviation beacons, but it was gradually replaced by electric lighting after 1920.11 Oxyacetylene welding and cutting remain widely used, offering a flame temperature of approximately 3150 °C, and it is one of the cheapest and most accessible methods for metalworking.27 The most significant application of C2H2 is as a feedstock in the chemical industry,31 with key downstream products summarized in Fig. 2. The industrial production of acetaldehyde, acetic acid, vinyl acetate, acrylic acid, and acrylonitrile from C2H2 has been largely replaced by routes using C2H4, C3H6, and methanol. Notably, vinyl acetate production in China primarily relies on the C2H2 route to date. Acetaldehyde and vinyl acetate are now primarily produced from C2H4 globally, with annual productions of 1.5 and 6.2 Mt, respectively.36,37 Acrylic acid and acrylonitrile are produced from C3H6, with annual productions of 8.8 and 7.0 Mt, respectively.38–40 Acrylic acid has the highest annual global market size at approximately USD 22.6 billion,38 while acetaldehyde has the lowest at around USD 1.9 billion. Acetic acid, previously produced via acetaldehyde oxidation, is now mainly produced by methanol carbonylation, with a high production of 18 Mt y–1 and a global annual market size of about USD 15 billion.41,42

Currently, major applications of C2H2 in chemical synthesis include the production of vinyl chloride, 1,4-butanediol, vinyl acetate, and acetylene black. Vinyl chloride has the highest production volume, reaching approximately 50 Mt y–1,43 predominantly used to produce polyvinyl chloride (PVC). While the C2H4-based PVC route dominates globally, accounting for about 65% of PVC output,44 approximately 78% of PVC production in China uses the C2H2 route, with the remainder using the C2H4 route.21 This significant production from C2H2 and C2H4 contributes to a global PVC market size of USD 68 billion. In contrast, acetylene black has the smallest market size of approximately USD 0.3 billion.45 It is a high-purity, highly crystalline form of carbon black produced from the thermal decomposition of C2H2, with a unique structure that makes it an excellent conductive additive for electrodes.11,45 Therefore, the market for acetylene black is expected to grow with the global shift towards electric-vehicles.45

Fig. 2. C2H2 production routes, past and current applications. The figure introduces various C2H2 production routes using fossil-derived feedstocks, including coal, natural gas, and oil, and several technologies, such as CaC2, PC, plasma, and TP routes. The figure highlights major industrial applications of C2H2 and provides data on approximate global annual production (Mt y–1) and market size (USD billion y–1). It distinguishes between commercial routes (currently in use), abandoned routes (past commercial processes replaced by other methods), and trial routes (processes with research interest in the past but never commercialized). aSince almost all VCM demand is for PVC production, this number reflects the global market size of PVC. a,bData from Statista.

Another critical application of C2H2 is the production of 1,4-butanediol via the classical Reppe process, which involves the reaction of C2H2 with formaldehyde. This process still dominates, accounting for 40% of total production (2–2.5 Mt y–1).46,47 Despite its advantages, several complex and multi-step alternative routes have been developed to address the scarcity of C2H2 in some regions. These include: i) the production of maleic anhydride from benzene or n-butane oxidation, followed by esterification and hydrogenolysis, ii) the reaction of butadiene with acetic acid to form 1,4-diacetoxy-2-butene, followed by hydrogenation and hydrolysis, and iii) the chlorination of butadiene and subsequent reaction with sodium acetate to give 1,4-diacetoxy-2-butene, followed by hydrogenation and hydrolysis.46 The production of 1,4-butanediol exemplifies the advantage of C2H2 as the feedstock in the chemical synthesis. However, research on the production of C2H2-based chemicals has not attracted sufficient attention since being largely replaced by olefin routes.

2.2.	Indirect acetylene production route via calcium carbide (ca. 514 words)

CaC2, also known as calcium acetylide, is a salt of the C22− anion. Unique among all C2H2 synthesis routes, the CaC2 method allows an easy reaction with water at room temperature to release the C22− anion as C2H2. The production process of C2H2 via the CaC2 route involves two reaction steps:

3C + CaO → CaC2 + CO                            ΔH = 465 kJ mol–1

CaC2 + 2H2O → C2H2 + Ca(OH)2           ΔH = –127 kJ mol−1

The formation of CaC2 from coke and lime (CaO) is highly endothermic, while the conversion of CaC2 to C2H2 is highly exothermic.48 Due to its simplicity and high selectivity for C2H2, the CaC2 route was first commercialized in 1894.3 This C2H2 manufacturing process (Fig. 3a) has low equipment investment but several disadvantages:49 (i) The CaC2 formation reaction requires temperatures of 2000 to 2300 °C in an arc furnace to overcome the high endothermicity; (ii) large amounts of coal are consumed, with one-third of carbon resources converted to CO; (iii) significant CO2 emissions result from the decomposition of limestone (CaCO3) and the combustion of byproduct CO to provide heat for the arc furnace; (iv) large amounts of solid slag waste, mostly Ca(OH)2, are produced. Due to these disadvantages and the transition of C2H2 to C2H4 as a key chemical industry feedstock, most CaC2 plants in Western countries have shutdown. However, in China, where coal reserves are abundant and imported oil for olefin production is costly, CaC2-based C2H2 production remains significant. After rapid growth in the 21st century (Fig. 3b),17 the annual CaC2 production reached approximately 28 Mt, accounting for more than 96% of the world’s production.50 Besides C2H2 production, CaC2 is also used to produce calcium cyanamide.51

In the CaC2 route (Fig. 3a, 3c), producing 1 kg of C2H2 consumes approximately 3.1 kg of 80% pure CaC2 and 2 kg of coal, while generating 2.8 kg of Ca(OH)2 slag waste and 3.6 kg of CO2.11,15 Electricity consumption is primarily associated with the CaC2 production process, requiring about 3 to 3.5 kWh kg–1 CaC2,52 and a total electricity consumption of about 10.6 kWh kg–1 C2H2.13 Several process improvements have been proposed over the years, such as reusing Ca(OH)2 slag waste as building materials or for lime production via dehydration,18,53,54 CO2 capture by Ca(OH)2 to form limestone through mineralization,48 CO recovery,11 the use of renewable electricity,55 and the use of biochar to replace coal.16,54,56,57 These methods could reduce slag waste, CO2 emissions, and coal consumption to some extent (Fig. 3c). Nevertheless, high electricity consumption remains inevitable due to the endothermic nature of the CaC2 formation reaction, so CaC2 plants are strategically placed in regions with cheap electricity. An oxythermal method, which heats a mixture of CaO, coke, and O2, can produce CaC2 with a lower energy requirement.48 However, significant energy consumption is still necessary, provided by the combustion of coke. Although other metal carbides (e.g., BaC2 and Li2C2) can be produced at relatively lower reaction temperatures, they are yet to be commercialized due to their more complex reaction pathways compared to the CaC2 route.49,58

Fig. 3. CaC2-based route. a. The process for CaC2-based C2H2 manufacturing. b. annual production of CaC2 in million metric tons (Mt) from 2000 to 2023 in China. c. four major issues associated with the CaC2 route including high energy consumption, high CO2 emission, production of slag waste, and consumption of large coal resource.

2.3.	Direct acetylene production routes (ca. 631 words)

Direct conversion of CH4 to C2 hydrocarbons (C2H2, C2H4, C2H6) is highly endothermic, with the CH4 to C2H2 reaction being the most endothermic, having a standard enthalpy of 376 kJ mol–1.32

2CH4 → C2H2 + 3H2                     ΔH = 376 kJ mol–1

2CH4 → C2H4 + 2H2                     ΔH = 202 kJ mol−1

2CH4 → C2H6 + H2                        ΔH = 65 kJ mol−1

For the CH4 to C2H2 process, the theoretical limit for energy input is 
189 kJ mol–1 CH4 and the energy required is 4.0 kWh kg–1 C2H2.59,60 Thermodynamic data show that CH4 is the most stable hydrocarbon at normal temperatures, while C2H2 is the least stable (Fig. 4a).32 With increasing temperature, the free energy of C2H2 decreases, making its formation more favorable above 1230 °C, when the free energy of formation of C2H2 becomes lower than that of all other hydrocarbons.32,61 Among various hydrocarbons as feedstocks, producing C2H2 from CH4 requires higher temperatures (Fig. 4a).32,61 As seen in Fig. 4b, CH4 conversion to C2H2 and C2H4 is thermodynamically limited at 600 °C, but these limitations can be alleviated at 1200 °C.26 C2H2 production becomes more thermodynamically favorable than C2H4 production under high temperatures, that however, also promote C2H2 decomposition. Thus, achieving reasonable C2H2 yields from CH4 necessitates high reaction temperatures and rapid quenching of the C2H2 stream.

Two primary commercial technologies for direct C2H2 production are PC, developed by BASF in 1950, and the electric or Hüls arc (known as thermal plasma), developed by Hüls in 1940 (Fig. 4c). These technologies have explored various carbon resources, including crude oil, naphtha, and coal, but natural gas remains the only industrial feedstock currently used. The reactor structures for CH4 conversion by PC and thermal plasma are illustrated in Fig. 4c. In the PC process, natural gas and O2 are preheated separately (to 600 °C when CH4 is used) before mixing in the diffuser and passing through the burner and reaction chamber.7 Approximately 60% of CH4 combusts with O2 in the burner to stabilize the flame at a temperature above 1500 °C, facilitating the conversion of the remaining CH4.62 The reaction gas exits the reaction chamber within milliseconds, followed by rapid quenching with water or oil sprays. This process results in a C2H2 concentration of around 8 vol.% in the cracked gas.62 Unlike PC, where chemical energy from combustion provides the necessary heat, thermal plasma relies on direct heat transfer from an electric arc to the reactants.63 For natural gas feedstock, the arc burns directly in the gas, with a vortex chamber between the cathode and anode stabilizing the arc by injecting gas tangentially (Fig. 4c). The arc’s center can reach a temperature of 20000 °C, while the electrode walls remain around 600 °C.7 Similar to the PC reactor, the residence time in the arc furnace is brief, necessitating rapid quenching. Fig. 4d compares these two technologies.7,11,15,32 The thermal plasma method yields higher C2H2 concentration in the cracked gas, lower natural gas consumption, and reduced CO2 emissions (excluding the impact of electricity). Despite these advantages, thermal plasma has largely been replaced by PC due to high soot formation and high electricity consumption (12.1 kWh kg–1 C2H2).7,64 Another type of thermal plasma plant operated by DuPont between 1963 and 1968 showed a higher C2H2 yield and lower electricity consumption of 
8.8 kWh kg–1 C2H2 using a rotating arc.65 However, the equipment design was complex, and H2 was co-fed as the plasma gas instead of the reactant CH4.65 With the increasing availability of green electricity, thermal plasma of natural gas and even coal has garnered attention in China and is likely to be commercialized soon.15,21,66 Additionally, C2H2 is also produced as an impurity in the steam cracking of hydrocarbons to olefins, typically removed by catalytic selective hydrogenation or solvent extraction.24

Fig. 4. Direct CH4 to C2H2 production routes. a. Gibbs free energy of formation per carbon atom of several hydrocarbons from the elements as a function of temperature (K). b. Gibbs free energy of formation of C2H2, C2H4, and carbon from CH4 conversion at 600 and 1200 °C. c. Simplified reactor structure for C2H2 production via PC and thermal plasma (electric arc), and d. comparison of these two technologies.

2.4.	Emerging acetylene synthesis routes and catalytic applications (ca. 831 words)

The TP of CH4 in reactor tubes, heated either by electricity or combustion gas, can produce C2H2. However, this method often results in lower energy efficiency and significant soot formation due to slow feed heating compared to PC. Adding H2 can shift the equilibrium to inhibit C2H2 decomposition, but it also reduces the CH4 conversion rate, making this approach commercially unviable.32 Recent advancements have explored non-thermal plasma technologies, including electrode-based plasma (dielectric barrier discharge (DBD), corona, pulsed, and spark) and electrodeless microwave plasma for C2H2 synthesis (Fig. 5a).33 Non-thermal plasmas channel energy preferentially to electrons, creating a much higher electron temperature than the temperature of ions and neutrals. When the neutral temperature is close to room temperature, the plasma is termed “cold plasma” (e.g., DBD, corona, and pulsed plasma), whereas microwave and spark plasma can heat neutrals up to several thousand kelvins, making thermal activation significant; these are known as “warm plasma”.33 Electrode-based plasmas require electrodes for gas ionization to form plasma, while microwave plasma induces ionization via microwave irradiation (Fig. 5a), avoiding soot deposition issues on electrodes that affect plasma stability.7

The comparison of representative results from (catalytic) thermal pyrolysis, thermal plasma, and (catalytic) non-thermal plasma processes is summarized in Fig. 5b (Supplementary Table S1 and S2). Overall performance, based on CH4 conversion and C2H2 selectivity indicates that thermal plasma (electric arc) outperforms non-thermal plasma, which, in turn, outperforms TP methods (Fig. 5b). For non-thermal plasma, cold plasma’s main advantage is eliminating the need for quenching since C2H2 is produced at low temperatures. In contrast, warm plasma typically achieves higher CH4 conversion due to higher neutral temperatures (Supplementary Table S1). Feedstocks typically include pure CH4 or CH4 diluted in inert gas, with some attempts using co-feeds such as O2,67,68 H2,69 and CO2.70 However, both cold and warm plasmas require substantial energy input to ensure plasma formation, often exceeding Hüls arc plasma processes, which require 12.1 kWh kg–1 C2H2 (Fig. 5c). Moreover, promising non-thermal results (Supplementary Table S1) mostly derive from microwave or pulsed plasma, which typically operates at low pressures or high frequencies, necessitating complex equipment requirements and limiting commercial viability.33 Catalytic attempts on non-thermal plasma have been widely explored, with DBD and microwave reactors being the most commonly used (Supplementary Table S1). For instance, in dielectric barrier plasma, adding the MFM-300(Fe) catalyst increased C2H2 selectivity from 17% to 38% at CH4 conversion of <3%.71 In contrast, in microwave plasma, catalysts appear to have limited effect, with microwave irradiation being essential for significant C2H2 production. For example, microwave irradiation increased C2H2 selectivity from 0% to 35% at 700 °C with around 10% CH4 conversion over Mo/ZSM-5@SiC catalyst.72 However, similar C2H2 selectivity was obtained under microwave irradiation with (33.7%) and without (38.4%) a Ni powder catalyst at 31% CH4 conversion.73 It is essential to consider thermal activation in warm plasma due to high neutral temperatures, as its role cannot be overlooked. While extensive literature highlights the significance of non-thermal plasma on product yield, detailed energy requirements are sometimes not provided.

For thermal pyrolysis, the oxidative coupling of CH4 (OCM) and non-oxidative coupling of CH4 (NCM) can produce C2 species, including C2H2, C2H4, and C2H6.26 The slower heat transfer in TP compared to PC technology limits commercial processes like the Wulff TP process.32 Therefore, TP could be competitive only if reaction temperatures are lowered (<1100 °C) using catalysts. OCM is thermodynamically more favorable than NCM, requiring lower reaction temperatures (450–1000 °C), but it is challenging to avoid overoxidation to COx.74 However, at low temperatures, the reaction of CH4 to C2H2 becomes less thermodynamically favorable compared to the formation of other hydrocarbons (Fig. 4a, 4b). As a result, C2H2 production from TP primarily comes from NCM rather than OCM (Supplementary Table S2). The highest C2H2 selectivity reported is 35% at 14.4% CH4 conversion at 975 °C on Pt1/CeO2 catalyst,75 outperforming pristine CeO2, Ptn/CeO2, metal carbides, and Fe/Ni-based catalysts (Fig. 5d),76–78 though CH4 conversion is typically below 30% at temperatures <1100 °C. A CH4 conversion of 63% is obtained on Pt on α-Al2O3 monolith (Fig. 5b), but a significant temperature gradient in the monolith from 200 to 1500 °C is observed, which is already close to the temperature in PC.79 Additionally, coke deposition is significant due to thermodynamically favorable coke formation (Fig. 4a). Advanced reactor designs, including membrane or millisecond reactors, have been explored for enhanced CH4 conversion due to the shift of reaction equilibrium to the product side and improved stability due to shortened contact time.80,81 However, achieving high CH4 conversion and high C2H2 selectivity simultaneously via TP remains challenging. As seen in Fig. 5d, other C2 products, especially C2H4 (Supplementary Table S2), are always present in large amounts, significantly affecting C2H2 selectivity. The presence of other byproducts such as C3-C5 hydrocarbons, aromatics, and coke further limits C2H2 formation.77,82 Therefore, converting CH4 to a broader product distribution, such as C2 (C2H2, C2H4, C2H6) or even C2+ hydrocarbons mixture, should be a more practical route for future CH4 utilization via catalytic TP.

Fig. 5. Emerging acetylene synthesis routes. a. Structure of electrode- and electrodeless-based plasma reactors among non-thermal plasma. b. Conversion-selectivity trend from thermal pyrolysis, non-thermal and thermal plasma. c. Conversion-selectivity trend and energy requirement (kWh kg–1 C2H2) for representative non-thermal (cold and warm) and thermal plasma routes. d. Representative catalyst type, reaction temperature (°C), CH4 conversion, C2H2 and C2 selectivity in non-oxidative CH4 conversion.

- Sustainable carbon resources for acetylene synthesis (ca. 734 words)

Fossil fuel-based C2H2 production relies on four primary technologies: CaC2, PC, TP, and plasma (Fig. 6a). Currently, coal-based CaC2, particularly prevalent in China, and natural gas-based PC dominate the market for C2H2 production. While these methods require relatively lower energy input compared to the thermal plasma route, they demand significant coal or natural gas input and have substantial environmental impacts, making them unsustainable for future chemical processes (Fig. 3c, 4d). Thus, developing C2H2 production approaches using sustainable carbon resources (e.g., biochar, biomethane, CO2 from air or biogas, plastic) and renewable electricity (e.g., wind and solar) is imminent. Fig. 6b illustrates potential combinations of sustainable carbon resources and relevant technologies for C2H2 synthesis. Biochar, derived from biomass pyrolysis, has been proposed for C2H2 synthesis via the CaC2 route with a lower carbon footprint, though it typically exhibits lower carbon and energy efficiency due to the lower energy density of biomass and the generation of volatile components during pyrolysis.16,54,56 Additionally, direct TP of various biomass feedstocks to C2H2 has been widely studied but is limited by the selectivity towards C2H2.63 Recently, the coal-based plasma route has gained attention in China as a greener alternative to the coal-based CaC2 route.21 Biomass as a substitute for coal in the plasma route has also been explored, such as lignin and hemicellulose cracking in a hydrogen plasma, further reducing the carbon footprint.83

Moreover, the pyrolysis of polyolefins and polyethylene terephthalate (PET) using rotating arc plasma technology has proven feasible for C2H2 production, achieving full carbon conversion and C2H2 selectivities of 55% for PET and 80% for polyolefins.84,85 However, the specific energy requirement is approximately 20 kWh kg–1 C2H2, significantly higher than the commercial Hüls arc plasma process. Biogas, as a renewable carbon feedstock composed mainly of CH4 and CO2, could replace natural gas for C2H2 synthesis with a lower carbon footprint. For instance, the conversion of biogas-like mixtures of CH4 and CO2 into C2 products can be achieved using DBD cold plasma, with C2H6 as the dominant product.86 Moreover, the addition of a catalyst (Ni–K/Al2O3) and increasing the reaction temperature above 220 °C can enhance C2H2 production from biogas in DBD cold plasma.87 Direct conversion of biogas via TP and PC is challenging and has not been extensively studied, despite CH4 being a common feedstock for direct C2H2 production. However, the abundant availability of CH4 in biogas or as a byproduct of CO2 hydrogenation in biogas presents an opportunity to utilize biogas-derived biomethane for C2H2 production using similar technologies (plasma, TP, PC) as those used for CH4 (Fig. 6b).31 Therefore, biomethane derived from biomass is a potential sustainable carbon resource alternative to natural gas.

Moreover, the CO2 in biogas or directly in air can be integrated into the C2H2 production process via CO2-CO or CO2-C2H2 cycles. A sustainable route for C2H2 production, coupled with CO2 capture and conversion to CO (CO2-CO cycle), based on the BaCO3-BaC2-Ba(OH)2-BaCO3 cycle, has been reported (Fig. 6c).49 Unlike the CaC2 route, where CaO is obtained from CaCO3 decomposition with significant CO2 emissions, the CO2-CO cycle does not directly emit CO2. If Ca replaces Ba in this cycle, a large amount of CO2 would still be released before reacting with carbon due to similar thermodynamic reaction temperatures between CaCO3 decomposition and the Boudouard reaction (CO2 + C → 2CO).49 Additionally, BaC2 synthesis requires lower temperatures and less energy input compared to CaC2. Notably, the carbon source in CO2 and half of the carbon sources (coal or biochar) convert to CO instead of C2H2 (Fig. 6c). In contrast, the CO2-C2H2 concept cycle directly transforms CO2 into C2H2 through a cyclic electrochemical-assisted strategy (Fig. 6d).58 This cycle involves three steps: (i) LiOH electrolysis to produce metallic Li0 with H2O and O2 as byproducts; (ii) the active Li0 reacting with CO2 to form Li2C2 with Li2O or LiCO3 as byproducts; and (iii) hydrolysis of Li2C2 to form C2H2 and regenerate LiOH. This cycle uses CO2 and H2O without additional carbon input, presenting a greener route than the CO2-CO cycle. However, the recycling of Li2O or LiCO3 byproducts was not addressed, limiting the viability of the electrochemical-assisted CO2-C2H2 cycle. The CO2-mediated concept cycle for C2H2 production shows promise for sustainability, but the research is still in its early stages. Given the current foundation of coal- and natural gas-derived technologies, incorporating biochar or biomethane as potential green alternatives for C2H2 production is more practical at this stage.

Fig. 6. Potential green C2H2 synthesis routes. a. Coal, natural gas, and oil-based technologies, including CaC2, PC, TP, and plasma routes. b. biochar, biogas, CO2, and plastic based technologies, including CaC2, plasma, cycle, TP, and PC. CO2-based concept cycles include c. BaC2-based route coupling CO2 capture and CO production (CO2-CO cycle), and d. Li2C2-based electrochemical-assisted three-step cycle from LiOH to Li, with Li reacting with CO2 to form Li2C2, followed by reaction with H2O to produce C2H2 (CO2-C2H2 cycle).

- Comparison of traditional and green acetylene synthesis routes (811 words)

Recently, a framework prioritizing the environmental, economic, and equity (EEE) pillars was proposed to guide the development of sustainable chemical processes.88 This section evaluates the environmental impact through a comprehensive life cycle assessment (LCA) and assesses the economic viability of various acetylene (C2H2) synthesis technologies. Additionally, we compare these technologies using both fossil and renewable carbon resources, assessing their environmental impacts within the context of a future decarbonized scenario. For detailed insights into our methodology, please refer to the Supplementary Information.

We specifically compared the climate change impacts (Fig. 7a) and production costs (excluding capital expenditure) (Fig. 7b) associated with the production of one kilogram of C2H2 using three different processes: CaC2, PC, and plasma (both thermal and non-thermal). Supplementary Table xx shows the mass balance, and Supplementary Table xx lists the prices of feedstock and utilities used in estimating the production costs for all C2H2 production routes. We evaluated two scenarios for all production technologies: fossil-based and green. For the CaC2 process, we used two feedstocks: coke (fossil) and biochar (green). For the technologies utilizing methane as a feedstock in the fossil scenario (PC and plasma routes), we replaced methane with biomethane in the green scenario. Furthermore, we conducted a prospective LCA to evaluate the projected climate change impacts for the year 2050 under a decarbonized scenario. Given that C2H2 production technologies are highly energy-intensive, requiring between 3.3 and 12.1 kWh kg–1 C2H2, they have the potential to perform exceptionally well with the anticipated future decarbonized electricity mix.

The highest climate change impacts are observed in the CaC2 production scenario (12.9 kg CO2-eq kg–1) due to its heavy reliance on coke and electricity to produce calcium carbide, in addition to the substantial direct emissions. In contrast, PC has the lowest emissions (7.8 kg CO2-eq kg–1), utilizing methane as the primary feedstock and resulting in much lower direct emissions compared to the CaC2 scenario. Both plasma technologies have higher climate change impacts than the PC routes, primarily due to the high electricity requirements for these pathways. Alternatively, emissions are significantly reduced across all green scenarios. Notably, PC displays a negative climate change impact of 
–6.1 kg CO2-eq kg–1 due to the utilization of biomethane feedstock, which results in negative net contributions to the overall impacts (on a cradle-to-gate basis) due to the biogenic nature of the carbon.89 Similarly, the impacts for the green CaC2 route show a 75% decrease due to the shift from coke to carbon-negative biochar.90 In contrast to the significant reductions observed in the CaC2 and PC routes, both plasma technologies show only a 45% decrease under the green scenario due to their heavy reliance on grid electricity.

In a future decarbonized scenario projected for 2050, emissions are expected to significantly decrease, primarily due to the adoption of a decarbonized electricity mix.91 Here, we highlight (purple scatter in Fig. 7a) the most optimistic scenario (1.5 °C – RCP 1.9).92 The CaC2 routes demonstrate a reduction of 63% compared to current impacts under the fossil scenario and 283% under the green scenario. Similar reductions are observed across all production routes. These results underscore the potential of a decarbonized grid to mitigate climate change impact, even in fossil-based production methods, and even more so in the green scenario.

For the production costs within the fossil scenarios, the CaC2 route has the lowest cost (1.2 USD kg–1), primarily driven by electricity. The PC route is more expensive 
(2.0 USD kg–1), largely due to methane costs. Similarly, both plasma routes have similar costs, averaging 1.7 USD kg–1, predominantly due to electricity. In the green scenario, the CaC2 route is the most economical, with a 32% increase in costs due to the slightly higher cost of biochar compared to coke. The green PC scenario is the most expensive route, with a 340% increase in costs, driven by the substantial cost of biomethane (five times more expensive than methane). Similarly, due to the higher price of biomethane, both plasma routes show at least a 110% increase in costs.

Overall, in the short term, the primary focus should be on optimizing the existing CaC2 production process. We can begin by transitioning from coke to a blend of coke and biochar, ensuring stable supply chains of biochar to maintain cost-effectiveness. Moreover, the lower costs of biochar and its excellent reduction in climate change impacts make the green CaC2 route the most favorable option to produce C2H2. In the long term, transitioning to renewable energy sources and engaging in grid decarbonization initiatives for electricity will further reduce the climate change impacts of plasma-based C2H2 at lower costs. Moreover, achieving higher selectivities towards C2H2 in the plasma processes will be key in the future. Comprehensive analysis is, therefore, essential in engaging with stakeholders and advocating for policies to move the chemical industry towards C2H2 while ensuring sustainability and economic viability.

Fig. 7. Environmental and economic comparison of processes for C2H2 manufacture. a. Climate change impacts and b. Production costs per kg of C2H2 via the CaC2-based route, and the direct thermal (PC) or plasma routes. Results for both the fossil and green routes, depending on the hydrocarbon source, are shown. Biochar, as an alternative to coke for the CaC2 route, and biomethane, as an alternative to methane for the PC and plasma technologies, are evaluated for the green routes. Future projections (in 2050) for climate change impacts for all technologies are presented to illustrate the potential reduction in impacts under a decarbonized electricity mix scenario.

- Perspective and outlook (ca. 754 words)

Commercial production of C2H2 relies on three well-established technologies: CaC2, thermal plasma, and PC, utilizing coal and natural gas as feedstocks. Emerging alternatives for C2H2 production include non-thermal plasma and TP. Non-thermal plasma demonstrates performance close to commercial thermal plasma technology, but the C2H2 yield through TP remains limited to less than 10% (Fig. 5b). Despite non-thermal plasma achieving high C2H2 yields that meet industrial standards, it typically requires higher energy and more complex equipment compared to thermal plasma. Therefore, future research should focus on increasing energy utilization efficiency to reduce energy requirements to the theoretical limit (i.e., 4.0 kWh kg–1 C2H2) and on designing durable equipment. Although warm plasma shows higher CH4 conversion due to high neutral temperatures, C2H2 formation in cold plasma at room temperature avoids C2H2 decomposition into carbon and eliminates costly cooling processes. TP of CH4 to C2H2 faces thermodynamic limitations at low temperatures. At high temperatures (>1500 °C), TP is less competitive than PC due to lower energy efficiency and significant soot formation. At lower temperatures (<1100 °C), catalysts are critical for improving activity and C2H2 selectivity. Despite decades of effort, C2H2 selectivity remains below 40% (Fig. 5b), with byproducts including other C2 (C2H4, C2H6), C3-C5, aromatics, and coke. Given that C2H2 is not thermodynamically favorable at low temperatures, the formation of other hydrocarbons is inevitable. Therefore, it might be more practical to focus TP on selective CH4 conversion to C2 products or mixtures of C2 and other hydrocarbons with enhanced C2H2 selectivity.

Without significant advancements in alternative technologies, well-established technologies, particularly coal-derived CaC2 (prevalent in China) and natural gas-derived PC, will continue to dominate C2H2 production from an economic perspective (Fig. 7b). However, these routes have high climate change impacts (Fig. 7a), particularly the CaC2 route (12.9 kg CO2-eq kg–1). Incorporating sustainable carbon feedstocks (biochar or biomethane) and decarbonized electricity into these technologies can lower the carbon footprint, albeit with increased production costs. Replacing natural gas with more expensive biomethane is not suitable for the PC route, as it consumes almost three times more biomethane compared to the plasma technologies. Consequently, biochar-based CaC2 and biomethane-based plasma routes will become more prominent in the future. Additionally, exploring innovative methods, such as catalytic TP for C2 production with enhanced C2H2 selectivity and (catalytic) cold/warm plasma for efficient C2H2 production with lower energy consumption, should also be considered. Electrocatalytic and photocatalytic CH4 coupling have been widely studied, but the dominant products are C2H6 or C2H4 instead of C2H2.93–95 Therefore, understanding why C2H2 production is inhibited in these processes is critical for future research. Plasma-assisted plastic upgrading to C2H2 is promising but requires a significant energy input 
(20 kWh kg–1 C2H2), making it unsuitable for practical applications without further development.84,85 Transforming CO2 and H2O into C2H2 and O2 via the CO2-C2H2 concept cycle represents an ideal but nascent approach, necessitating further academic research on Li recycling, safety, C2H2 yield, and reactor design.58

To revive C2H2 chemistry, C2H2 must compete with olefins in producing downstream bulk chemicals, such as VCM, acrylic acid, and 1,4-butanediol (Fig. 2). The C2H2 route typically involves fewer steps due to its high reactivity. For instance, VCM production via direct C2H2 hydrochlorination is more straightforward than C2H4-based methods.43 The main challenge lies in producing VCM from C2H2 in an environmentally friendly manner without utilizing mercury catalysts. The recent commercialization of an Au/C catalyst in China for C2H2 hydrochlorination addresses some environmental concerns associated with mercury catalysts.96,97 Additionally, many applications of C2H2 were only studied by Reppe in the mid-20th century but have not attracted sufficient attention in the past decades. In the future, more research on efficient C2H2 conversion is needed before the revival of C2H2 chemistry. Given the explosive potential of C2H2, on-site generation for applications is advantageous.32 For the CaC2 route, shipping solid CaC2 instead of C2H2 presents a significant advantage despite the high climate impact associated with this route using coal feedstock.

The transition of C2H2 production from fossil resources (coal and natural gas) and grid electricity (currently fossil-based) to renewable carbon sources (biochar and biomethane) and a decarbonized electricity mix is crucial for reducing the carbon footprint of mature technologies. Due to the high cost of renewable carbon sources, especially biomethane, PC appears to be an infeasible option in the long term. However, CaC2 and plasma technologies show promise, depending on the availability and cost of biochar and biomethane, respectively. Other emerging technologies, such as non-thermal plasma, TP, plasma-assisted plastic upgrading, and CO2-derived cycles, will face commercialization challenges without technological advancements.

Acknowledgements

This publication was created as part of NCCR Catalysis (grant number 180544), a National Centre of

Competence in Research funded by the Swiss National Science Foundation.

Author contributions

Z.Z. and J.P.R. conceived the review. A.N. created Figure 7, while Z.Z. produced the other figures. A.N. and G.G.G completed the life cycle assessment study. Z.Z and A.N. co-wrote the manuscript. G.G.G and J.P.R. supervised the project. All authors reviewed, edited and enriched each section of the review.

Competing interests

The authors declare no competing interests.

References

1.	Davy, E. Notice of Carburet of Potassium, and of a New Gaseous Bi-Carburet of Hydrogen. J. Frankl. Inst. 19, 216 (1837).

2.	Rasmussen, S. C. Acetylene and Its Polymers: 150+ Years of History. (Springer International Publishing, Cham, 2018).

3.	Carolina, N. Discovery of the Commercial Processes for Making Calcium Carbide and Acetylene. (American Chemical Society, 1998).

4.	Cave, H. The Development of the Oxy-Acetylene Welding and Cutting Industry in the United States. Proc. Inst. Mech. Eng. 96, 73–86 (1919).

5.	Trotuş, I.-T., Zimmermann, T. & Schüth, F. Catalytic Reactions of Acetylene: A Feedstock for the Chemical Industry Revisited. Chem. Rev. 114, 1761–1782 (2014).

6.	BIGELOW, M. H. Reppe’s Acetylene Chemistry. Chem. Eng. News Arch. 25, 1038–1042 (1947).

7.	Jiang, C. & Hu, J. Conversion of Methane to Acetylene. (Royal Society of Chemistry, 2023).

8.	Bixler, G. H. & Coberly, C. W. Wulff Process Acetylene. Ind. Eng. Chem. 45, 2596–2606 (1953).

9.	Weissermel, K. & Arpe, H.-J. Industrial Organic Chemistry. (John Wiley & Sons, 2003).

10. Crafts, J. M. A Lecture Upon Acetylene. Science 3, 377–392 (1896).

11. Schobert, H. Production of Acetylene and Acetylene-based Chemicals from Coal. Chem. Rev. 114, 1743–1760 (2014).

12. Crynes, B. L., Albright, L. F. & Tan, L.-F. Thermal Cracking. in Encyclopedia of Physical Science and Technology (3rd ed.) (ed. Meyers, R. A.) 613–626 (Academic Press, 2003).

13. Shlyapin, D. A., Afonasenko, T. N., Glyzdova, D. V., Leont’eva, N. N. & Lavrenov, A. V. Acetylene Production Technologies in the 21st Century: Main Trends of Their Development in the Paradigm of Low-Carbon Economy of the Future. Catal. Ind. 14, 251–267 (2022).

14.	Chauhan, R., Sartape, R., Minocha, N., Goyal, I. & Singh, M. R. Advancements in Environmentally Sustainable Technologies for Ethylene Production. Energy Fuels 37, 12589–12622 (2023).

15.	Zhang, S. et al. Life cycle assessment of acetylene production from calcium carbide and methane in China. J. Clean. Prod. 322, 129055 (2021).

16.	Jiang, P. et al. Towards carbon neutrality of calcium carbide-based acetylene production with sustainable biomass resources. Green Energy Environ. 9, 1068–1078 (2024).

17.	Huo, H. et al. Case study of a novel low rank coal to calcium carbide process based on techno-economic assessment. Energy 228, 120566 (2021).

18.	Liu, Y. et al. Life cycle assessment of beneficial use of calcium carbide sludge in cement clinker production: A case study in China. J. Clean. Prod. 418, 138003 (2023).

19.	Stournaras, Y. A. Are Oil Price Movements Perverse? A Crit. Explanation of Oil Price Levels 1950 - 1985. (Oxford Inst. for Energy Studies, Oxford, 1985).

20.	Gadea, M. D., Gómez-Loscos, A. & Montañés, A. Oil Price and Economic Growth: A Long Story? Econometrics 4, 41 (2016).

21.	Yan, B., Lu, W. & Cheng, Y. China goes green: cleaner production of chemicals. Green Process Synth 1, 33–47 (2012).

22.	Zimmermann, H. & Walzl, R. Ethylene. in Ullmann’s Encyclopedia of Industrial Chemistry (John Wiley & Sons, Ltd, 2009). doi:10.1002/14356007.a10_045.pub3.

23.	Amghizar, I., N. Dedeyne, J., J. Brown, D., B. Marin, G. & Geem, K. M. V. Sustainable innovations in steam cracking: CO 2 neutral olefin production. React. Chem. Eng. 5, 239–257 (2020).

24.	Ren, T., Patel, M. & Blok, K. Olefins from conventional and heavy feedstocks: Energy use in steam cracking and alternative processes. Energy 31, 425–451 (2006).

25.	Zhang, M. & Yu, Y. Dehydration of Ethanol to Ethylene. Ind. Eng. Chem. Res. 52, 9505–9514 (2013).

26.	Gao, Y. et al. Recent Advances in Intensified Ethylene Production—A Review. ACS Catal. 9, 8592–8621 (2019).

27.	Singh, R. P., Kumar, S., Dubey, S. & Singh, A. A review on working and applications of oxy-acetylene gas welding. Mater. Today Proc. 38, 34–39 (2021).

28.	Khodabakhshi, S., Fulvio, P. F. & Andreoli, E. Carbon black reborn: Structure and chemistry for renewable energy harnessing. Carbon 162, 604–649 (2020).

29.	Zhu, Y., Yang, J., Mei, F., Li, X. & Zhao, C. Bio-based 1,4-butanediol and tetrahydrofuran synthesis: perspective. Green Chem. 24, 6450–6466 (2022).

30.	Chiang, C. K. et al. Electrical Conductivity in Doped Polyacetylene. Phys. Rev. Lett. 39, 1098–1101 (1977).

31.	Bowker, M. et al. Advancing Critical Chemical Processes for a Sustainable Future: Challenges for Industry and the Max Planck–Cardiff Centre on the Fundamentals of Heterogeneous Catalysis (FUNCAT). Angew. Chem. Int. Ed. 61, e202209016 (2022).

32.	Pässler, P. et al. Acetylene. in Ullmann’s Encyclopedia of Industrial Chemistry (ed. Wiley-VCH) (Wiley, 2011). doi:10.1002/14356007.a01_097.pub4.

33.	Scapinello, M., Delikonstantis, E. & Stefanidis, G. D. The panorama of plasma-assisted non-oxidative methane reforming. Chem. Eng. Process. Process Intensif. 117, 120–140 (2017).

34.	Bilera, I. V. & Lebedev, Yu. A. Plasma-Chemical Production of Acetylene from Hydrocarbons: History and Current Status (A Review). Pet. Chem. 62, 329–351 (2022).

35.	Wu, C., Chen, J. & Cheng, Y. Thermodynamic analysis of coal pyrolysis to acetylene in hydrogen plasma reactor. Fuel Process. Technol. 91, 823–830 (2010).

36.	Zhang, N., Wang, S., Gibril, M. E. & Kong, F. The copolymer of polyvinyl acetate containing lignin-vinyl acetate monomer: Synthesis and characterization. Eur. Polym. J. 123, 109411 (2020).

37.	KK, S., Verma, R. & Kumar, L. Ethylene-Vinyl Acetate Foam. in Polymeric Foams: Fundamentals and Types of Foams (Volume 1) vol. 1439 205–221 (American Chemical Society, 2023).

38.	Okoro, O. V., Nie, L., Alimoradi, H. & Shavandi, A. Waste Apple Pomace Conversion to Acrylic Acid: Economic and Potential Environmental Impact Assessments. Fermentation 8, 21 (2022).

39.	Zhang, J. et al. Advances in the catalytic production of acrylonitrile. Chem Catal. 4, 100825 (2024).

40.	Reesh, A. & M, I. Acrylonitrile Process Enhancement through Waste Minimization: Effect of Reaction Conditions and Degree of Backmixing. Sustainability 13, 7923 (2021).

41.	Pal, P. & Nayak, J. Acetic Acid Production and Purification: Critical Review Towards Process Intensification. Sep. Purif. Rev. 46, 44–61 (2017).

42.	Sarchami, T., Batta, N. & Berruti, F. Production and separation of acetic acid from pyrolysis oil of lignocellulosic biomass: a review. Biofuels Bioprod. Biorefining 15, 1912–1937 (2021).

43.	Ma, H., Wang, Y., Qi, Y., Rout, K. R. & Chen, D. Critical Review of Catalysis for Ethylene Oxychlorination. ACS Catal. 10, 9299–9319 (2020).

44.	Faust Akl, D. et al. Reaction-Induced Formation of Stable Mononuclear Cu(I)Cl Species on Carbon for Low-Footprint Vinyl Chloride Production. Adv. Mater. 35, 2211464 (2023).

45.	Cannon, S., Levine, S. & Ritter, A. Natural Gas Conversion into Clean Hydrogen and Carbon for Li-ion Battery Electrodes Using Renewable Energy.

46.	Satam, C. C. & Realff, M. J. Comparison of two routes for the bio-based production of economically important C4 streams. J. Adv. Manuf. Process. 2, e10054 (2020).

47.	De Bari, I. et al. From Cardoon Lignocellulosic Biomass to Bio-1,4 Butanediol: An Integrated Biorefinery Model. Processes 8, 1585 (2020).

48.	Wang, H. et al. Carbon-calcium composite conversion of calcium carbide-acetylene system: On the imperative roles of carbon capture and solid waste recycling. Appl. Energy 327, 120139 (2022).

49.	Li, M. et al. Reengineering of the carbon-to-acetylene process featuring negative carbon emission. Green Chem. 25, 8584–8592 (2023).

50.	Ma, S. et al. Energy and exergy analysis of a new calcium carbide production process. Fuel Process. Technol. 226, 107070 (2022).

51.	Ropp, R. C. Chapter 5 - Group 14 (C, Si, Ge, Sn, and Pb) Alkaline Earth Compounds. in Encyclopedia of the Alkaline Earth Compounds (ed. Ropp, R. C.) 351–480 (Elsevier, Amsterdam, 2013). doi:10.1016/B978-0-444-59550-8.00005-3.

52.	Guo, J. & Zheng, D. Thermodynamic Analysis of Low-Rank-Coal-Based Oxygen-Thermal Acetylene Manufacturing Process System. Ind. Eng. Chem. Res. 51, 13414–13422 (2012).

53.	Gong, X. et al. Recycling and utilization of calcium carbide slag - current status and new opportunities. Renew. Sustain. Energy Rev. 159, 112133 (2022).

54.	Pääkkönen, A., Tolvanen, H. & Kokko, L. The economics of renewable CaC2 and C2H2 production from biomass and CaO. Biomass Bioenergy 120, 40–48 (2019).

55.	Huo, H. et al. Life cycle assessment and sustainable production evaluation of calcium carbide industry in China. J. Clean. Prod. 360, 132176 (2022).

56.	Li, G. et al. Production of Calcium Carbide from Fine Biochars. Angew. Chem. Int. Ed. 49, 8480–8483 (2010).

57.	Li, Z., Liu, Z., Wang, R., Guo, X. & Liu, Q. Conversion of bio-char to CaC2 at low temperatures-morphology and kinetics. Chem. Eng. Sci. 192, 516–525 (2018).

58.	M. McEnaney, J. et al. A cyclic electrochemical strategy to produce acetylene from CO2 , CH4 , or alternative carbon sources. Sustain. Energy Fuels 4, 2752–2759 (2020).

59.	Kang, H. et al. Methane to acetylene conversion by employing cost-effective low-temperature arc. Fuel Process. Technol. 148, 209–216 (2016).

60.	Radoiu, M. T., Chen, Y. & Depew, M. C. Catalytic conversion of methane to acetylene induced by microwave irradiation. Appl. Catal. B Environ. 43, 187–193 (2003).

61.	Holmen, A., Olsvik, O. & Rokstad, O. A. Pyrolysis of natural gas: chemistry and process concepts. Fuel Process. Technol. 42, 249–267 (1995).

62.	Vicari, M. BASF and acetylene. 70 years of reppe chemistry - long-standing reliability and promising future - and now, the only natural gas based clean technology for acetylene production. (2013).

63.	Gyrdymova, Y., Lebedev, A., Du, Y.-J. & Rodygin, K. Production of Acetylene from Viable Feedstock: Promising Recent Approaches. ChemPlusChem n/a, e202400247.

64.	Fincke, J. R. et al. Plasma Thermal Conversion of Methane to Acetylene. Plasma Chem. Plasma Process. 22, 105–136 (2002).

65.	Anderson, R. P., Fincke, J. R. & Taylor, C. E. Conversion of natural gas to liquids via acetylene as an intermediate. Fuel 81, 909–925 (2002).

66.	Yan, B., Xu, P., Guo, C. Y., Jin, Y. & Cheng, Y. Experimental study on coal pyrolysis to acetylene in thermal plasma reactors. Chem. Eng. J. 207–208, 109–116 (2012).

67.	Kado, S., Sekine, Y. & Fujimoto, K. Direct synthesis of acetylene from methane by direct current pulse discharge. Chem. Commun. 0, 2485–2486 (1999).

68.	Han, Q. et al. Conversion of methane to C 2 and C 3 hydrocarbons over TiO 2 /ZSM-5 core–shell particles in an electric field. RSC Adv. 9, 34793–34803 (2019).

69.	Wnukowski, M., van de Steeg, A. W., Hrycak, B., Jasiński, M. & van Rooij, G. J. Influence of hydrogen addition on methane coupling in a moderate pressure microwave plasma. Fuel 288, 119674 (2021).

70.	Zhang, X., Dai, B., Zhu, A., Gong, W. & Liu, C. The simultaneous activation of methane and carbon dioxide to C2 hydrocarbons under pulse corona plasma over La2O3/γ-Al2O3 catalyst. Catal. Today 72, 223–227 (2002).

71.	Ma, Y. et al. Direct Conversion of Methane to Ethylene and Acetylene over an Iron-Based Metal–Organic Framework. J. Am. Chem. Soc. 145, 20792–20800 (2023).

72.	Julian, I., Ramirez, H., Hueso, J. L., Mallada, R. & Santamaria, J. Non-oxidative methane conversion in microwave-assisted structured reactors. Chem. Eng. J. 377, 119764 (2019).

73.	Suib, S. L. & Zerger, R. P. A Direct, Continuous, Low-Power Catalytic Conversion of Methane to Higher Hydrocarbons via Microwave Plasmas. J. Catal. 139, 383–391 (1993).

74.	Ortiz-Bravo, C. A., Chagas, C. A. & Toniolo, F. S. Oxidative coupling of methane (OCM): An overview of the challenges and opportunities for developing new technologies. J. Nat. Gas Sci. Eng. 96, 104254 (2021).

75.	Xie, P. et al. Nanoceria-Supported Single-Atom Platinum Catalysts for Direct Methane Conversion. ACS Catal. 8, 4044–4048 (2018).

76.	Y. Zhang, S. B. X. et al. Role and dynamics of transition metal carbides in methane coupling. Chem. Sci. 14, 5899–5905 (2023).

77.	Han, S. J., Lee, S. W., Kim, H. W., Kim, S. K. & Kim, Y. T. Nonoxidative Direct Conversion of Methane on Silica-Based Iron Catalysts: Effect of Catalytic Surface. ACS Catal. 9, 7984–7997 (2019).

78.	Hao, J. et al. Enhanced Methane Conversion to Olefins and Aromatics by H-Donor Molecules under Nonoxidative Condition. ACS Catal. 9, 9045–9050 (2019).

79.	Hohn, K. L., Witt, P. M., Davis, M. B. & Schmidt, L. D. Methane coupling to acetylene over Pt-coated monoliths at millisecond contact times. Catal. Lett. 54, 113–118 (1998).

80.	Sakbodin, M., Wu, Y., Oh, S. C., Wachsman, E. D. & Liu, D. Hydrogen-Permeable Tubular Membrane Reactor: Promoting Conversion and Product Selectivity for Non-Oxidative Activation of Methane over an Fe©SiO2 Catalyst. Angew. Chem. Int. Ed. 55, 16149–16152 (2016).

81.	Oh, S. C. et al. Direct Non-Oxidative Methane Conversion in a Millisecond Catalytic Wall Reactor. Angew. Chem. Int. Ed. 58, 7083–7086 (2019).

82.	Guo, X. et al. Direct, Nonoxidative Conversion of Methane to Ethylene, Aromatics, and Hydrogen. Science 344, 616–619 (2014).

83.	Dimitrakellis, P., Delikonstantis, E., Stefanidis, G. D. & Vlachos, D. G. Plasma technology for lignocellulosic biomass conversion toward an electrified biorefinery. Green Chem. 24, 2680–2721 (2022).

84.	Zhang, M. et al. Pyrolysis of Polyolefins Using Rotating Arc Plasma Technology for Production of Acetylene. Energies 10, 513 (2017).

85.	Zhang, M. et al. Gas production from polyethylene terephthalate using rotating arc plasma. Chem. Eng. Process. - Process Intensif. 128, 257–262 (2018).

86.	Kolb, T., Kroker, T. & Gericke, K.-H. Conversion of biogas like mixtures to C2 hydrocarbon in a plug flow reactor supported by a DBD at atmospheric pressure. Vacuum 88, 144–148 (2013).

87.	Zeng, Y. et al. Biogas reforming for hydrogen-rich syngas production over a Ni–K/Al2O3 catalyst using a temperature-controlled plasma reactor. Int. J. Hydrog. Energy 48, 6192–6203 (2023).

88.	Mitchell, S., Martín, A. J., Guillén-Gosálbez, G. & Pérez-Ramírez, J. The Future of Chemical Sciences is Sustainable. Angew. Chem. Int. Ed. 63, e202318676 (2024).

89.	Rodin, V., Lindorfer, J., Böhm, H. & Vieira, L. Assessing the potential of carbon dioxide valorisation in Europe with focus on biogenic CO2. J. CO2 Util. 41, 101219 (2020).

90.	Glaser, B., Parr, M., Braun, C. & Kopolo, G. Biochar is carbon negative. Nat. Geosci. 2, 2–2 (2009).

91.	Nabera, A., Martín, A. J., Istrate, R., Pérez-Ramírez, J. & Guillén-Gosálbez, G. Integrating climate policies in the sustainability analysis of green chemicals. Green Chem. 26, 6461–6469 (2024).

92.	Sacchi, R. et al. PRospective EnvironMental Impact asSEment (premise): A streamlined approach to producing databases for prospective life cycle assessment using integrated assessment models. Renew. Sustain. Energy Rev. 160, 112311 (2022).

93.	Yu, X. et al. Stoichiometric methane conversion to ethane using photochemical looping at ambient temperature. Nat. Energy 5, 511–519 (2020).

94.	Wu, S. et al. Ga-Doped and Pt-Loaded Porous TiO2–SiO2 for Photocatalytic Nonoxidative Coupling of Methane. J. Am. Chem. Soc. 141, 6592–6600 (2019).

95.	Zhu, C. et al. Electrochemical conversion of methane to ethylene in a solid oxide electrolyzer. Nat. Commun. 10, 1173 (2019).

96.	Johnston, P., Carthey, N. & Hutchings, G. J. Discovery, Development, and Commercialization of Gold Catalysts for Acetylene Hydrochlorination. J. Am. Chem. Soc. 137, 14548–14557 (2015).

97.	Malta, G. et al. Identification of single-site gold catalysis in acetylene hydrochlorination. Science 355, 1399–1403 (2017).