Response to Reviewers for Manuscript NATCHEMENG-24061044

Code:	Comments in blue | Replies in black | Actions in bold

Note:	Indicated page, line, figure, or reference numbers refer to the revised manuscript and/or supplemental information with changes highlighted.

Reviewer #1

There is not much to say about this manuscript: it is a very well written, comprehensive review about acetylene production with some comments also on downstream processes, including comments on sustainability and cost of the different pathways. As a review, it doesn't provide new discoveries, but it provides an interesting angle on a somewhat neglected topic. It is timely, since the chemical industry is facing a conversion process, in which acetylene could play a more important role again, and there are no recent comparable papers available, as far as I am aware. It has a very different focus compared to the review by Trotus et al., which is already ten years old. Within the scope of the review, all relevant aspects are adequately covered, the references are also appropriate. Thus, I suggest acceptance of the manuscript, I would require no changes. Whether this review format fits the scope of Nature Chemical Engineering as a recently launched journal is a question the editors have to answer - for me it would be an interesting supplement to the other papers that Nature Chemical Engineering carries.

xxx

Reviewer #2

This review aims to provide a holistic overview of acetylene production technologies, proposes sustainable production initiatives and quantifies their economic and environmental performance, and explores potential applications. From my point of view, the starting point is that the authors seek to rekindle interest and foster collaborative efforts in advancing sustainable acetylene production and broadening its use as a feedstock for bulk chemicals. However, as a review paper submitted to Nature Chem Eng, the current form is still far from being considered for publication. Following are my specific comments on the review.

xxx

2.1. As a review article, this manuscript spends the major part to discuss the conventional technologies to produce acetylene, such as CaC2 route, partial oxidation and thermal plasma pyrolysis. To be noted, the acetylene production technologies nowadays all have no connections with catalysis. The reason would be attributed to the thermodynamic features of C2H2. The group of the authors are expertized in catalysis. I can’t clearly see the C2H2-related research background of the authors, as well as the potential solutions to produce C2H2 via thermal catalysis route. Regarding the downstream organic synthesis of C2H2, I also cannot justify the new chemistry or new big market given from the review context. From my personal opinion, the authors for a review paper should have had deep understanding on such a research area with sound publications (i.e., research experience).

xxx

2.2. Some review papers on acetylene are well written in comparison with the current review manuscript, for example, Production of Acetylene and Acetylene-based Chemicals from Coal. Chem. Rev. 2014, 114, 3, 1743–1760; Catalytic Reactions of Acetylene: A Feedstock for the Chemical Industry Revisited. Chem. Rev. 2014, 114, 3, 1761–1782; Production of Acetylene from Viable Feedstock: Promising Recent Approaches. ChemPlusChem 2024, e202400247. https://doi.org/10.1002/cplu.202400247.

xxx

2.3. The difference of this review from the existing reviews in literature would be the emphasis on the green footprint of acetylene production, e.g., changing coal to biochar. Considering the scale of acetylene production in market, this replacement (coal to biochar) is not convincing, and of course not realistic, especially in China. A possible way to promote the green footprint would be the utilization of renewable energy generated electricity to drive the arc furnace or arc plasma for the high-temperature reaction environment in CaC2 route or plasma pyrolysis of hydrocarbons.

Countries other than China have largely transitioned from acetylene-based downstream production of valuable products to ethylene-based production, primarily due to the higher environmental impact of conventional calcium carbide-based acetylene production. This perspective aims to promote the production of acetylene in countries outside China by showcasing how it can be produced in an economically and environmentally sustainable manner. Different markets, depending on the country, may respond differently to the adoption of such technologies.

We acknowledge the Reviewer’s concern regarding the availability of biochar and biomethane. However, future projections indicate that by 2050, the annual availability of biochar will range from approximately 0.3 to 0.5 Gt (Fawzy, S. et al., Environ. Chem. Lett. 19, 3023 (2021)). Considering that the calcium carbide process requires 2 kg of biochar per kg of acetylene produced, rough estimates suggest that only 4 to 7% of the available biochar could meet the global acetylene demand, estimated at about 11 Mt per year. Similarly, for biomethane-based acetylene production pathways, specifically the partial oxidation route, approximately 2.8 kg of biomethane is required to produce 1 kg of acetylene. It is projected that the biomethane potential within the European Union (EU) will reach around 0.1 Gt by 2050 (Biomethane production potentials in the EU, European Biogas Association (2022)). Consequently, only 26% of this biomethane potential would be needed to meet the current acetylene demand of 11 Mt.

Nonetheless, we agree with the Reviewer that the current availability of biochar (0.4 Mt per year) and biomethane (0.03 Gt per year) is insufficient to meet the global demand for acetylene. However, numerous research studies have highlighted the environmental benefits of biochar and biomethane, leading to optimistic projections for their future availability, as mentioned earlier. We have emphasized this aspect in the main manuscript (page xx, line xx).

As highlighted by the reviewer, the prospects of using green or decarbonized electricity represents one of the most promising technologies for enhancing the environmental sustainability of acetylene production. In this context, we highlight the potential future climate change impacts of acetylene production under a decarbonized electricity grid in Fig. xx of the main manuscript (purple scatter). It is important to note that these results reflect an optimistic scenario, assuming that the 1.5 °C temperature target is achieved by 2050. However, these results are based on a global average to provide a general overview; regional electricity grids will vary significantly depending on the specific scenario and location.

2.4. The authors mentioned the major applications of C2H2 to synthesize VCM or 1, 4-butanediol. First, C2H2-VCM-PVC is indeed an important value chain, for instance, in China. However, this product chain can be totally substituted by C2H4-PVC route. One of the reasons for China to have to produce C2H2 for PVC is the balance of the chlorine from the chlor-alkali industry during the production of a large amount of alkali. Another reason would be the limited petroleum resources, which further limit the application of C2H4. Second, the technique to synthesize 1, 4-butanediol has been mature so that the production of 1, 4-butanediol is already serious overcapacity. Therefore, this part of the reviewed contents is not stimulating in the current and future world.

xxx

2.5. Regarding the environmental and economic assessment of acetylene synthesis routes, fossil based and green, I am not convinced by the replacement of methane by biomethane, coal by biochar as mentioned above. The production of C2H2 must consider where feedstock comes and how much volume of feedstock can be supplied sufficiently. Also, as mentioned above, C2H2 industry is closely connected with chlor-alkali industry. Therefore, the environmental and economic assessment of acetylene synthesis routes must be carried out in a specific chemical industrial park, or in a province/state, or across a country, even beyond. From my point of view, to improve the green footprint with acceptable environmental and economic assessment, the key is the green electricity, instead of the “green” feedstock, because of the process features.

We acknowledge the concern regarding the availability of biochar and biomethane. As noted in comment 2.3, by 2050, their projected availability–4% and 26% of their potential, respectively–will be sufficient to meet current global acetylene demands, despite present limitations. The discussion in the main manuscript has been expanded to highlight this point (page xx, line xx).

We would like to highlight that the system boundary for this analysis has been defined as the acetylene production process only. It is assumed that the calcium carbide and hydrogen chloride production facilities are integrated with the acetylene plant. Consequently, calcium carbide and hydrogen chloride are treated as inputs to the economic and environmental assessment, with their associated climate change impacts and costs accounted for accordingly. Section Sxx of the Supplementary Information has been updated to include these details.

In the context of using renewable electricity for acetylene production, as highlighted in comment 2.3, our analysis includes a climate change impact assessment based on the assumption of a decarbonized (green) electricity mix in 2050, as shown in Fig. xx. However, recognizing that no single solution fits all scenarios, we broaden our focus beyond green electricity for sustainable acetylene production to consider renewable feedstocks.

2.6. In the context, the major technical terms, such as partial combustion, hot plasma, thermal plasma, non-thermal plasma, cold plasma, warm plasma, thermal pyrolysis and so on, should not be confused.

xxx

2.7. Figure 1 gives some historical information. However, why does not anticipate the near future, e.g., extend the timeline to 2050?

xxx

2.8. Figure 2 provides the C2H2 production routes, involving past and current applications. However, trial routes and abandoned routes are not that justified. On feedstock, why not add the biomethane/biochar in? On products, why not predict possible market from the C2H2 chemistry for near future?

xxx

2.9. Figure 3: in 3(a), some technical words are not appropriate, such as dryer, carbon, C2H2 furnace(?), CO→CO2(?), etc., 3(b) is not informative at all; in 3(c) the numbers are not representative, which should be specified with the corresponding C2H2 plant.

xxx

2.10. Figure 4: 4(a) has been in many existing references; 4(b) is a bit redundant with 4(a); 4(c) is not very informative at all.

xxx

2.11. Figure 5: 5(a) is somewhat like a textbook or a slide of PPTs; 5(b) has little valued information; 5(c), such a plot (conversion vs. selectivity) can be often seen for comparing catalyst performance, however, it does not fit for C2H2 production via partial oxidation or thermal pyrolysis; 5(d) is not an appropriate evaluation for C2H2 production

xxx

2.12. Figure 6: most chemical engineers would not have interest in those “Potential green C2H2 synthesis routes.”

In the context of the ambitious targets set by global frameworks such as the Paris Climate Agreement and the United Nations Sustainable Development Goals (SDGs), sustainability is a critical issue for all industries, and particularly for the chemical industry due to its heavy reliance on fossil fuels for feedstock and as an energy vector. This reliance makes the chemical industry one of the most difficult sectors to decarbonize, accounting for approximately 5.6 Gt CO2e, or 10% of global emissions (Bauer, F. et al. Curr. Opin. Chem. Eng., 39, 100881 (2023)). As a result, the chemical industry is undergoing a major transition to achieve sustainable production by utilizing defossilized feedstocks and energy. Consequently, these potential green acetylene synthesis routes align with the broader global imperatives mentioned above.

Other comments:

2.13.1 Sections 2.2 and 2.3 discuss indirect methods of acetylene production (the calcium carbide method) and direct methods of acetylene production, are there any new developments of the methods to the research considered in the relevant reviews Schobert H. Production of acetylene and acetylene-based chemicals from coal[J]. Chemical Reviews, 2013, 114(3):1743-1760., Trotuş I T, Zimmermann T, Schüth F. Catalytic reactions of acetylene: a feedstock for the chemical industry revisited[J]. Chemical Reviews, 2014, 114: 1761-1782. a decade ago?

xxx

2.13.2. The title of section 2.3 (Direct commercial acetylene production routes) does not seem to be very appropriate, since the title of 2.2 (Indirect acetylene production route via calcium carbide) does not emphasize the word commercial. Therefore, this title may mislead readers unfamiliar with the relevant field into thinking that the calcium carbide method is not a commercial acetylene preparation method.

xxx

2.13.3. Emerging routes for acetylene synthesis are discussed in Section 2.4, where the authors describe a route for the production of acetylene from CH4 using non-thermal plasma technology. Should this technique be viewed as a different way of generating plasma or as a revolutionary new technology for acetylene production? In the paper, the authors point out that the use of non-thermal plasma actually requires a greater energy input than the use of thermal plasma (Line 237), and that the CH4 conversion and acetylene selectivity are not as good as in the thermal plasma approach. Thus, despite the savings in the quenching, non-thermal plasma technology should be far less economical than thermal plasma technology at this time. So, if this is viewed as a promising technology, what are the drivers for its development? What are the current key technical barriers? What technological breakthroughs are needed to industrialize it? At present the authors seem to be merely talk about the results reported in the literature, but not point out and analyze these issues clearly.

xxx

2.13.4. As far as I know, the production of acetylene from coal as a feedstock using plasma technology is also a promising technology and large-scale experimental installations have been established in China. However, only the acetylene production technologies based on natural gas as feedstock using plasma (thermal and non-thermal) are discussed in this paper. It is proposed to add a discussion in section 2.4 on the new developments and prospects of plasma technology for acetylene production from coal as a feedstock, in particular the new trends in recent years compared to what has been discussed in the previous reviews.

xxx

2.13.5. In section 3, the authors argue that the adoption of bio-based feedstocks for the production of acetylene, and hence PVC, could be an important driver for the revival of acetylene. However, the fact that more than 45% of the market for PVC is held by China, and that this PVC is mainly produced from calcium carbide acetylene, implies that the ATV pathway would have had an importance comparable to that of EBP. Therefore, in terms of VCM or PVC production, it needs further consideration to whether the topic about the revival of acetylene exist. In addition, it is suggested that the production and environmental costs of the four methods in Figure 6 (i.e., fossil and green ATV or EBP) be put together in order to visualize the advantages of green ATV when combining production and environmental costs.

xxx

2.14.1. The second half of section 2.1 (Summary of acetylene production routes and its applications) puts too much emphasis on the substitution of acetylene applications, contrary to the main theme which calls attention to the reuse of acetylene. Suggests that the fact that acetylene applications have been replaced is coupled with the main advantages of using acetylene for synthesis and the problems that need to be overcome.

xxx

2.14.2. Section 2.4 (Emerging direct acetylene synthesis routes and catalytic applications) is rather informative and the refined title is not accurate enough. Acetylene from thermally cracked CH4 is not part of the emerging direct acetylene synthesis route and the main focus of the presentation is on the design of new catalysts for the TP process. It is suggested that plasma acetylene production and the development of new catalyst design for the TP process be presented in two separate sections.

xxx

2.14.3. Section 5 (Environmental and economic assessment of acetylene synthesis routes) provides a more in-depth analysis of the environmental and economic evaluation of acetylene synthesis. However, processes related to acetylene inevitably need to consider safety as an equally important or even higher priority than economic and environmental aspects. It is proposed to supplement the evaluation of the safety performance related to the several synthesis methods mentioned with a summary of the risks and challenges faced by each process in terms of safety.

We agree that safety is a critical consideration in acetylene synthesis due to its high flammability, explosive nature, and susceptibility to decomposition under specific conditions. For instance, in the calcium carbide process, calcium carbide must be stored in a dry, airtight container to prevent unintended contact with moisture. Similarly, in the partial combustion process, thermal runaway reactions can occur if temperature is not carefully monitored and controlled. Finally, plasma-based processes for producing acetylene require high-voltage electrical systems, which pose risks such as shock and electrical arcing. Each acetylene production process has unique safety challenges. Accordingly, a summary of the risks and challenges associated with each process is provided in Section Sxx of the Supplementary Information.

Reviewer #3

This review comprehensively introduced acetylene production technologies and provided extensive analyses regarding the possible ways to sustainably produce acetylene. Thus, it undoubtfully demonstrate the underlying and unexcavated value of acetylene. This review is expected to contribute to the clarification of the developing trend of acetylene in industry and advance relevant research with the basic guidelines and updated data. I recommend its publication in Nature Chemical Engineering after minor revisions.

xxx

3.1. The authors planned a footprint of the revival of acetylene by only proposing the possibility of developing green acetylene, which seems not convincing enough. The authors may add extra part of discussion about the existing technologies or emerging technologies that may enlarge the application field of acetylene to make the plan more acceptable. For instance, recent work about the conversion of acetylene to ethylene using electrocatalysis way and to vinyl chloride using photocatalysis way provides the newest application field for acetylene.

xxx

3.2. The authors briefly introduced several kinds of catalysts in part 2.4. What are the key issues for these catalysts during acetylene production, which hinders efficient acetylene production. It would be better to improve several perspectives about the current challenging problems in different catalysts and thus give some guidance for future research.

xxx