Towards Green Acetylene Synthesis and Application

Zihao Zhang, Abhinandan Nabera, Gonzalo Guillén-Gosálbez, Javier Pérez-Ramírez*

Institute for Chemical and Bioengineering, Department of Chemistry and Applied Biosciences, ETH Zurich, Vladimir-Prelog-Weg 1, 8093 Zurich, Switzerland.

* Corresponding author. E-mail: jpr@chem.ethz.ch

Abstract

Acetylene, produced from coal or natural gas, was a cornerstone of the chemical industry until the 1960s due to its rich triple bond chemistry. However, the long-term global availability of inexpensive petroleum before 1973 accelerated the production of olefins, such as ethylene, diminishing acetylene’s market prominence due to their similar applications in downstream chemical production. As petroleum prices fluctuate, acetylene has regained economic viability in certain regions, notably accounting for approximately one-third of the global vinyl chloride production, despite its high carbon footprint. Environmental and economic assessment shows that replacing coal with biochar in acetylene-derived vinyl chloride production not only lowers carbon footprint but could also remain economically viable compared to the traditional ethylene route. Despite this potential, research on acetylene has been largely overlooked for decades. This review perspective provides a holistic overview the extensive analysis of acetylene production technologies, proposes sustainable production initiatives and quantifies their economic and environmental performance, and explores potential applications. By showcasing this promising trajectory, we seek to rekindle interest and foster collaborative efforts in advancing sustainable acetylene production and broadening its use as a feedstock for bulk chemicals.

Keywords: Acetylene; Catalyst design; Reactor design; Renewable carbon sources; Environmental and economic assessment.

- Introduction

Acetylene was accidentally discovered in 1836 by Edmund Davy in Ireland through the reaction of water with potassium carbide (Fig. 1).1 Significant study only began after its rediscovery by the French chemist Marcellin Berthelot who coined the name “acetylene” in 1860 and reported its synthesis at high temperatures using an electric arc in 1862.2 The first commercial calcium carbide (CaC2) plant for acetylene (C2H2) production was established in 1894, following the discovery by Thomas Willson of CaC2 production through the electrothermal reduction of lime (CaO) with carbon sources and its simple reaction with water to form C2H2.3 Early C2H2 applications were mainly in lighting and oxyacetylene welding and cutting due to the brightness and high temperature of its flame.2 The importance of C2H2 in the chemical industry stems from its triple bond, which makes the molecule reactive and offers rich chemical possibilities. Industrial acetaldehyde production via C2H2 hydration in 1916, along with studies on C2H2 chemistry by Walter Reppe to produce various commodity chemicals, positioned C2H2 as a pillar of the chemical industry.4 Coal-based CaC2 was the sole source for C2H2 production until the development of commercial light hydrocarbon cracking methods, including the electric arc (Hüls arc) process in 1940, and the partial combustion (PC) and Wulff thermal pyrolysis (TP) processes in 1950.5,6

The birth and rapid growth of the petrochemical industry in the USA during the 1940s and in Europe during the 1940s–1950s, coupled with the safer handling of olefins compared to C2H2acetylene, led to facilitated the development of more cost-effective steam cracking technologies for olefin production, gradually displacing the C2H2 market primarily with ethylene (C2H4).7,8 Quantitatively, global production of C2H2 and ethylene (C2H4) reached 3.0 and 3.5 million metric tons per year (Mt y–1) by 1960, respectively,9–11 and these figures rose to 11 Mt y–1 for C2H2 and a staggering 225 Mt y–1 for C2H4 by 2021.12–14 Between the 1970s and 1980s, PC became the dominant technology for global C2H2 production, as other methods were mostly abandoned. In recent years, China has seen a resurgence in CaC2 production, rising from ca. 3 Mt y–1 in 1998 to 28 Mt y–1 in 2020.15 This growth is linked to a significant increase in polyvinyl chloride (PVC) production, from less than 1.6 Mt y–1 in 1998 to over 20 Mt y–1 in 2020.16,17 China now leads global C2H2 production (>8 Mt y–1), accounting for over 70% of world production.13,14,18 The historical transition from acetylene to ethylene derived from was driven by the long-term availability of inexpensive oil before 1973 (Fig. 1).19,20 Nonetheless, as oil prices fluctuate, C2H2 production has become economically viable in certain regions where oil prices are high and electricity, coal, or natural gas costs are low.21 leading to the recent increase in C2H2 production in China. Additionally, the triple bond of C2H2 makes it unique for certain applications, such as the production of acetylene black, polyacetylene (which led to the 2000 Nobel Prize in Chemistry in the year 2000), and Reppe chemicals like 1,4-butanediol, which are difficult to replace with olefins.22–25

Acetylene production processes generally show a higher carbon footprint per kilogram compared to ethylene production when both are derived from fossil carbon sources.13,26–28 As the industry shifts towards sustainability, reducing the carbon footprint of acetylene production through the use of renewable energy, sustainable carbon resources, and new technologies presents an opportunity for its revitalization Developing more sustainable C2H2 production methods through new technologies, non-fossil carbon, and renewable energy offers a promising strategy to reduce its carbon footprint.13 However, research on alternative C2H2 acetylene production has lagged for decades, with most studies only focusing on C2H4 olefin production.29–31 By offering a comprehensive analysis of conventional C2H2 production methods, summarizing emerging technologies, and outlining pathways toward sustainable processes, As the industry shifts towards sustainability, reducing the carbon footprint of C2H2 industry presents an opportunity for its revitalization. In this review, we this perspective aims to rekindle interest and foster collaborative efforts in advancing green acetylene production while addressing provide a comprehensive overview of C2H2 production methods, sustainable initiatives, and the associated challenges and opportunities associated with this transition.

- Production of acetylene

2.1. Summary of acetylene production routes and its applications

Acetylene can be produced from fossil feedstocks such as coal, natural gas, and oil using four primary technologies, namely CaC2, partial combustion (PC), plasma, and thermal pyrolysis (TP) (Fig. 2).27 Among these, oil-based routes have never are yet to reached the commercial stage. Plasma technologies include thermal plasma, where all generated species reach thermal equilibrium, and non-thermal plasma, where energy is preferentially channeled to electrons.33 Thermal plasma, known as the electric arc, has extremely high energy consumption (12.1 kWh kg–1 C2H2),5,34 and was therefore were largely replaced by PC processes utilizing natural gas as feedstock due to their extremely high energy consumption.5,28 Plasma technologies for converting natural gas and coal to C2H2 continues at the pilot scale in China.13,35 Thermal pyrolysis (TP) of hydrocarbons was commercialized via the Wulff process;6, however, these which involved preheating bricks to 1300 °C followed by feeding hydrocarbons for cracking to produce C2H2.6 However plants eventually shut down due to a slow heating rate and significant soot formation compared to PC, Wulff plants have been shut down.9 Currently, coal-derived CaC2 and natural gas-derived PC technologies dominate C2H2 production. Notably, C2H2 production is region-dependent, with China producing over 8 Mt y–1 primarily from the CaC2 route (~90%) due to its large coal reserves, while other countries produce less than 3 Mt y–1, mainly via PC.9,13Additionally, C2H2 can be produced as an impurity in the steam cracking of hydrocarbons to olefins, typically removed by catalytic selective hydrogenation or solvent extraction.28

The first commercial application of C2H2 after 1894 was for lighting, which was later replaced by electric lighting. However, its use in oxyacetylene welding and cutting, introduced in 1903, remains widespread due to the flame temperature of approximately 3150 °C, making it one of the cheapest and most accessible methods for metalworking.22 The most significant major application of C2H2 is as a feedstock in the chemical industry,29 with key downstream products summarized in Fig. 2. Among all, Currently, major applications of C2H2 include the production of vinyl chloride, 1,4-butanediol, acetylene black, and vinyl acetate. vVinyl chloride has the highest production volume, reaching approximately 50 Mt y–1,30 predominantly used to produce PVC with a market size of USD 68 billion. While the Globally, C2H4-basedthe PVC market is route dominateds by the C2H4-based route globally, accounting for about 65% of PVC output.,31 However, in China, ca. 78% of PVC production in China uses relies on the C2H2 route, with the remainder using produced via the C2H4 route.21 The market for aAcetylene black, valued has the smallest market size of at approximately USD 0.3 billion, is expected to grow with the global shift towards electric vehicles .38 It is a high-purity and highly crystalline form of carbon black produced from the thermal decomposition of C2H2, makes it as due to its effectiveness as a conductive additive for electrodes.9,32Therefore, the market for acetylene black is expected to grow with the global shift towards electric vehicles.38

Another critical application of C2H2 is the production of 1,4-butanediol via the classical Reppe process, which involves the reaction of acetylene with formaldehyde. This process still dominates, accounting for 40% of total production (2–2.5 Mt y–1).39,40 Despite the advantages of the said route, s Although several complex and multi-step alternative routes not relying on C2H2 currently exist, with maleic anhydride or 1,4-diacetoxy-2-butene as the intermediate, the recent resurgence of the C2H2The production of 1,4-butanediol route has significantly lowered market prices. This route now accounts for over 55% of the world’s capacity and 40% of global production (2–2.5 Mt y–1), resulting in overcapacity of 1,4-butanediol.33,34 exemplifies the unique advantages of C2H2 as the feedstock in chemical synthesis. This highlights the distinct advantages of C2H2 as a feedstock, which typically enables more streamlined downstream synthesis due to its reactive C≡C triple bond. However, research on converting C2H2 to chemicals has not attracted sufficient attention since being largely replaced by olefins. The While the global industrial production of vinyl acetate, acetaldehyde, acetic acid, acrylic acid, and acrylonitrile from C2H2 has been largely replaced by shifted to routes utilizing C2H4, propylene, and methanol (Fig. 2),Acetaldehyde and vinyl acetate are primarily produced from C2H4 globally, with annual productions of 1.5 and 6.2 Mt, respectively. Notably, vinyl acetate production in China primarily relies on the C2H2 route.35-41 to date. Acrylic acid and acrylonitrile are produced from C3H6, with annual productions of 8.8 and 7.0 Mt, respectively.43–45 Acrylic acid has the largest annual global market size among the replaced commodity chemicals listed in Fig. 2,43 at approximately USD 23 billion, while acetaldehyde has the smallest, at around USD 1.9 billion. Acetic acid, previously produced via C2H2-derived acetaldehyde oxidation, is now mainly produced by methanol carbonylation, with a high production of 18 Mt y–1 and a global annual market size of about USD 15 billion.46,47

2.2.	Indirect Commercial acetylene production routes via calcium carbide

The CaC2 route involves two reaction steps, as shown below. In the CaC2 route, Unlike other methods, the C2H2 production is produced occurs at room temperature without thermodynamic limitations. However, the production of CaC2 is highly endothermic and occurs only at high temperatures through the reaction of carbon and lime.42 CaC2, also known as calcium acetylide, is a salt of the C22− anion. The CaC2 method allows for an easy reaction with water at room temperature to release the C22− anion as C2H2. The production process of C2H2 via the CaC2 route involves two reaction steps:

3C + CaO → CaC2 + CO                            ΔH = 465 kJ mol–1

CaC2 + 2H2O → C2H2 + Ca(OH)2           ΔH = –127 kJ mol–1

The formation of CaC2 is highly endothermic, while the conversion of CaC2 to C2H2 is exothermic.48 Due to Despite its process simplicity and high selectivity for C2H2, and the low equipment investment, the CaC2 route still plays a dominant role in C2H2 production. This C2H2 manufacturing process (Fig. 3a) has low equipment investment but several disadvantages exist (Fig. 3a):49 (i) the CaC2 formation reaction requires temperatures of 2000–2300 °C to overcome the high endothermicity, leading to the electricity consumption of ca. 3–3.5 kWh kg–1 CaC2 and approximately 10.6 kWh kg–1 C2H2;43 (ii) large amounts of coal is consumed, with one-third of carbon resources converted to CO;13 (iii) significant CO2 emissions of 3.6 kg result from the decomposition of limestone (CaCO3) and the combustion of byproduct CO to provide heat for the arc furnace;13 (iv) large amounts of solid slag waste, mostly Ca(OH)2, are produced.13 Due to these disadvantages and the transition from C2H2 to olefins, most CaC2 plants in Western countries have shut down. However, in China, where coal reserves are abundant and imported oil for olefin production is costly, the CaC2 route remains significant, reflected by significant increase in CaC2 production from 3 to 28 Mt y–1. After rapid growth in the 21st century (Fig. 3b),15 CaC2 production reached approximately 28 Mt y–1, accounting for more than 96% of the global production.15,44

In the CaC2 route (Fig. 3a, 3c), producing 1 kg of C2H2 consumes approximately 3.1 kg of 80% pure CaC2 derived from 2 kg of coal, while generating 2.8 kg of Ca(OH)2 slag waste and 3.6 kg of CO2.9,13 Electricity consumption is primarily associated with the CaC2 production process, requiring about 3 to 3.5 kWh kg–1 CaC2,51 and a total electricity consumption of about 10.6 kWh kg–1 C2H2.11 Several process improvements have been proposed over the years, such as reusing Ca(OH)2 slag waste as building materials or for lime production via dehydration,18,52,53 CO2 capture by Ca(OH)2 to form limestone through mineralization,48 CO recovery,9 the use of renewable electricity,54 and the use of biochar to replace coal.14,53,55,56 These methods could reduce slag waste, CO2 emissions, and coal consumption to some extent (Fig. 3c). Nevertheless, high electricity consumption remains inevitable due to the endothermic nature of the CaC2 formation reaction, so CaC2 plants are strategically placed in regions with cheap electricity. An oxythermal method, which heats a mixture of CaO, coke, and O2, can produce CaC2 with a lower energy requirement.48 However, significant energy consumption is still necessary, often supplied by the combustion of coke. Other metal carbides (e.g., BaC2 and Li2C2) can be produced at relatively lower reaction temperatures, but they are yet to be commercialized due to more complex reaction pathways.49,57

2.3.	Direct commercial acetylene production routes

Direct conversion of CH4 to C2 hydrocarbons (C2H2, C2H4, C2H6) is highly endothermic, with the CH4 to C2H2 reaction being the most endothermic, having a standard enthalpy of 376 kJ mol–1.27 This corresponds to the theoretical limit for the energy requirement of 4.0 kWh kg–1 C2H2.45

2CH4 → C2H2 + 3H2                     ΔH = 376 kJ mol–1

2CH4 → C2H4 + 2H2                     ΔH = 202 kJ mol–1

2CH4 → C2H6 + H2                        ΔH = 65 kJ mol–1

For the CH4 to C2H2 process, the theoretical limit for the energy requirement is 4.0 kWh kg–1 C2H2.58,59 Thermodynamic data show that CH4 is the most stable hydrocarbon at low temperatures, while C2H2 is the least stable (Fig. 3b).27 With increasing temperature, the free energy of C2H2 decreases, making its formation more favorable than other hydrocarbons above 1230 °C.27,46 As seen in Fig. 4b, CH4 conversion to C2H2 and C2H4 is thermodynamically limited at 600 °C, but these limitations can be alleviated at 1200 °C.30 Although C2H2 production becomes more thermodynamically favorable However, at high temperatures, these conditions also promote C2H2 decomposition to carbon and hydrogen. Thus, achieving reasonable yields from CH4 requires high reaction temperatures and rapid quenching of the C2H2 stream, such as in .Two primary commercial technologies for direct C2H2 production are the PC and thermal plasma technologies. These technologies have explored various carbon resources, including crude oil, naphtha, and coal, but natural gas remains the only feedstock currently used industrially. For PC, The reactor concepts are illustrated in Fig. 4c. In the PC process, natural gas and O2 are preheated separately (to 600 °C when CH4 is used) before mixing in the diffuser and passing through the burner and reaction chamber.5 approximately 60% of the CH4 combusts with O2 in the burner to stabilize the flame at a temperature above 1500 °C, facilitating the conversion of the remaining CH4.27 The reaction gas exits the reactor within milliseconds, followed by rapid quenching with water or oil sprays. This process results in a C2H2 concentration of around 8 vol.% in the cracked gas.32 Thermal plasma, in contrast, uses known as the an electric arc for direct heating,47 achieves temperatures of up to 20,000 °C at the arc center, while the electrode walls remain around 600 °C.5 A vortex chamber stabilizes the arc between the cathode and anode by injecting gas tangentially. Fig. 4c highlights the critical role of reactor and process design in the industrialization of C2H2 production via PC and thermal plasma. Fig. 3c compares these two technologies.5,9,13,27 The thermal plasma method yields a higher C2H2 concentration in the cracked gas, lower natural gas consumption, and reduced direct CO2 emissions, but it . Despite these advantages, thermal plasma has requires significantly higher electricity consumption (12.1 kWh kg–1 C2H2) and leads to extensive soot formation.5,48 Another type of thermal plasma plant, operated by DuPont between 1963 and 1968, showed a higher C2H2 yield and lower electricity consumption of 8.8 kWh kg–1 C2H2 using a rotating arc.63 However, the equipment design was more complex, with decreased stability, and costly H2 was co-fed as the plasma gas.63 With the increasing availability of green electricity, thermal plasma has garnered more attention.13,21,49

2.3.	Emerging direct acetylene synthesis routes and catalytic applications Emerging C2H2 synthesis routes via the plasma process

Coal-to-C2H2 production via thermal plasma technologies has seen significant development in China recently, driven by high carbon footprint associated with the CaC2 route. Since 2014, Zhejiang University and Xinjiang Yuehetai Chemical Technology Co. Ltd. have collaborated to develop 2 MW and 10 MW H2 plasma systems for C2H2 production, claiming that the costs are over 20% lower than conventional CaC2 routes, with a significantly reduced carbon footprint.50,51 In contrast to thermal plasma, Recent advancements have explored non-thermal plasma technologies, including electrode-based plasma (dielectric barrier discharge (DBD), corona, pulsed, and spark) and electrodeless microwave plasma for C2H2 synthesis from CH4 (Fig. 5a).33 N non-thermal plasmas preferentially channel energy to electrons, creating a much higher electron temperature than that of ions and neutrals. When the neutral temperature is close to room temperature, the plasma is termed “cold plasma” (e.g., dielectric barrier discharge (DBD), corona, and pulsed plasma). In contrast, microwave and spark plasmas can heat neutrals to several thousand degrees Celsius, making thermal activation significant. These are known as “warm plasmas”.52 The main advantage of cold plasma’s main advantage is that it eliminates the need for quenching, as C2H2 is produced at low temperatures. In contrast, warm plasma typically achieves higher CH4 conversion due to the higher neutral temperatures (Supplementary Table 1). Feedstocks typically include pure CH4 or CH4 diluted in O2,53,54 H2,55 and CO2.56Electrode-based plasmas require electrodes for gas ionization to form plasma condition, while microwave plasma induces ionization via microwave irradiation, avoiding soot deposition issues on electrodes that affect plasma stability.5

Reactor designs for non-thermal plasma technologies vary widely (Fig. 4a), necessitating case-by-case considerations to optimize energy efficiency. Generally, electrode-based plasmas require electrodes for gas ionization to form plasma conditions, while microwave plasma induces ionization via microwave irradiation, thus avoiding soot deposition on electrodes, which can affect plasma stability.5 For example, DBD uses a dielectric barrier to prevent arcing and ensure uniform plasma discharge, while corona reactors rely on a pointed electrode to generate a localized, non-uniform electric field for plasma formation (Fig. 5a). In addition to reactor design, catalysts also play an important role in non-thermal plasma, with DBD and microwave being the most commonly used (Supplementary Table 1). For instance, in DBD plasma, adding the MFM-300(Fe) catalyst increased C2H2 selectivity from 17% to 38% at a CH4 conversion of <3%.57 In contrast, in microwave plasma, microwave irradiation seems more essential for C2H2 production than catalysts in most cases.58,59 While extensive literature emphasizes the significance of non-thermal plasma in C2H2 production, detailed energy requirements are sometimes not provided, an essential indicator for evaluating relevant technologies. For non-thermal plasma, cold plasma’s main advantage is eliminating the need for quenching since C2H2 is produced at low temperatures. In contrast, warm plasma typically achieves higher CH4 conversion due to higher neutral temperatures (Supplementary Table 1). Feedstocks typically include pure CH4 or CH4 diluted in inert gas, with some attempts using co-feeds such as O2,67,68 H2,69 and CO2.70 However The A possible reason is that, non-thermal plasmas it requires substantial energy input, often similar to or even exceeding thermal plasma processes (Fig. 4d).Moreover, promising non-thermal plasma results (Supplementary Table 1) mostly derive from microwave or pulsed plasma, which typically operate at low pressures or high frequencies, requiring complex equipment design for durable operation.33

2.4.	Advancements in catalyst design for the thermal pyrolysis process

The thermal pyrolysis (TP) of CH4 in reactor tubes, heated either by electricity or combustion gas, can produce C2H2. However, this method often results in low conversion and significant C2H2 decomposition due to slow feed heating compared to PC. Adding H2 can shift the equilibrium to inhibit C2H2 decomposition, but it also reduces the CH4 conversion rate, making this approach commercially unviable.32 Advanced reactor designs, including membrane reactors (Fig. 5b) or millisecond reactors, have been explored to enhance CH4 conversion by shifting the reaction equilibrium toward the product side or reducing soot formation due to shortened contact time.65,66 These reactor developments underscore the importance of reactor design in non-thermal plasma and TP technologies.

The comparison of representative results from thermal plasma, (catalytic) non-thermal plasma, and catalytic TP processes is summarized in Fig. 5c (Supplementary Table 1 and 2). Overall performance, based on CH4 conversion and C2H2 selectivity, indicates that thermal plasma outperforms non-thermal plasma, which, in turn, outperforms TP processes (Fig. 5c).

For instance, in DBD plasma, adding the MFM-300(Fe) catalyst increased C2H2 selectivity from 17% to 38% at CH4 conversion of <3%.71 In contrast, in microwave plasma, catalysts appear to have limited effect, with microwave irradiation being essential for C2H2 production. For example, microwave irradiation increased C2H2 selectivity from 0% to 35% at 700 °C with around 10% CH4 conversion over a Mo/ZSM-5@SiC catalyst.72 However, similar C2H2 selectivity was obtained under microwave irradiation with (33.7%) and without (38.4%) a Ni powder catalyst at 31% CH4 conversion.73 It is essential to consider thermal activation in warm plasma, as the role of high neutral temperatures cannot be overlooked. While extensive literature stresses the significance of non-thermal plasma on C2H2 production, detailed energy requirements are sometimes not provided, an essential indicator to evaluate the relevant technology.

For thermal pyrolysis (TP), the oxidative coupling of methane (OCM) and non-oxidative coupling of methane (NCM) can produce C2 species, including C2H2, C2H4, and C2H6.30 However, the slower heat transfer in TP, compared to PC limits its commercial viability.27 Therefore, TP could only be competitive if reaction temperatures are reduced (<1100 °C) using catalysts. OCM is thermodynamically more favorable than NCM, requiring as it requires lower reaction temperatures (450–1000 °C), but it is faces challenging challenges to avoid in preventing C2H2 conversion or overoxidation to COx in the presence of O2.60 Additionally, at low temperatures, the reaction of CH4 to C2H2 becomes less thermodynamically favorable compared to the formation of other hydrocarbons (Fig. 3b). As a result, C2H2 production from catalytic TP primarily comes from NCM rather than OCM (Supplementary Table 2). Advanced reactor designs, including membrane (Fig. 4b) or millisecond reactors, have been explored for NCM to enhance CH4 conversion by shifting the reaction equilibrium toward the product side or reducing soot formation due to shortened contact time.61,62

The highest C2H2 selectivity reported is 35% at with approximately ca. 14% CH4 conversion at 975 °C on a Pt1/CeO2 (Pt1: Pt single atom) catalyst,63 outperforming pristine CeO2, Ptn/CeO2 (Ptn: Pt nanoparticles). However, recent studies have revealed that Pt1 in Pt1/CeO2 tends to sinter under reaction conditions, leading to the proposal of a Ce-Pt alloy or Ptn as the potential active site and the obtained acetylene selectivity is relatively low.64–66 Metal carbides, Fe1 and Fen catalysts (Fig. 5e) also show good promise,79–81 though CH4 conversion is typically below 30% at temperatures <1100 °C. A CH4 conversion of 63% is achieved with Pt on -Al2O3 monolith (Fig. 4c), but a significant temperature gradient in the monolith from 200 to 1500 °C is observed, which is already close to the temperature in PC.67 Metal carbides and Fe1 catalysts (Fig. 4e) also show good promise,68,69 though CH4 conversion is typically below 30% at temperatures under 1100 °C. Additionally, coke deposition and soot formation are significant in catalytic TP, although millisecond reactor designs can partially address this issue.62 To date, achieving both high CH4 conversion and high C2H2 selectivity simultaneously via catalytic TP remains challenging, with performance significantly worse than plasma processes (Fig. 4c, Supplementary Table 1 and 2). C2 products, especially C2H4 (Fig. 5e, Supplementary Table 2), are always present in large amounts, significantly affecting C2H2 selectivity. The presence of byproducts, such as other C2, C3-C5 hydrocarbons, aromatics, and coke/soot, limits C2H2 formation.69,70 Therefore, converting CH4 into a broader range of products, such as C2 or even C2+ hydrocarbons, may be a more practical route for future CH4 utilization via catalytic TP. In contrast, cold and warm plasma routes can overcome thermodynamic limitations and achieve a higher yield of C2H2, though they usually require significantly higher energy input. Overall, advancing these emerging acetylene synthesis methods will require a focus on reactor design, catalyst development, and deeper understanding of reaction mechanisms.

- The drive for acetylene’s revival

The primary application of acetylene is in the production of vinyl chloride monomer (VCM) and subsequently PVC, a process that accounts for approximately 35% of the global PVC production (Fig. 2). A production cost analysis (Fig. 5a), based on current data (refer to the Supplementary Notes for a detailed methodology), shows that the coal-based acetylene to VCM (ATV) scenario has a total cost of 0.6 USD kg–1 of VCM, making it more economical than the fossil-based (naphtha) ethylene balanced process (EBP) showing a total cost of 1.1 USD kg–1. However, the climate change impacts (Fig. 5b) associated with ATV are notably higher than those of the EBP route (5.7 vs. 2.4 kg CO2-eq kg–1, assuming the same feedstock alternatives, respectively), primarily due to the more energy-intensive production of C2H2. This economic and environmental trade-off has led to the coexistence of both ATV and EBP routes in the market. Supplementary Tables 3–5 provide the mass and energy balance, Supplementary Table 4 displays the life cycle inventories used for the environmental assessment, and Supplementary Table 5 lists the prices of feedstock and utility prices, respectively, which were used to estimate the production costs and climate change impacts of VCM. production routes. A more detailed techno-economic and environmental assessment of various acetylene production technologies is provided in Section 5.

As the chemical industry moves towards sustainability, both ATV and EBP routes must explore more environmentally friendly alternatives, particularly for the ATV route. By utilizing biochar-based C2H2 and C2H4 derived from the dehydration of bioethanol, a green scenario could emerge. The latter is already being deployed in countries like Brazil, using sugarcane as a sustainable feedstock. Interestingly, the cost of the biochar-based ATV route only slightly increases to 0.8 USD kg–1 when considering poplar as biomass source, while the cost of bioethanol-based EBP process rises significantly to 2.3 USD kg–1 (Fig. 5a), considering corn stover as feedstock. Although the climate change impacts in the green scenario remain higher for the ATV compared to the EBP route (1.7 vs. 0.8 kg CO2-eq kg–1), both routes show a significant reduction in carbon footprint simply by replacing the feedstocks. Notably, compared to the fossil-based EBP route, the biochar-based ATV route exhibits lower costs and reduced climate change impacts. Therefore, developing green acetylene production methods could drive a transition in PVC production from EBP to ATV routes, potentially revitalizing the acetylene industry. Furthermore, with PVC demand projected to grow at an annual rate of 3.8% until 2031 (data from Statista), the sustainable ATV route could gain momentum, particularly if the conventional EBP route does not achieve significant breakthroughs. This shift Acetylene’s market share could also further enhance expand acetylene’s market share by expanding its use through increased utilization in producing 1,4-butanediol, acetylene black, polyacetylene, and vinyl acetate, as well as by reclaiming its role in manufacturing products like (Fig. 2). Additionally, past acetylene applications, such as the production of acetaldehyde, acetic acid, acrylic acid, and acrylonitrile (Fig. 2). , which have been replaced by other methods, might also see a revival. Furthermore, Recent advancements in acetylene-based technologies further highlight its industrial relevance, such as the electrocatalytic conversion of C2H2 to butadiene or C2H4,71,72,73 the photocatalytic hydrochlorination of C2H2 to VCM,74 and the coupling of C2H2 with CO to synthesize esters and acids.75,76

Supplementary Table 3 presents the mass and energy balance, Supplementary Table 4 displays the life cycle inventories used for the environmental assessment, and Supplementary Table 5 lists the prices of feedstock and utilities used to estimate production costs for two VCM production routes. A more detailed techno-economic and environmental assessment of various acetylene production technologies is provided in Section 5.

- Renewable carbon resources for acetylene synthesis

Given the performance of biochar-based C2H2 in the ATV route (Fig. 5a, 5b), it is essential to explore sustainable C2H2 production methods using renewable resources (e.g., biochar, biomethane, CO2, plastic) and innovative production technologies to replace traditional fossil-based approaches (Fig. 5c). Additionally, integrating renewable electricity (e.g., wind and solar) into these processes is crucial for further advancing sustainability. Fig. 5d illustrates potential combinations of renewable carbon resources and relevant technologies for C2H2 synthesis. Biochar, derived from biomass pyrolysis, has been proposed for C2H2 synthesis via the CaC2 route with a lower carbon footprint. However, it typically exhibits lower carbon and energy efficiency compared to the coal-based CaC2 route due to the lower energy density of biomass and the generation of volatile components during pyrolysis.14,77 Direct TP of various biomass feedstocks to C2H2 has been widely studied but is limited by the selectivity towards C2H2.47 Recently, the coal-based plasma route regained attention in China as a greener alternative to the coal-based CaC2 route.21 Biomass as a substitute for coal in the plasma route has also been explored, such as lignin and hemicellulose cracking in a hydrogen plasma, further reducing the carbon footprint.78

The production of C2H2 from polyolefins and polyethylene terephthalate (PET) via rotating arc plasma has proven feasible for C2H2 production, achieving full carbon conversion and C2H2 selectivity of 55% for PET and 80% for polyolefins.79,80 However, the specific energy requirement is approximately 20 kWh kg–1 C2H2, even higher than the Hüls arc plasma process. Biogas, a renewable carbon feedstock composed mainly of CH4 and CO2, could replace natural gas for C2H2 synthesis with a lower carbon footprint. For instance, the conversion of biogas-like mixtures of CH4 and CO2 into C2 products can be achieved using DBD cold plasma, with C2H6 as the dominant product.81 Moreover, the addition of a catalyst (Ni–K/Al2O3) and increasing an increase in the reaction temperature above 220 °C can enhance C2H2 selectivity from biogas.82 Direct conversion of biogas via TP and PC is challenging due to the presence of CO2, despite CH4 being a common feedstock for C2H2 production. However, t The abundant availability of CH4, or its presence as a byproduct of CO2 utilization in biogas, presents an opportunity to utilize biomethane for C2H2 production using those technologies like plasma, TP, PC (Fig. 5d).29

CO2 in biogas or directly from the air can be integrated into the C2H2 production process via CO2-CO or CO2-C2H2 cycles. A sustainable route for C2H2 production, coupled with CO2 capture and conversion to CO (CO2-CO cycle), is based on the BaCO3-BaC2-Ba(OH)2-BaCO3 cycle, has been reported (Fig. 5e).83 Unlike the CaC2 route, where CaO is obtained from CaCO3 decomposition, leading to CO2 emissions, the CO2-CO cycle does not directly emit CO2. Additionally, BaC2 synthesis requires lower temperatures, and less energy input compared to CaC2. Notably, the carbon from CO2 and half of the carbon from coal or biochar converts to CO instead of C2H2 in the CO2-CO cycle (Fig. 5e), lowering atom economy if CO is not effectively utilized. In contrast, the proposed CO2-C2H2 concept cycle directly transforms CO2 into C2H2 via a cyclic electrochemical-assisted strategy (Fig. 5f).84 This CO2-mediated cycle involves three steps that hold: (i) LiOH electrolysis to produce metallic Li0 with H2O and O2; (ii) the active Li0 reacting with CO2 to form Li2C2 with Li2O or LiCO3 as byproducts; and (iii) hydrolysis of Li2C2 to form C2H2 and regenerate LiOH. CO2 is used as the sole carbon input, presenting a totally green route. However, the recycling of Li2O or LiCO3 byproducts was not addressed. The CO2-mediated concept cycle for C2H2 production shows the most significant promise for sustainability,. However, it is still in the early stages, as the recycling of Li2O or LiCO3 is yet to be addressed.

In the context of renewable carbon feedstock, projections suggest that by 2050, the annual availability of biochar and biomethane will reach 0.3–0.5 Gt and 0.1 Gt, respectively.85,86 This implies that only 4–7% of the available biochar (via the CaC2 process) or 26% of the biomethane (via the PC route, and significantly less via plasma routes) would be sufficient to meet current global C2H2 demands, estimated at about 11 Mt y–1. Given the current foundation of coal- and natural gas-derived technologies Therefore, incorporating biochar or biomethane as potential alternatives in those mature technologies green acetylene production holds significant promise at this stage.

- Environmental and economic assessment of acetylene synthesis routes

Recently, a In light of the framework was proposed to guide the development of sustainable chemical processes by prioritizing the environmental, economic, and equity (EEE) pillars,87 To simplify the model, this section evaluates the environmental impact and economic viability of various acetylene synthesis technologies through techno-economic and life cycle assessment (LCA) analyses. We compare these technologies using both fossil and renewable carbon resources, assessing their environmental impacts within a future decarbonized scenario. For detailed insights into our methodology, along with the safety risks linked to each acetylene production route, please refer to the Supplementary Notes.

We specifically compare the climate change impacts shown in Fig. 6a and production costs (excluding capital expenditure) in Fig. 6b, both associated with the production of one kilogram of C2H2 using three different processes: CaC2, PC, and plasma (both thermal and non-thermal). Supplementary Tables 6–8 present the mass and energy balance, Supplementary Table 7 displays the life cycle inventories used for the environmental assessment, and Supplementary Table 8 lists the prices of feedstock and utility prices, respectively, which were utilized ies used to estimate the production costs and climate change impacts for all of C2H2 production routes. We evaluated two scenarios for all production technologies: fossil-based and green. For the CaC2 process, we used coke (fossil) and biochar (green) as feedstock. For the technologies utilizing methane as a feedstock in the fossil scenario (PC and plasma routes), we replaced methane with biomethane in the green scenario. Furthermore, we conducted a prospective life cycle assessment (LCA) to evaluate the projected climate change impacts for the year 2050 under a decarbonized scenario.

The highest climate change impacts are observed in the CaC2 production scenario (12.9 kg CO2-eq kg–1) due to its heavy reliance on coke and electricity to produce calcium carbide, as well as substantial direct emissions. In contrast, the PC route has the lowest emissions (7.8 kg CO2-eq kg–1), utilizing methane as the primary feedstock and resulting in much lower direct emissions compared to the CaC2 scenario. Both plasma technologies have exhibit higher climate change impacts than the PC route, primarily due to the high electricity requirements of these pathways. However, emissions are significantly reduced across all green scenarios. Notably, the PC route displays a negative climate change impact of 
–6.1 kg CO2-eq kg–1 due to the utilization of biomethane feedstock, which results in negative net contributions to the overall impacts (on a cradle-to-gate basis) due to the biogenic nature of the carbon.88 Similarly, the impacts for the green CaC2 route show a 75% decrease reduction due to the shift from coke to carbon-negative biochar.89 In contrast to the significant reductions observed in the CaC2 and PC routes, both plasma technologies show only a 45% decrease under the green scenario, due to their heavy reliance on grid electricity, which is yet to be decarbonized.

In a future decarbonized scenario projected for 2050, emissions are expected to decrease significantly, primarily due to the adoption of a decarbonized electricity mix.90 The most optimistic scenario (1.5 °C – RCP 1.9) is highlighted in Fig. 6a (purple scatter).91 The CaC2 route demonstrates a 63% reduction in emissions compared to current impacts under the fossil scenario, and a 283% reduction under the green scenario. Similar reductions are observed across all production routes. These results underscore the potential of a decarbonized grid to mitigate climate change impacts, even in fossil-based production methods, and especially in the green scenario, as biochar also uses decarbonized electricity compared to the current scenario.

Regarding production costs within the fossil scenarios, the CaC2 route has the lowest cost (1.2 USD kg–1), primarily driven by electricity. The PC route is more expensive 
(2.0 USD kg–1), largely due to methane costs. Similarly, both plasma routes have similar comparable costs, averaging 1.7 USD kg–1, predominantly due to electricity. In the green scenario, the CaC2 route remains the most economical, with a 32% increase in costs due to the slightly higher cost of biochar compared to coke. The green PC scenario is the most expensive route, with a 340% increase in costs, driven by the substantial cost of biomethane (five times more expensive than methane). Similarly, plasma routes show at least a 110% increase in costs.

In the short term, the focus should be on optimizing the existing CaC2 production process by transitioning from coke to a blend of coke and biochar, ensuring stable biochar supply chains of biochar to maintain cost-effectiveness. The lower costs of biochar and its excellent significant reduction in climate change impacts make the green CaC2 route the most favorable option for producing C2H2. In the long term, transitioning to renewable sources and engaging in grid decarbonization initiatives for electricity will further reduce the climate change impacts of plasma-based C2H2 production at lower costs. Therefore, Moreover, lowering energy consumption in plasma processes will be key in the future. comprehensive analysis is essential for engaging with stakeholders and advocating for policies that steer the chemical industry toward sustainable and economically viable C2H2 production.

- Perspective and o Outlook

Transitioning from a fossil-based to a sustainable chemical industry is a global trend imperative that presents an opportunity to revitalize the acetylene industry. To achieve this, increased focus on both C2H2 production and its chemistry is essential. For production, we consider two main strategies are considered: integrating mature technologies with renewable carbon sources and decarbonized electricity to reduce the carbon footprint, and exploring emerging technologies to improve C2H2 yield and energy efficiency. Integrating renewable carbon feedstocks (biochar or biomethane) and decarbonized electricity with mature technologies can efficiently reduce the carbon footprint, but it also increases Replacing natural gas with more expensive biomethane is not suitable for the PC route, as it requires nearly three times more biomethane than plasma technologies. Consequently, biochar-based CaC2 and biomethane-based plasma routes are expected to become more prominent. Their development will depend on the availability and cost of biochar and biomethane, with competitive uses emerging (e.g., biochar for carbon removal and biomethane for defossilizing industrial heating). Given the explosive potential of C2H2, on-site generation for downstream applications is advantageous.27 The CaC2 route offers a significant advantage by eliminating the risk of explosion through the transport of solid CaC2 instead of C2H2, positioning the biochar-based CaC2 route favorably for the future.

Emerging technologies, such as non-thermal plasma and thermal pyrolysis (TP), face commercialization challenges, requiring advancements in reactor design, catalyst development, and a deeper understanding of reaction mechanisms. Non-thermal plasma technologies It demonstrates performance close to commercial thermal plasma but typically requires similar or even higher energy input and more complex equipment. While catalysts can favor C2H2 production, their use does not appear to reduce energy requirements, and the design of catalytic plasma reactors remains complex and insufficiently explored catalytic and plasma-assisted mechanisms are still not well understood. Therefore, future research should aim to improve C2H2 yield, energy efficiency, and equipment lifespan, with the goal of reducing energy requirements to the theoretical limit of 4.0 kWh kg–1 C2H2.

Cold plasma offers advantages by preventing C2H2 decomposition and eliminating costly cooling processes, while warm plasma usually achieves higher CH4 conversion. The future dominance of either plasma type remains uncertain. Unlike non-thermal plasma, C2H2 yield through TP remains below 10% (Fig. 4c) due to thermodynamic limitations at low temperatures. To improve activity, selectivity, and catalyst stability, advanced catalyst designs (e.g., single-atom Pt1 or Fe1) and reactor designs (e.g., membrane or millisecond reactors) were have been developed. Despite decades of effort, C2H2 selectivity remains below 40% due to the formation of other hydrocarbons catalytic TP is not yet commercially viable and .Therefore, catalytic TP may be is therefore more practical for selective CH4 conversion to C2 products or mixtures of C2 and other hydrocarbons. Additionally, kinetic studies and advanced operando characterization will be valuable in elucidating the complex CH4 reaction mechanisms, likely involving coupled surface and gas-phase chemistry. Designing high-temperature reactors that operate up to 1100 °C for synchrotron-based spectroscopy studies, especially under low-pressure conditions, will present a significant challenge in the future. Electrocatalytic and photocatalytic CH4 coupling has been widely studied, but the dominant products are C2H6 or C2H4 instead of C2H2.94–96 Understanding why C2H2 production is inhibited in these novel processes is critical. Plasma-assisted plastic upgrading to C2H2 is promising but currently requires significant energy input (20 kWh kg–1 C2H2), making it impractical without further developmentbreakthrough.79,80 Similarly, transforming CO2 and H2O into C2H2 and O2 via the CO2-C2H2 concept cycle is an ideal but nascent approach.84

Revitalizing the C2H2 industry also requires a substantial market for C2H2 applications. As discussed in Section 3, t The development of sustainable C2H2 production could prompt a shift in VCM production from ethylene to acetylene routes, based on the driven by environmental and economic perspectives assessments. Moreover, the longstanding challenge in C2H2-derived VCM production– regarding specifically the use of mercury catalysts–has been addressed due to the recent commercialization of an Au/C catalyst and the development of advanced single-atom catalysts for C2H2 hydrochlorination.92,93 The anticipated growth in the PVC market, along with the potential shift from ethylene- to acetylene-based production, exemplifies the possible resurgence of the acetylene industry. Since C2H2 chemistry was only extensively studied by Reppe in the mid-20th century, there remains significant untapped potential beyond VCM production and other conventional applications. Emerging technologies, such as electrocatalysis and photocatalysis, along with novel acetylene chemistry (e.g., its coupling with CO or CO2), warrant greater attention. Overall, we believe the expansion of the downstream market, combined with advancements in green acetylene production, could serve as a catalyst for the revitalization of the C2H2 industry. Since C2H2 chemistry was only extensively studied by Reppe in the mid-20th century, this revival might also reawaken the previously overlooked research on converting C2H2 into bulk chemicals.

Acknowledgements

This publication was created as part of NCCR Catalysis (grant number 180544), a National Centre of Competence in Research funded by the Swiss National Science Foundation.

Author contributions

Z.Z. and J.P.-R. conceived the review perspective. Z.Z. and A.N. produced figures. A.N. and G.G.G. completed the life cycle assessment and techno-economic study. Z.Z. and A.N. co-wrote the manuscript. G.G.G. and J.P.-R. revised the manuscript. J.P.-R. supervised the project. All authors reviewed, edited and enriched each section of the reviewperspective.

Competing interests

The authors declare no competing interests.

References

- Davy, E. Notice of carburet of potassium, and of a new gaseous bi-carburet of hydrogen. J. Frankl. Inst. 19, 216 (1837).

- Rasmussen, S. C. Acetylene and Its Polymers: 150+ Years of History. (Springer International Publishing, 2018).

- Carolina, N. Discovery of the Commercial Processes for Making Calcium Carbide and Acetylene. (American Chemical Society, 1998).

- Trotuş, I. T., Zimmermann, T. & Schüth, F. Catalytic reactions of acetylene: a feedstock for the chemical industry revisited. Chem. Rev. 114, 1761–1782 (2014). Systematic review of catalytic C2H2 applications.

- Jiang, C. & Hu, J. Conversion of Methane to Acetylene. 93 - 116 (Royal Society of Chemistry, 2023). The most comprehensive book chapter on CH4-to-C2H2 routes, covering calcium carbide, partial combustion, hot plasma, warm/cold plasma, and thermal pyrolysis.

- Bixler, G. H. & Coberly, C. W. Wulff process acetylene. Ind. Eng. Chem. 45, 2596–2606 (1953).

- Weissermel, K. & Arpe, H.-J. Industrial Organic Chemistry. (Wiley, 2008).

- Crafts, J. M. A lecture upon acetylene. Science 3, 377–392 (1896).

- Schobert, H. Production of acetylene and acetylene-based chemicals from coal. Chem. Rev. 114, 1743–1760 (2014). Systematic review of coal-based C2H2 synthesis, encompassing commercial calcium carbide route and all related attempts.

- Crynes, B. L., Albright, L. F. & Tan, L. F. Thermal Cracking. in Encyclopedia of Physical Science and Technology. (3rd ed.) 613–626 (Academic Press, 2003).

- Shlyapin, D. A., Afonasenko, T. N., Glyzdova, D. V., Leont’eva, N. N. & Lavrenov, A. V. Acetylene production technologies in the 21st century: main trends of their development in the paradigm of low-carbon economy of the future. Catal. Ind. 14, 251–267 (2022). Prediction on potential transition of carbide and partial combustion routes to plasma processes using renewable energy sources.

- Chauhan, R., Sartape, R., Minocha, N., Goyal, I. & Singh, M. R. Advancements in environmentally sustainable technologies for ethylene production. Energy Fuels 37, 12589–12622 (2023).

- Zhang, S. et al. Life cycle assessment of acetylene production from calcium carbide and methane in China. J. Clean. Prod. 322, 129055 (2021). Comparison of three commercial C2H2 synthesis routes—calcium carbide, partial combustion, and hot plasma—via life cycle assessment.

- Jiang, P. et al. Towards carbon neutrality of calcium carbide-based acetylene production with sustainable biomass resources. Green Energy Environ. 9, 1068–1078 (2024).

- Huo, H. et al. Case study of a novel low rank coal to calcium carbide process based on techno-economic assessment. Energy 228, 120566 (2021).

- Saeki, Y. & Emura, T. Technical progresses for PVC production. Prog. Polym. Sci. 27, 2055–2131 (2002).

- Jiang, M. et al. Tracing fossil-based plastics, chemicals and fertilizers production in China. Nat. Commun. 15, 3854 (2024).

- Liu, Y. et al. Life cycle assessment of beneficial use of calcium carbide sludge in cement clinker production: A case study in China. J. Clean. Prod. 418, 138003 (2023).

- Stournaras, Y. A. Are Oil Price Movements Perverse? A Critical Explanation of Oil Price Levels 1950–1985. (Oxford Institute for Energy Studies, 1985).

- Gadea, M. D., Gómez-Loscos, A. & Montañés, A. Oil price and economic growth: a long story? Econometrics 4, 41 (2016).

- Yan, B., Lu, W. & Cheng, Y. China goes green: cleaner production of chemicals. Green Process. Synth. 1, 33–47 (2012).

- Singh, R. P., Kumar, S., Dubey, S. & Singh, A. A review on working and applications of oxy-acetylene gas welding. Mater. Today Proc. 38, 34–39 (2021).

- Khodabakhshi, S., Fulvio, P. F. & Andreoli, E. Carbon black reborn: structure and chemistry for renewable energy harnessing. Carbon 162, 604–649 (2020).

- Zhu, Y., Yang, J., Mei, F., Li, X. & Zhao, C. Bio-based 1,4-butanediol and tetrahydrofuran synthesis: perspective. Green Chem. 24, 6450–6466 (2022).

- Chiang, C. K. et al. Electrical conductivity in doped polyacetylene. Phys. Rev. Lett. 39, 1098–1101 (1977).

- Zimmermann, H. & Walzl, R. Ethylene. in Ullmann’s Encyclopedia of Industrial Chemistry (Wiley, 2009).

27. Amghizar, I., N. Dedeyne, J., J. Brown, D., B. Marin, G. & Geem, K. M. V. Sustainable innovations in steam cracking: CO2 neutral olefin production. React. Chem. Eng. 5, 239–257 (2020).

28. Ren, T., Patel, M. & Blok, K. Olefins from conventional and heavy feedstocks: Energy use in steam cracking and alternative processes. Energy 31, 425–451 (2006).

29. Zhang, M. & Yu, Y. Dehydration of ethanol to ethylene. Ind. Eng. Chem. Res. 52, 9505–9514 (2013).

30. Gao, Y. et al. Recent advances in intensified ethylene production—a review. ACS Catal. 9, 8592–8621 (2019).

- Pässler, P. et al. Acetylene. in Ullmann’s Encyclopedia of Industrial Chemistry (Wiley, 2011). The most comprehensive introduction of C2H2, covering its physical and chemical properties, industrial applications, synthesis routes, safety precautions, transportation, and storage.

- Bilera, I. V. & Lebedev, Yu. A. Plasma-chemical production of acetylene from hydrocarbons: history and current status (a review). Pet. Chem. 62, 329–351 (2022).

- Bowker, M. et al. Advancing critical chemical processes for a sustainable future: challenges for industry and the Max Planck–Cardiff centre on the fundamentals of heterogeneous catalysis (FUNCAT). Angew. Chem. Int. Ed. 61, e202209016 (2022). The review pointing out the importance of C2H2 production derived from biomethane.

- Ma, H., Wang, Y., Qi, Y., Rout, K. R. & Chen, D. Critical review of catalysis for ethylene oxychlorination. ACS Catal. 10, 9299–9319 (2020).

- Faust Akl, D. et al. Reaction-induced formation of stable mononuclear Cu(I)Cl species on carbon for low-footprint vinyl chloride production. Adv. Mater. 35, 2211464 (2023).

- Cannon, S., Levine, S. & Ritter, A. Natural Gas Conversion into Clean Hydrogen and Carbon for Li-ion Battery Electrodes Using Renewable Energy (Penn collection, 2022).

- Satam, C. C. & Realff, M. J. Comparison of two routes for the bio-based production of economically important C4 streams. J. Adv. Manuf. Process. 2, e10054 (2020).

- De Bari, I. et al. From cardoon lignocellulosic biomass to bio-1,4 butanediol: an integrated biorefinery model. Processes 8, 1585 (2020).

- Zhang, N., Wang, S., Gibril, M. E. & Kong, F. The copolymer of polyvinyl acetate containing lignin-vinyl acetate monomer: Synthesis and characterization. Eur. Polym. J. 123, 109411 (2020).

- KK, S., Verma, R. & Kumar, L. Ethylene-Vinyl Acetate Foam. in Polymeric Foams: Fundamentals and Types of Foams 205–221 (American Chemical Society, 2023).

- Okoro, O. V., Nie, L., Alimoradi, H. & Shavandi, A. Waste apple pomace conversion to acrylic acid: economic and potential environmental impact assessments. Fermentation 8, 21 (2022).

- Zhang, J. et al. Advances in the catalytic production of acrylonitrile. Chem Catal. 4, 100825 (2024).

- Reesh, A. & M, I. Acrylonitrile process enhancement through waste minimization: effect of reaction conditions and degree of backmixing. Sustainability 13, 7923 (2021).

- Pal, P. & Nayak, J. Acetic acid production and purification: critical review towards process intensification. Sep. Purif. Rev. 46, 44–61 (2017).

- Sarchami, T., Batta, N. & Berruti, F. Production and separation of acetic acid from pyrolysis oil of lignocellulosic biomass: a review. Biofuels Bioprod. Bioref. 15, 1912–1937 (2021).

- Wang, H. et al. Carbon-calcium composite conversion of calcium carbide-acetylene system: on the imperative roles of carbon capture and solid waste recycling. Appl. Energy 327, 120139 (2022).

- Guo, J. & Zheng, D. Thermodynamic analysis of low-rank-coal-based oxygen-thermal acetylene manufacturing process system. Ind. Eng. Chem. Res. 51, 13414–13422 (2012).

- Ma, S. et al. Energy and exergy analysis of a new calcium carbide production process. Fuel Process. Technol. 226, 107070 (2022).

52. Gong, X. et al. Recycling and utilization of calcium carbide slag-current status and new opportunities. Renew. Sustain. Energy Rev. 159, 112133 (2022).

53. Pääkkönen, A., Tolvanen, H. & Kokko, L. The economics of renewable CaC2 and C2H2 production from biomass and CaO. Biomass Bioenergy 120, 40–48 (2019).

54. Huo, H. et al. Life cycle assessment and sustainable production evaluation of calcium carbide industry in China. J. Clean. Prod. 360, 132176 (2022).

55.Li, G. et al. Production of calcium carbide from fine biochars. Angew. Chem. Int. Ed. 49, 8480–8483 (2010). Systematic study of the biochar to CaC2 process.

56. Li, Z., Liu, Z., Wang, R., Guo, X. & Liu, Q. Conversion of bio-char to CaC2 at low temperatures-morphology and kinetics. Chem. Eng. Sci. 192, 516–525 (2018).

57.M. McEnaney, J. et al. A cyclic electrochemical strategy to produce acetylene from CO2, CH4, or alternative carbon sources. Sustain. Energy Fuels 4, 2752–2759 (2020). The CO2-C2H2 cycles.

58. Kang, H. et al. Methane to acetylene conversion by employing cost-effective low-temperature arc. Fuel Process. Technol. 148, 209–216 (2016).

- Radoiu, M. T., Chen, Y. & Depew, M. C. Catalytic conversion of methane to acetylene induced by microwave irradiation. Appl. Catal. B: Environ. 43, 187–193 (2003).

- Holmen, A., Olsvik, O. & Rokstad, O. A. Pyrolysis of natural gas: chemistry and process concepts. Fuel Process. Technol. 42, 249–267 (1995).

- Gyrdymova, Y., Lebedev, A., Du, Y.-J. & Rodygin, K. Production of acetylene from viable feedstock: promising recent approaches. ChemPlusChem, e202400247 (2024).

- Fincke, J. R. et al. Plasma thermal conversion of methane to acetylene. Plasma Chem. Plasma Process. 22, 105–136 (2002).

63.Anderson, R. P., Fincke, J. R. & Taylor, C. E. Conversion of natural gas to liquids via acetylene as an intermediate. Fuel 81, 909–925 (2002).

- Yan, B., Xu, P., Guo, C. Y., Jin, Y. & Cheng, Y. Experimental study on coal pyrolysis to acetylene in thermal plasma reactors. Chem. Eng. J. 207–208, 109–116 (2012).

- Xia, W. et al. The plasma energy route to industrial carbon neutrality in China. Chin. J. Theor. Appl. Mech. 55, 2779–2795 (2023).

- Guo, X. et al. Analysis and prospect on plasma pyrolysis technology for coal-to-acetylene production. Environ. Prot. Chem. Ind. 41, 229–234 (2021).

65.Sakbodin, M., Wu, Y., Oh, S. C., Wachsman, E. D. & Liu, D. Hydrogen-permeable tubular membrane reactor: promoting conversion and product selectivity for non-oxidative activation of methane over an Fe©SiO2 catalyst. Angew. Chem. Int. Ed. 55, 16149–16152 (2016).

66.Oh, S. C. et al. Direct non-oxidative methane conversion in a millisecond catalytic wall reactor. Angew. Chem. Int. Ed. 58, 7083–7086 (2019).

- Scapinello, M., Delikonstantis, E. & Stefanidis, G. D. The panorama of plasma-assisted non-oxidative methane reforming. Chem. Eng. Process. Process Intensif. 117, 120–140 (2017). The most comprehensive introduction of non-oxidative methane coupling via plasma methods.

- Kado, S., Sekine, Y. & Fujimoto, K. Direct synthesis of acetylene from methane by direct current pulse discharge. Chem. Commun. 24, 2485–2486 (1999).

- Han, Q. et al. Conversion of methane to C2 and C3 hydrocarbons over TiO2/ZSM-5 core–shell particles in an electric field. RSC Adv. 9, 34793–34803 (2019).

- Wnukowski, M., van de Steeg, A. W., Hrycak, B., Jasiński, M. & van Rooij, G. J. Influence of hydrogen addition on methane coupling in a moderate pressure microwave plasma. Fuel 288, 119674 (2021).

- Zhang, X., Dai, B., Zhu, A., Gong, W. & Liu, C. The simultaneous activation of methane and carbon dioxide to C2 hydrocarbons under pulse corona plasma over La2O3/γ-Al2O3 catalyst. Catal. Today 72, 223–227 (2002).

- Ma, Y. et al. Direct conversion of methane to ethylene and acetylene over an iron-based metal–organic framework. J. Am. Chem. Soc. 145, 20792–20800 (2023).

- Julian, I., Ramirez, H., Hueso, J. L., Mallada, R. & Santamaria, J. Non-oxidative methane conversion in microwave-assisted structured reactors. Chem. Eng. J. 377, 119764 (2019).

- Suib, S. L. & Zerger, R. P. A direct, continuous, low-power catalytic conversion of methane to higher hydrocarbons via microwave plasmas. J. Catal. 139, 383–391 (1993).

- Ortiz-Bravo, C. A., Chagas, C. A. & Toniolo, F. S. Oxidative coupling of methane (OCM): An overview of the challenges and opportunities for developing new technologies. J. Nat. Gas Sci. Eng. 96, 104254 (2021).

- Sakbodin, M., Wu, Y., Oh, S. C., Wachsman, E. D. & Liu, D. Hydrogen-permeable tubular membrane reactor: promoting conversion and product selectivity for non-oxidative activation of methane over an Fe©SiO2 catalyst. Angew. Chem. Int. Ed. 55, 16149–16152 (2016).

- Oh, S. C. et al. Direct non-oxidative methane conversion in a millisecond catalytic wall reactor. Angew. Chem. Int. Ed. 58, 7083–7086 (2019).

- Xie, P. et al. Nanoceria-supported single-atom platinum catalysts for direct methane conversion. ACS Catal. 8, 4044–4048 (2018). The highest C2H2 selectivity from non-oxidative methane coupling via catalytic thermal pyrolysis.

- Zhang, H. et al. Pt/CeO2 as catalyst for nonoxidative coupling of methane: oxidative regeneration. J. Phys. Chem. Lett. 14, 6778–6783 (2023).

- Talpade, A. D. et al. Catalytic reactivity of Pt sites for non-oxidative coupling of methane (NOCM). Chem. Eng. J. 481, 148675 (2024).

- Eggart, D. et al. Operando XAS Study of Pt-Doped CeO2 for the Nonoxidative Conversion of Methane. ACS Catal. 12, 3897–3908 (2022).

- Hohn, K. L., Witt, P. M., Davis, M. B. & Schmidt, L. D. Methane coupling to acetylene over Pt-coated monoliths at millisecond contact times. Catal. Lett. 54, 113–118 (1998).

- Y. Zhang, S. B. X. et al. Role and dynamics of transition metal carbides in methane coupling. Chem. Sci. 14, 5899–5905 (2023).

- Han, S. J., Lee, S. W., Kim, H. W., Kim, S. K. & Kim, Y. T. Nonoxidative direct conversion of methane on silica-based iron catalysts: effect of catalytic surface. ACS Catal. 9, 7984–7997 (2019).

81.Hao, J. et al. Enhanced methane conversion to olefins and aromatics by H-donor molecules under nonoxidative condition. ACS Catal. 9, 9045–9050 (2019).

- Guo, X. et al. Direct, nonoxidative conversion of methane to ethylene, aromatics, and hydrogen. Science 344, 616–619 (2014). The breakthrough of non-oxidative methane coupling.

- Teh, W.J. et al. Selective electroreduction of acetylene to 1,3-butadiene on iodide-induced Cuδ+–Cu0 sites. Nat. Catal. (2024). https://doi.org/10.1038/s41929-024-01250-0. The new acetylene application.

- Zhao, B.H. et al. Economically viable electrocatalytic ethylene production with high yield and selectivity. Nat. Sustain. 6(7), 827-837 (2023).

- Shi, R. et al. Room-temperature electrochemical acetylene reduction to ethylene with high conversion and selectivity. Nat. Catal. 4(7), 565-574 (2021).

- Zhao, Z.H. et al. Photocatalytic acetylene hydrochlorination by pairing proton reduction and chlorine oxidation over g-C3N4/BiOCl catalysts. J. Am. Chem. Soc. 146(43), 29441-29449 (2024).

- Wang, A., Cao, H., Zhang, L. & Wang, A. Co/SiO2 catalyst for methoxycarbonylation of acetylene: on catalytic performance and active species. Molecules 29(9), 1987 (2024).

- Li, X. et al. Palladium and ruthenium dual‐single‐atom sites on porous ionic polymers for acetylene dialkoxycarbonylation: synergetic effects stabilize the active site and increase CO adsorption. Angew. Chem. Int. Ed. 62(33), e202307570 (2023).

- Li, G. et al. Production of calcium carbide from fine biochars. Angew. Chem. Int. Ed. 49, 8480–8483 (2010). Systematic study of the biochar to CaC2 process.

- Dimitrakellis, P., Delikonstantis, E., Stefanidis, G. D. & Vlachos, D. G. Plasma technology for lignocellulosic biomass conversion toward an electrified biorefinery. Green Chem. 24, 2680–2721 (2022).

- Zhang, M. et al. Pyrolysis of polyolefins using rotating arc plasma technology for production of acetylene. Energies 10, 513 (2017).

- Zhang, M. et al. Gas production from polyethylene terephthalate using rotating arc plasma. Chem. Eng. Process. Process Intensif. 128, 257–262 (2018).

- Kolb, T., Kroker, T. & Gericke, K.-H. Conversion of biogas like mixtures to C2 hydrocarbon in a plug flow reactor supported by a DBD at atmospheric pressure. Vacuum 88, 144–148 (2013).

- Zeng, Y. et al. Biogas reforming for hydrogen-rich syngas production over a Ni–K/Al2O3 catalyst using a temperature-controlled plasma reactor. Int. J. Hydrog. Energy 48, 6192–6203 (2023).

- Li, M. et al. Reengineering of the carbon-to-acetylene process featuring negative carbon emission. Green Chem. 25, 8584–8592 (2023).

- M. McEnaney, J. et al. A cyclic electrochemical strategy to produce acetylene from CO2, CH4, or alternative carbon sources. Sustain. Energy Fuels 4, 2752–2759 (2020). The CO2-C2H2 cycles.

- Fawzy, S., Osman, A.I., Yang, H., Doran, J. & Rooney, D.W. Industrial biochar systems for atmospheric carbon removal: a review. Environ. Chem. Lett. Letters, 19, 3023-3055 (2021).

- Alberici, S., Grimme, W. & Toop, G. Biomethane Production Potentials in the EU - Feasibility of REPowerEU 2030 Targets, Production Potentials in the Member States and Outlook to 2050 - A Gas for Climate Report (2022).

- Mitchell, S., Martín, A. J., Guillén-Gosálbez, G. & Pérez-Ramírez, J. The future of chemical sciences is sustainable. Angew. Chem. Int. Ed. 63, e202318676 (2024).

- Rodin, V., Lindorfer, J., Böhm, H. & Vieira, L. Assessing the potential of carbon dioxide valorisation in Europe with focus on biogenic CO2. J. CO2 Util. 41, 101219 (2020).

- Glaser, B., Parr, M., Braun, C. & Kopolo, G. Biochar is carbon negative. Nat. Geosci. 2, 2 (2009).

- Nabera, A., Martín, A. J., Istrate, R., Pérez-Ramírez, J. & Guillén-Gosálbez, G. Integrating climate policies in the sustainability analysis of green chemicals. Green Chem. 26, 6461–6469 (2024).

- Sacchi, R. et al. PRospective EnvironMental Impact asSEment (premise): A streamlined approach to producing databases for prospective life cycle assessment using integrated assessment models. Renew. Sustain. Energy Rev. 160, 112311 (2022).

94.Yu, X. et al. Stoichiometric methane conversion to ethane using photochemical looping at ambient temperature. Nat. Energy 5, 511–519 (2020).

95.Wu, S. et al. Ga-doped and Pt-loaded porous TiO2–SiO2 for photocatalytic nonoxidative coupling of methane. J. Am. Chem. Soc. 141, 6592–6600 (2019).

96.Zhu, C. et al. Electrochemical conversion of methane to ethylene in a solid oxide electrolyzer. Nat. Commun. 10, 1173 (2019).

- Johnston, P., Carthey, N. & Hutchings, G. J. Discovery, development, and commercialization of gold catalysts for acetylene hydrochlorination. J. Am. Chem. Soc. 137, 14548–14557 (2015).

- Malta, G. et al. Identification of single-site gold catalysis in acetylene hydrochlorination. Science 355, 1399–1403 (2017).

99.Chen, Z., Wang, S., Zhao, J. & Lin, R. Advances in single-atom-catalyzed acetylene hydrochlorination. ACS Catal. 14, 965-980 (2024).

Fig. 1 | The acetylene timeline. The timeline illustrates the evolution of various industrial production technologies for C2H2 since its discovery, advancements in C2H2 chemistry, global production changes of relevant chemicals at key historical points, and the gradual market transition from C2H2 to C2H4.

Fig. 2 | C2H2 production routes, past and current applications. The figure introduces various C2H2 production routes using fossil-derived feedstocks, including coal, natural gas, and oil, along with several technologies such as CaC2, PC, plasma, and TP. The figure highlights major industrial applications of C2H2 and provides data on approximate global annual production (Mt y–1) and market size (USD billion y–1). It distinguishes between commercial routes (currently in use), abandoned routes (past commercial processes replaced by other methods), and trial routes (processes with research interest but never largely commercialized). aSince almost all VCM demand is for PVC production, this number reflects the global market size of PVC. a,bData from Statista.

Fig. 3 | Commercial acetylene production routes. a. Four major issues associated with the CaC2 route, including high energy input, high direct CO2 emissions, production of slag waste, and large coal input. b. Gibbs free energy of formation per carbon atom for several hydrocarbons from the elements as a function of temperature. c. Comparison of partial combustion and thermal plasma routes. Panel b. adapted with permission from ref. 32, Wiley.

Fig. 4 | Emerging acetylene synthesis routes. a. Electrode- and electrodeless-based non-thermal plasma reactors.5,69 b. Structure of membrane reactor in TP.65 c. Conversion-selectivity trend in TP, non-thermal and thermal plasma. d. Conversion-selectivity trend and energy requirement (kWh kg–1 C2H2) for representative non-thermal (cold and warm) and thermal plasma routes. e. Representative catalyst type, reaction temperature, CH4 conversion, and C2H2 and C2 selectivity in non-oxidative CH4 conversion. Panels adapted with permission from: a. ref. 5, Royal Society of Chemistry; b. ref. 65, Wiley.

Fig. 5 | Potential green C2H2 synthesis routes. a. Production costs (only operational costs) and b. climate change impacts per kg of VCM via the acetylene-to-VCM (ATV) route and the ethylene-based balanced process (EBP). Results for both the fossil and green routes, depending on the feedstock source. Uncertainties (error bars) in both climate change impacts and production costs are displayed (see Supplementary Notes for details). c. Coal, natural gas, and oil-based technologies, including CaC2, PC, TP, and plasma routes. The commercial routes, coal-based CaC2 and natural gas-based PC, have a larger width due to their extensive use. d. biochar, biogas, CO2, and plastic-based technologies, including CaC2, plasma, cycle, TP, and PC. Renewable source-based routes are not yet commercialized, so their width remains uniform. CO2-based concept cycles include e. BaC2-based route coupling CO2 capture and CO production (CO2-CO cycle), and f. Li2C2-based electrochemical-assisted three-step cycle from LiOH to Li, with Li reacting with CO2 to form Li2C2, followed by reaction with H2O to produce C2H2 (CO2-C2H2 cycle). Panels adapted with permission from: e. ref. 49, Royal Society of Chemistry; f. ref. 57, Royal Society of Chemistry.

Fig. 6 | Environmental and economic comparison of C2H2 manufacture. a. Climate change impacts and b. production costs (only operational costs) per kg of C2H2 via the CaC2 route and the PC or plasma routes. Biochar (from poplar), as an alternative to coke for the CaC2 route, and biomethane, as an alternative to methane for the PC and plasma technologies, are evaluated for the green routes. Future projections (for 2050) of climate change impacts for all technologies are presented to illustrate the potential reduction in impacts under a decarbonized electricity mix scenario. Uncertainties (error bars) in both climate change impacts and production costs are displayed (see Supplementary Notes for details).